SA WG2 Meeting #161	S2-2403431

# 26th Feb – 01st March, 2024; Athens, Greece                 (revision of S2-2403144)

Source: 	CATT

Title: 	KI#2&#3, New Sol: NAS-based information transfer for Ambient IoT Services

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper is to propose a new solution to support NAS-based information transfer for Ambient IoT Services.

1. Proposal

It is proposed to agree the following changes into TR 23.700-13.

1st Change

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |  |
| X |  | x | x |  |

2nd Change

6.X	Solution #X:  NAS-based information transfer for Ambient IoT Services

6.X.1	Description

The solution applies to both the Key Issue #2 "Identification, Subscription, Registration and Connection management" and the Key Issue #3 "Support of Ambient IoT Services". This solution applies to Topology 1.

Considering the nature of the AIoT Devices (e.g. ultra-low complexity power, cost and resource-constrained), the PDU Session/QoS Flow based data transfer is not suitable for such AIoT Devices. This solution proposes to use NAS-based message for information transfer for Ambient IoT Services. The AF requests service operation for an AIoT Device or a group of AIoT Devices via the NEF, and the service operation is forwarded to the AMF/AIoTF (new function introduced to support AIoT services) which then triggers the N2 like procedure and AS procedure to the AIoT Devices. After receiving AS message, the AIoT Devices includes Device ID and AIoT data requested by service operation in the NAS message and send it to the AMF/AIoTF, and then the AMF/AIoTF sends the Device ID and AIoT data to the AF via the NEF.

6.X.2	Procedures

Depicted in Figure 6.X.2-1 is the procedure for NAS-based information transfer for Ambient IoT Services.

Figure 6.X.2-1: Procedure for NAS-based information transfer for Ambient IoT Services

1.	AF sends a AIoT Service Request (AF Identifier, Device ID, service operation) message to the NEF. The service operation indicates the service (e.g. Inventory, Command) the AF requested for the Ambient IoT Device(s). Device ID is used to identify an Ambient IoT Device or a group of Ambient IoT Devices.

Editor's note: Whether service operation is an end-to-end parameter between AF and Ambient IoT Device is FFS.

2.	The NEF checks if the AF is authorized to request the AIoT service.

3.	The NEF selects the AMF/AIoTF supporting AIoT service and forwards the service operation to the AMF/AIoTF using AIoT Service Request (Device ID, service operation) message.

Editor's note: The details on AMF/AIoTF selection and Reader (BS) selection are FFS.

4.	The AMF/AIoTF sends a N2 like message (NAS message (Device ID, service operation)) to Reader (BS).

Editor's note: Whether existing NAS message is reused or new NAS like message is defined is FFS.

5.	The Reader (BS) performs AS procedure with Ambient IoT Devices. The NAS message (Device ID, service operation) is included in the AS message to Ambient IoT Devices.

6.	For each Ambient IoT Device, if Ambient IoT Device receives AS message and is matched with the Device ID, the Ambient IoT Device initiates Random Access like procedure.

NOTE X:	The steps 5 and 6 are to be defined by RAN WGs.

7.	The Ambient IoT Device sends NAS message (Device ID, AIoT data) over AS message. AIoT data is included in the NAS message if requested by the service operation.

8.	The NAS message is forwarded by Reader (BS) to the AMF/AIoTF.

Editor's note: The security protection of NAS message is FFS.

9.	The AMF/AIoTF checks if the Ambient IoT Device is authorized to transfer the AIoT data.

Editor's note: It is FFS whether it can be assumed for all scenarios that the device and the CN can be pre-provisioned with CN level per device information (e.g. network layer AIoT device ID, security material).

10.	The AMF/AIoTF responds to the NEF using AIoT Service Response (Device ID, AIoT data) message.

11.	The NEF responds to the AF using AIoT Service Response (Device ID, AIoT data) message.

6.X.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

End of Changes