3GPP TSG-SA2 Meeting #161	S2-2403500

Athens, Greece, 26th Feb 2024 - 1st Mar 2024                  	(revision of S2-2403219)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4686 | rev | 4 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on PDU Sessions due to Network Slice Replacement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Huawei, Ericsson, LGU Plus, LG Electronics, ZTE, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some issues related to the PDU Session due to Network Slice Replacement remain unresovled: -Clarification on NSAC for such PDU Session during the network slice replacement |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Add NSAC for the PDU Session caused by Network Slice Replacement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | -The number of PDU session on the Alternative S-NSSAI and/or the S-NSSAI is not updated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.11.4x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## FIRST CHANGE

#### 4.2.11.4x	Number of PDU Sessions per network slice availability check and update procedure for Network Slice Replacement

If the S-NSSAI is to be replaced by an Alternative S-NSSAI (or if the replaced S-NSSAI is used again), and if the Alternative S-NSSAI and/or the replaced S-NSSAI is subject to NSAC, the enforcement of maximum number of PDU Session established for the Alternative S-NSSAI and/or the replaced S-NSSAI is performed as defined in clause 4.2.11.4 (or 4.2.11.4a) with the following differences:

1.	In step 1 of clause 4.2.11.4 (or 4.2.11.4a), the SMF performs the NSAC procedure twice (i.e., for the increase of Alternative S-NSSAI and the decrease of replaced S-NSSAI) when the SMF is requested to replace the Network Slice of the existing PDU Session and the PDU session is retained (i.e., at the beginning of a PDU Session Modification procedure (clause 4.3.3.2 and clause 4.3.3.3)).

2. In step 2 of clause 4.2.11.4 (or 4.2.11.4a), if the SMF is requested to replace the S-NSSAI with the Alternative S-NSSAI, the SMF includes the Alternative S-NSSAI and the update flag set to ‘increase’ in the message when the Alternative S-NSSAI is subject to NSAC, and the SMF includes the replaced S-NSSAI and the update flag set to ‘decrease’ in the message when the replaced S-NSSAI is subject to NSAC.

NOTE 1:	During PDU Session Modificaion procedure, based on the operator policy, SMF may not invoke the NSACF to decrease the count for maximum number of PDU Session for replaced S-NSSAI which means for the same PDU Session, maximum number of PDU Session will be counted twice.

If SMF receives a new PDU Session with both S-NSSAI and Alternative S-NSSAI and if the Alternative S-NSSAI and/or the replaced S-NSSAI is subject to NSAC, the enforcement of maximum number of PDU Session established for the Alternative S-NSSAI and/or the replaced S-NSSAI is performed as defined in clause 4.2.11.4 (or 4.2.11.4a) with the following differences:

1.	In step 1 of clause 4.2.11.4 (or 4.2.11.4a), the SMF performs the NSAC procedure (i.e., for the increase of Alternative S-NSSAI).

2. In step 2 of clause 4.2.11.4 (or 4.2.11.4a), the SMF includes the Alternative S-NSSAI and the update flag set to ‘increase’ in the message when the Alternative S-NSSAI is subject to NSAC.

NOTE 2:	During new PDU Session establishment procedure, based on the operator policy, SMF may invoke the NSACF to increase the count for maximum number of PDU Session for replaced S-NSSAI which means for the same PDU Session, maximum number of PDU Session will be counted twice.

## END of CHANGES