3GPP TSG-WG SA2 Meeting #161	S2-2402149

# 26 Feb – 1 Marc, 2024, Greece, Athens		          	 (revison of S2-240xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | draft CR | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Deferred 5GC-MT-LR Procedure for Periodic Location Events based NRPPa Periodic Measurement Reports |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_DLPMR |  |  |  |  |  |  | Date: |  | 2023-02-16 |  |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-19 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |
| Reason for change: |  | RAN provides solution on NRPPa in NR to receive periodic location measurements from gNB. LMF could use this periodical location information to generate location estimates periodically. However, there is no SA2 procedure which could leverage this periodical received location information to generate location estimates periodically. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new procedure Deferred 5GC-MT-LR Procedure for Periodic Location Events based NRPPa Periodic Measurement Reportrs |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (New) |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

### 6.x	Deferred 5GC-MT-LR Procedure for Periodic Location Events based on NRPPa Periodic Measurement

### 6.x.1	Initiation and Reporting of Periodic Events based on NRPPa Periodic Measurement

1.	Deferred 5GC-MT-LR steps 1-13 are executed as described in clause 6.1.1. AF or LCS clients triggers periodic location event.

2. LMF performs one or more of the positioning procedures as it is described in step 15 in clause 6.1.1.

3. Deferred 5GC-MT-LR steps 18-21 are executed as described in clause 6.1.1 with the following changes

- Indicator added to step 18,19, and 20 to indicate Periodic Location Estimates are generated based on NRPPa Periodic Measurement.

4. LMF based on UE capability information received in step 2 and based on configuration decides to perform on NRPPa Periodic Measurements. LMF invokes Namf_EventExposure_Subscribe to receive events from serving AMF about NG-RAN change using “RAN_Node” as LocationFilter.

5. LMF performs NRPPa Periodic Measurement Inititation (clause 8.2.1.2.in TS 38.455[15]). LMF uses the time interval between successive location reports, the total number of reports received in step1 to setup periodicity. LMF is receiving Measurement Report from NG-RAN periodically (clause 8.2.3.2 in TS 38.455).

6. LMF invokes Nlmf_Location_EventNotify service operation as described in step 28 in clause 6.1.1 in order to transfer location information to VGMLC or (H)GMLC.

7. Deferred 5GC-MT-LR steps 29 is executed as described in clause 6.1.1.

8a.: Deferred 5GC-MT-LR steps 30a is executed as described in clause 6.1.1.

8b-1: Deferred 5GC-MT-LR steps 30b-1 is executed as described in clause 6.1.1.

8b-2: Deferred 5GC-MT-LR steps 30b-2 is executed as described in clause 6.1.1.

### 6.x.2	Cancellation of Reporting of Periodic Events by an AF, an NF or External LCS Client or GMLC

Cancellation of Reporting of Periodic Events by an AF, an NF or External LCS Client or GMLC based on NRPPa Periodic Measurement executed as decribed in 6.3.3 with the following changes:

- Steps from 6 and 9 are not executed based on indicator provide by GMLC in step 3 and 4. (ie.: UE’s related procedures are not executed)

- When in step 5 LMF receives Namf_Location_CancelLocation, LMF stops NRRPa periodic measurement report by requesting Measurement Termination (clause 8.2.4.2 in TS 38.455[15]) and unsubscribe for AMF event.

### 6.x.3	NG-RAN handover

When LMF receives from gNB Serving NG-RAN node changed cause as error indication or receives Namf_EventExposure_Notify indication NG-RAN change then LMF terminates measurements and reinitiate NRPPa Periodic Measurement Initiation.

It is LMF responsibility to reinitiate the NRPPa Periodic Measurement in the time to satisfy the required periodicity from AF, NF and LCS client.

### 6.x.4	Handling of CM-IDLE Mode

### When LMF receives from gNB error cause that NG-RAN node is unable to provide the measurement results, probably due to UE moved in CM-Idle Mode then LMF can decide to reinitiate NRPPa Periodic Measurement Initiation or cancel the whole periodic location report.

### 6.x.5	AMF handover

### During Inter NG-RAN node N2 based handover procedure, when there is a change of AMF the new AMF receives all event subscriptions from old AMF. New AMF informs LMF about the new Subscription. LMF is informed about the new AMF address and LMF performs NRPPa Periodic Measurement Initiation towards new serving AMF.

AMF handover scenario is applicable for UE associated NRPPa requests.

### 6.x.6	LMF Change Procedure

The LMF Change procedure supports change of a serving LMF during a deferred 5GC-MT-LR procedure for Periodic Location Events based on NRPPa Periodic Measurement. LMF change can be initiated due to new NG-RAN or new serving AMF.

Figure 6.x.5-1: Change of serving LMF for Periodic Events based on NRPPa Periodic Measurement

1.	NG-RAN or AMF change happens as described in 6.x.3 and 6.x.4.

2.	LMF1 may evaluate and determine that it is unsuitable or unable to support location for the current UE NG-RAN and determines LMF2 as being a more suitable LMF. LMF1 may already have LMF2 information e.g. from previous NRF discovery or locally configured, otherwise LMF1 queries. LMF1 selects new LMF based on NG-RAN location based and other information as listed in clause 5.1.

3.	LMF1 invokes an Nlmf_Location_LocationContextTransfer Request service operation towards LMF2 to provide the current location context of the UE. The service operation includes the AMF identity and all the information originally received by LMF1 for the periodic location request either from the AMF or from an earlier serving LMF according to this procedure. LMF2 invokes Namf_EventExposure_Subscribe to receive events from serving AMF about NG-RAN change.

4.	LMF2 informs LMF1 of the location context transfer operation results.

5. 	LMF1 releases all resources for the procedure and unsubscribe.

* * * * End of Changes * * * *