SA WG2 Meeting #161	S2-2402214r02

Athens, Greece, 26 Feburary 26 – March 01, 2024

# Title:	KI#2, New Sol: Enhancement for BDT with Energy related analytics

# Source:	KDDI, TOYOTA MOTOR CORPORATION, Lenovo, InterDigital Inc.

# Document for:	Approval

Agenda Item:	19.4 (FS_EnergySys)

Abstract of the contribution: It is proposed a new Solution for KI#2.

1 Discussion

According to the FS_EnergySys (SP-231391), the following Key Issue #2 need to be studied:

The following aspects will be studied for this key issue:

-	Whether and how to enhance the existing subscription and policy control framework to support energy related information as service criteria, including:

-	Whether and what new energy related UE subscription information are to be defined, and whether and how to use the energy related UE subscription information.

-	Whether and what new energy related policies are to be defined, and how to perform energy related policy control, e.g. to determine, provision and enforce energy related policies.

-	At what granularity (e.g., network slice, UE, NF, PDU Session, QoS flow, application ID, etc.) the energy related policy control can be performed.

-	What network energy related information is required for subscription and policy control and how it is obtained.

-	Whether and how the above enhancements on subscription and policy control will impact charging.

The solution for NWDAF-based Energy Analytics (Solution #12) was implemented in TR 23.700-66. This contribution proposes how to enhance the Negotiation for the future BDT and the BDT warning notification considering the Energy related analytics.

2. Proposal

It is proposed to add the following new solution to TR 23.700-66 " Study on Energy Efficiency and Energy Saving".

# *** 1st Changes ***

6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | X |
| 1 | x |  |  |  |
| 2 | x |  |  |  |
| 3 | x |  | x |  |
| 4 | x |  |  |  |
| 5 | x |  | x |  |
| 6 | x |  |  |  |
| 7 |  | x |  |  |
| 8 |  | x |  |  |
| 9 |  | x |  |  |
| 10 |  | x |  |  |
| 11 |  | x | x |  |
| 12 |  |  | x |  |
| 13 |  |  | x |  |
| 14 |  |  | x |  |
| 15 |  | x | x |  |
| X |  |  |  |  |

# *** 2nd Change  (all new text)***

6.x	Solution #X: Enhancement for BDT with Energy related analytics

6.x.1	Key Issue mapping

This solution maps to KI#2.

6.x.2		Functional Description

This solution is proposed to address Key Issue #2: 5GS enhancements for network energy saving and efficiency.

In case of negotiation for future BDT (background data transfer), a H-PCF may interact with the NWDAF and request the Network Performance analytics information for the Desired time window and the Network Area Information as defined in TS 23.502 [3]. For BDT warning notification, the H-PCF may subscribe to the analytics on the Network Performance analytics from NWDAF for the area of interest and time window of a background data transfer policy, including a Reporting Threshold in the Analytics Reporting information as defined in TS 23.502 [3]. However, the H-PCF does not consider Energy related information for determination of BDT policies.

The proposed solution enhances the existing BDT policy selection and re-negotiation process by introducing energy related information as service criteria. Specifically, it enhances the existing subscription and policy control framework to support energy consumption and energy efficiency service criteria.

The AF can also consider the target energy cost value, an energy identifier (i.e., to indicate the selection of a policy that minimizes the energy cost value), a target energy efficiency value, the charging rate, and the other attributes of the background data transfer policies when deciding which background data transfer policy to select. In other words, the AF can decide what trade-offs to make between energy cost value, monetary cost, the time of the transfer, and the maximum bit rates of the transfer.

When a BDT policy warning notification is issued, the PCF can also provide a list of new set of candidate BDT policies considering the energy criteria on the new network conditions, from which the AF can choose. It can also provide the reasons for triggering such re-negotiation to allow the AF to make a choice regarding the selection of the new BDT policy considering the time schedule and data transfer rate vs the imposed energy criteria. Alternatively, the PCF can notify the AF indicating only that the energy criteria cannot be met, then the AF can select a different BDT policy.

6.x.3	Procedure

6.x.3.1	Procedure for Negotiation for future BDT with Energy related analytics

Figure 6.x.3-1: Procedure for Negotiation for future BDT with Energy related analytics

The procedure described in this clause is based on the negotiation for future BDT as defined in clause 4.16.7.2 of TS 23.502 [3].

In step 1, the AF invokes the Nnef_BDTPNegotiation_Create Request. The request may contain Area of interest, Desired time window and following Energy Criteria:

‐	Energy related KPIs (e.g. Maximum allowed EE, EC or Renewable energy ratio for NG-RANs and/or UPFs in the Area of interest).

‐	Energy optimization activation flag; once this identifier is enabled it indicates to the PCF that it needs to select a BDT policy that optimizes energy costs or energy efficiency considering the current network conditions.

- Target energy cost value; when the AF needs to collect or transmit background data from or towards a set of UEs based on the expected average data volume.

- Target energy efficiency considering the expected performance to the total energy cost related to the background data transfer.

In step 2, the NEF invokes the Npcf_BDTPolicyControl_Create Request. Examples of actions for the H-PCF are:

-	If the Npcf_BDTPolicyControl_Create Request contains the Energy related KPIs, the H-PCF interacts with the NWDAF/EECF and sends a request or subscription for the Energy related analytics (Area of interest, Desired time window, the Energy related KPIs requested by AF, the target energy cost value requested by AF). In addition, the H-PCF may obtain more information to make this decision from the NWDAF by requesting additionally network analytics (e.g., Analytics ID = “Network Performance”) for the desired time window and/or the area of interest.

-	If the Npcf_BDTPolicyControl_Create Request contains the Energy optimization activation flag, the H-PCF interacts with the NWDAF/EECF and sends a request or subscription for the Energy related analytics (Area of interest, Desired time window, Energy related KPIs (e.g. Maximum allowed EE, EC or Renewable energy ratio for NG-RANs and/or UPFs in the Area of interest, the target energy cost value requested by AF). The Energy related KPIs for the request or subscription are determined based on operator configuration. In addition, the H-PCF may obtain more information to make this decision from the NWDAF by requesting additionally network analytics (e.g., Analytics ID = “Network Performance”) for the desired time window and/or the area of interest.

In Step3, The UDR provides the request stored BDT policies and corresponding related information (as defined in clause 6.1.2.4 of TS 23.503 [4] also including Energy Criteria) to the H-PCF.

In step 5, the NWDAF/EECF sends a response or notify with Energy related analytics to the H-PCF, if the requested Energy related KPIs are met.

NOTE:	How the NWDAF/EECF generates the analytics for Energy related conditions is not addressed in the proposed solution but uses other solutions (e.g. Solution #10, Solution #12).

In step 6, the H-PCF determines one or more BDT policies, based on information provided by the AF, the NWDAF/EECF and other available information. Then it sends the acknowledge message to the NEF with the acceptable BDT policies (including the energy cost provided by the NWDAF/EECF) and Background Data Transfer Reference ID as defined in clause 4.16.7.2 of TS 23.502 [3].

In step 9, the AF selects a background data transfer policy and the AF can consider the energy cost of each policy when selecting a policy. The AF invokes the Nnef_BDTPNegotiation_Update service to provide the NEF with Background Data Transfer Reference ID and the selected background data transfer policy.

In step 13, the H-PCF stores the estimated, or predicted, energy cost with the new Background Data Transfer policy.

6.x.3.2	Procedure for BDT warning notification with Energy related analytics

Figure 6.x.3-2: Procedure for BDT warning notification

The procedure described in this clause is based on the BDT warning notification as defined in clause 4.16.7.3 of TS 23.502[3].

The negotiation for BDT described in clause 6.x.3.1 is completed. In addition, the H-PCF has subscribed to Energy related analytics from the NWDAF/EECF for the Area of interest and the Desired time window of a BDT policy.

In step 2-1, the H-PCF is notified that the network condition has changed by subscribing to network performance analytics as per TS 23.288 [14]. Additionally, the H-PCF is notified with the Energy related analytic in the Area of interest from the NWDAF/EECF when the NWDAF/EECF determines that the requested Energy related KPIs goes below the threshold.

NOTE:	How the NWDAF/EECF generates the analytics for Energy related conditions is not addressed in the proposed solution but uses other solutions (e.g. Solution #10, Solution #12).

In step 2-2, the H-PCF is notified that the energy related performance has degraded (the H-PCF subscribe to e.g., MDA (as per TS 28.104 [16]), or to energy related performance measurements (as per TS 28.552 [10]) and KPIs (as per TS 28.554 [6])).

In step 3, the H-PCF may retrieve all the BDT Policies together with the relevant information received from the AF and identifies the BDT policies affected by the notification received from the NWDAF/EECF, the MDA or the OAM. The H-PCF decides based on operator policies, whether a new list of candidate BDT policies can be calculated for the ASP, and sends the BDT warning notification to the AF via the NEF as defined in clause 4.16.7.3 of TS 23.502 [3].

6.x.4	Impacts on existing services, entities and interfaces

NWDAF/EECF:

-	Support for the Energy related analytics.

NEF:

-	Nnef_BDTPNegotiation service is extended to support the Energy related KPIs, Energy optimization activation flag, an energy cost value and Target energy efficiency.

H-PCF:

-	Request/Subscribe to the Energy related analytics and determines BDT policies based on information.

- Stores the estimated, or predicted, energy cost with the new Background Data Transfer policy in the UDR.

UDR:

- Stores the estimated, or predicted, energy cost with the new Background Data Transfer policy.

# *** End of Changes ***