3GPP TSG-SA WG2#161	S2-2403188

Athens, Greece, 26 February 26 –1 March, 2024	revision of S2-2402827

Source:	TOYOTA MOTOR CORPORATION, KDDI

Title:	TR 23.700-66: KI#2 – Solution for energy related UE subscription information

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract: This contribution proposes a solution for KI#2 about energy related UE subscription information.

# 1. Discussion

In this contribution, we propose a solution for energy related UE subscription information about the following KI#2.

The following aspects will be studied for this key issue:

-	Whether and how to enhance the existing subscription and policy control framework to support energy related information as service criteria, including:

-	Whether and what new energy related UE subscription information are to be defined, and whether and how to use the energy related UE subscription information.

-	Whether and what new energy related policies are to be defined, and how to perform energy related policy control, e.g. to determine, provision and enforce energy related policies.

-	At what granularity (e.g., network slice, UE, NF, PDU Session, QoS flow, application ID, etc.) the energy related policy control can be performed.

-	What network energy related information is required for subscription and policy control and how it is obtained.

-	Whether and how the above enhancements on subscription and policy control will impact charging.

# 2. Proposal

This paper proposes to add a solution for Key Issue #2 to TR23.700-66 as follows.

---------- Start of change ----------

6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | X |
| 1 | x |  |  |  |
| 2 | x |  |  |  |
| 3 | x |  | x |  |
| 4 | x |  |  |  |
| 5 | x |  | x |  |
| 6 | x |  |  |  |
| 7 |  | x |  |  |
| 8 |  | x |  |  |
| 9 |  | x |  |  |
| 10 |  | x |  |  |
| 11 |  | x | x |  |
| 12 |  |  | x |  |
| 13 |  |  | x |  |
| 14 |  |  | x |  |
| 15 |  | x | x |  |
| x |  | x |  |  |

---------- Next change (all new text) ----------

6.x	Solution #X: Energy related UE subscription information

6.x.1	Key Issue mapping

This solution resolves KI#2 for energy related UE subscription information.

6.x.2	Functional Description

UE subscription information needs to be consolidated to support energy saving effectively. The proposed energy related UE subscription information is structured in Table 6.x.2-1 and the details are as follows.

- Energy saving application Indicator: Indicates whether the NW should apply energy saving related procedures to the UE.

- Energy Efficiency (EE) threshold: Specifies the threshold of energy efficiency for energy saving related procedures such as energy related information exposure, policy control and etc.

- Energy Consumption (EC) Requirements: Specifies the requirements for the UE's energy consumption in a specific period of time or space, or the requirements for the UE's maximum energy consumption.

- Renewable energy ratio threshold: Specifies the threshold of renewable energy ratio for energy saving related procedures such as energy related information exposure, policy control and etc.

- Conditional Energy Requirements: Indicates the combined conditions or hierarchies related to Energy Efficiency (EE), Energy Consumption (EC), and the use of renewable energy. Examples include:

  - Preference for EE above other conditions.

  - Preference for renewable energy usage above other conditions.

  - Requirement to meet both EE and renewable energy considerations simultaneously.

  - Requirement to meet either EE or renewable energy requirements based on availability.

- Service impact flag: Indicates the UE's tolerance for potential impacts on service quality in exchange for energy savings. This includes various aspects, such as:

  - Acceptance of relaxed Quality of Service (QoS) parameters.

  - Permission for communication service interruption (e.g., PDU session release).

  - Other trade-offs between service performance and energy efficiency.

Table 6.x.2-1: Energy related UE subscription data

| Subscription data type | Field | Description |
| --- | --- | --- |
| Energy related subscription data (data needed for considering EE, EC, renewable energy usage) | Energy saving application Indicator | whether the NW should apply energy saving related procedures to the UE |
|  | Energy Efficiency (EE) threshold | Energy Efficiency threshold |
|  | Energy Consumption (EC) Requirements | Energy consumption related requirements |
|  | Renewable energy ratio threshold | Renewable energy ratio threshold |
|  | Conditional Energy Requirements | Combined conditions or hierarchies related to Energy Efficiency (EE), Energy Consumption (EC), and the use of renewable energy |
|  | Service impact flag | UE's tolerance for potential impacts on service quality in exchange for energy savings |

6.x.3	Procedures

The energy related UE subscription information is maintained in UDM/UDR and can be retrieved by other NFs for energy saving related operations such as AM/SM policy control, PDU session establishment/modification and etc.

NOTE: The proposed UE subscription information is general to other energy saving related solutions. We reuse the procedures of solution #7 to show how to use the proposed UE subscription information.

Figure 6.x.3-1: UE subscription data based energy saving operations

- The energy related UE subscription data is provisioned in UDM/UDR. This provisioning process can occur during the initial user contract phase, or dynamically via UDM services.

- The same procedure of Solution #7 step 1-3.

- EECF checks Energy saving application indicator of UE subscription data. If the indicator is yes, EECF continues the energy saving related procedure. Otherwise EECF stops the energy saving related procedure.

- The same procedure of Solution #7 step 4-6.

- PCF retrieves the energy related UE subscription data from UDR.

- PCF checks Service impact flag of UE subscription data. If PCF decides to release current PDU session and the flag shows it is acceptable, PCF continues the energy saving related procedure to release the PDU session. Otherwise PCF stops the energy saving related procedure.

- The same procedure of Solution #7 step 7-10 to release the PDU session.

Editor's Note: Control of UE energy consumption requirement based on Energy related UE subscription information is FFS.

6.x.4	Impacts on existing services, entities and interfaces

UDM/UDR:

- Should support the proposed energy related UE subscription data.

PCF/AMF/SMF/EECF:

- Should support the energy related UE subscription data retrieval and processing.

---------- End of change ----------