3GPP SA WG2 Meeting #161 	S2-2402321-2-r1

# Athens, Greece, 2024‑02‑26 -- 2024‑03‑01

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | - | rev | - | Current version: | 18.4.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inter-PLMN Indirect Communication (re)selection of target NF producer delegation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_TPlmnNfSel |  |  |  |  |  | Date: |  |  | 2024-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support for Inter-PLMN Indirect Communication (re)selection of target NF producer delegation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce support for Inter-PLMN Indirect Communication (re)selection of target NF producer delegation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-PLMN Indirect Communication (re)selection of target NF producer delegation not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR nnnn |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.17.X	Inter-PLMN Indirect Communication (re)selection of target NF producer delegation capability

The "Inter-PLMN Indirect Communication (re)selection of target NF producer delegation" capability of a network informs about whether this PLMN, acting as a target PLMN, is able to perform (re)selection of target NF producer when delegated from the source PLMN.

The target PLMN’s capability for inter-PLMN delegation may be either "Indirect Communication with delegated discovery" or "Indirect Communication without delegated discovery", depending on whether incoming requests from a remote network including discovery and selection parameters according respectivelly to "Indirect Communication with delegated discovery" or "Indirect Communication without delegated discovery" are supported,

NOTE: 	The assumption is that if an SCP supports "Indirect Communication with delegated discovery", then "Indirect Communication without delegated discovery" is as well supported.

The target PLMN may deploy a variety of communication models (direct or indirect with/without delegated discovery) and deploy one or several SCPs to receive the incoming requests from inbound traffic.

The "Inter-PLMN Indirect Communication (re)selection of target NF producer delegation" capability is known by the target PLMN’s own SEPP. The target PLMN's capability may be fetched by the source SEPP. This is described by following figure.

Figure 4.17.X-1: PLMN delegation capability exchange

The request in the source PLMN follows Indirect Communication via one or several SCPs. For intra-PLMN Indirect Communication one SCP in the path shall perform (re)selection of NF producer. For inter-PLMN requests, this SCP may decide to perform delegation of (re)selection to the target network, only if the target PLMN's "Inter-PLMN Indirect Communication (re)selection of target NF producer delegation" capability is compatible with the Indirect Communication model of the request. Therefore, this SCP may interrogate the SEPP to get target PLMN capability, and only if compatible, the (re)selection will be delegated to the target PLMN. Otherwise, the (re)selection is performed as described for intra-PLMN Indirect Communication.

The following figure describes the interaction between SCP and SEPP to obtain "Inter-PLMN Indirect Communication (re)selection of target NF producer delegation" capability.

Figure 4.17.X-2: SCP obtains Inter-PLMN delegation capability

**** Next Change ****

### 4.17.10	Indirect Communication With Delegated service discovery when NF service consumer and NF service producer are in different PLMNs

Figure 4.17.10-1: Indirect Communication with Delegated discovery when NF service consumer and NF service producer are in different PLMNs

1.	The NF service consumer intends to communicate with an NF service producer. The NF service consumer sends the request to an SCP. The request includes the source PLMN ID, the target PLMN ID in the discovery and selection parameters necessary for the SCP to discover and select a NF service producer instance on behalf of the NF consumer. The discovery and selection parameters are included in the request by the NF service consumer in a way that the SCP does not need to parse the request body.

Following steps apply when the source SCP does not support "Inter-PLMN delegation":

2.	The SCP recognises that the request is for a NF service producer in another PLMN. SCP interacts with NRF using the Nnrf_NFDiscovery service.

3.	NRF in PLMN-1 and NRF in PLMN 2 interact using the Nnrf_NFDiscovery service. See step 2 in clause 4.17.5.

4.	SCP gets Nnrf_NFDiscovery service response with NF profile(s).

5.	SCP selects a NF service producer instance in PLMN-2.

6.	SCP forwards the request to the selected NF service producer instance in PLMN-2, in this example via an SCP in the target PLMN2.

7. 	The request reaches the selected NF service producer. This request may be routed to the selected NF service producer via one or more SCPs in the target network, this is omitted in the figure for simplicity.

8. 	If reselection is required, this is performed in the source PLMN-1.

Following steps apply when the source SCP supports "Inter-PLMN delegation":

2.	The source SCP checks whether the Indirect Communication (re)selection of target NF producer delegation capability allows the delegation of (re) selection of the target NF service producer to the target PLMN. The SCP gets this information by interrogating the SEPP (see clause 4.17.X). Based on that information, the SCP knows whether required inter-PLMN delegation is possible. In this example, the target PLMN for inter-PLMN delegation supports “Indirect Communication with delegated discovery”.

If the target PLMN does not support “Indirect Communication with delegated discovery”, the process is the same described from step 2 when the SCP does not support "Inter-PLMN delegation".

3.	The source SCP identifies the communication model of the request is compatible with the target PLMN capability and the delegation is possible. The source SCP sends the request to the local SEPP (omitted in figure for simplicity). Since initial selection is delegated to the target network, the source SCP includes a fake target destination in the request that is forwarded. This fake target destination is identified by the target SEPP as an indication that (re)selection is delegated and the request is routed to a target SCP. This target SCP supports the Inter-PLMN delegation capabilities that have been provided by the target SEPP to the source SEPP and then to the source SCP.

4. 	Selected target SCP identifies (re)selection is delegated by the presence of the fake target destination and then it performs initial selection using discovery and selection parameters received from source network (for that it may need to perform NRF discovery or use cached NF producer profiles).

5. 	The request reaches the selected NF service producer. This request may be routed to the selected NF service producer via one or more SCPs in the target network, this is omitted in the figure for simplicity.

6. 	If reselection of target NF service producer is required, this SCP in the target PLMN is responsible to execute that, as indicated by the presence of the fake target destination, avoiding the signalling back to the source PLMN and the new inter PLMN request.

Alternatively, SCP may send the discovery request directly to the NRF in PLMN-2, if it has the relevant NRF address and is authorized by the NRF in PLMN-2. Thus step 2 goes from SCP to NRF in PLMN-2 and step 4 goes from NRF in PLMN-2 to SCP and step 3 is omitted.

**** Next Change ****

### 4.17.11a	Indirect Communication Without delegated discovery procedure when NF service consumer and NF service producer are in different PLMNs

This clause provides the call flow for indirect communication model without delegated discovery when NF service consumer and NF service producer are in different PLMNs.

Figure 4.17.11a-1: Indirect Communication without delegated discovery when NF service consumer and NF service producer are in different PLMNs

1.	The NF service consumer intends to communicate with an NF service producer. The NF service consumer sends the request to an SCP. The NF interacts with NRF using the Nnrf_NFDiscovery service.

2.	NRF in PLMN-1 and NRF in PLMN 2 interact using the Nnrf_NFDiscovery service. See step 2 in clause 4.17.5.

3.	NF gets Nnrf_NFDiscovery service response with NF profile(s).

4.	NF selects an NF service producer instance in PLMN-2.

5. 	NF sends the request (including selected NF producer instance Id or selected NF producer instance Set, or both) to the SCP in its own network.

Following steps apply when the source SCP does not support "Inter-PLMN delegation":

6. 	If only NF producer instance Set was included in the request, the SCP needs to perform initial selection within the Set (for that it may need to perform NRF discovery or use cached NF producer profiles).The SCP forwards the request to the selected NF service producer instance in PLMN-2, in this example via an SCP in the target PLMN2.

7. 	The request reaches the selected NF service producer.

8.	If reselection is required, this is performed in the source PLMN-1, as indicated by the presence of the fake target destination if step 7.

Following steps apply when the source SCP supports "Inter-PLMN delegation":

6.	The source SCP checks whether the Indirect Communication (re)selection of target NF producer delegation capability allows the delegation of (re) selection of the target NF service producer to the target PLMN. The source SCP gets this information by interrogating the source SEPP (see clause 4.17.X). Based on that information, the SCP knows whether required inter-PLMN delegation is possible. In this example, the target PLMN for inter-PLMN delegation supports "Indirect Communication with delegated discovery" or "Indirect Communication without delegated discovery".

If the target PLMN does not support either "Indirect Communication with delegated discovery" or "Indirect Communication without delegated discovery", the process is the same described from step 6 when the SCP does not support "Inter-PLMN delegation"..

7.	The source SCP identifies the communication model of the request is compatible with the target PLMN capability and the delegation is possible. The source SCP sends the request to the local SEPP (omitted in figure for simplicity).

If the source SCP receives a target NF producer instance and a target NF producer instance Set, the SCP may remove the selected NF producer instance and include a fake target destination in the request that is forwarded. This fake target destination is identified by the target SEPP as an indication that initial selection is delegated.

If the source SCP receives only a target NF producer instance, the SCP adds the corresponding NF producer instance Set to indicate the NF producer instance reselection (if required) is delegated.

If the source SCP receives only a target NF producer instance Set, the SCP adds the fake destination that is required to route the request.

The request is routed to a target SCP. This target SCP supports the Inter-PLMN delegation capabilities that have been provided by the target SEPP to the source SEPP and then to the source SCP.

8. 	The selected target SCP identifies initial selection is delegated by the presence of the fake target destination. In this case, the SCP performs initial selection (for that it may need to perform NRF discovery or use cached NF producer profiles).

9. 	The request reaches the selected NF service producer.

10. If reselection of target NF service producer is required, this SCP in the target PLMN is responsible to execute that as indicated by the presence the NF producer instance Set in step 7, avoiding the signalling back to the source PLMN and the new request inter PLMN.

**** End of Changes ****