SA WG2 Meeting #162	S2-2404567

China, Changsha, April 15 – April 19, 2024	(revision of S2-240xxxx)

Source:	LG Electronics

Title:	KI#2, Sol #2: Update to resolve EN and avoid direct interaction between AMF and UDR

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract of the contribution: This paper proposes to update solution #2 to resolve EN and remove direct interaction between AMF and UDR.

Discussion

In solution #2 there is following EN.

Editor's note:	How visited UDR contacts the AMF or finds AMF is FFS.

Also, this solution proposes that AMF can receive visited CAG information from the UDR directly.

According to current architecture, however, there is no interface between AMF and UDR. The UDR has interface only with UDM, PCF and NEF. Therefore, adding such interface have architecture impact should be avoided unless really necessary.

In order to resolve above EN and to remove such architecture impact, it is proposed that AMF establishes AM Policy Association to receive visited CAG information from V-UDR.

Proposal

It is proposed to agree the following changes into TR 23.700-45.

* * * * Start of 1st Change * * * *

6.2	Solution #2: CAG provisioning and access control via V-UDR in the visited network

6.2.1	Description

In PNI-NPN, Closed Access Group (CAG) identifies a group of subscribers who are permitted/allowed to access one or more CAG cells of the PLMN identified by CAG ID(s), which is assumed to be reused for 5G Femto access control. A CAG cell is a cell broadcasting one or several CAG IDs. CAG membership of UE is configured in the user subscription data and on the UE. If a UE is roaming, the access control should be performed in the visited network based on CAG IDs configured in visited PLMN (VPLMN), and UE needs be provisioned with the allowed visited CAG cell access information in the visited network.

When introducing the concept of 5G Femto in FS_5G_Femto study, the scenario that the UE is roaming to VPLMN (or EHPLMN) and access to CAG cells of 5G Femto in VPLMN must be considered. Currently in 5G, allowed CAG list is provisioned in the HPLMN, of which the allowed CAG list in VPLMN may be also provisioned by HPLMN if SLA is assigned between HPLMN and VPLMN. However, in 5G Femto scenario, the provisioning of CAG access may be rather dynamic e.g. when end users visit friend's home or office in another country. Thus, in roaming cases, it may happen that the UE is not configured with CAG IDs valid in the VPLMN and UE subscription data in HPLMN do not contain list of CAG IDs of the VPLMN. When the UE is registered in the VPLMN (or EHPLMN), the provisioned allowed CAG list must contain VPLMN related CAG IDs to allow the UE to access CAG cells in the VPLMN. This problem applies also to PNI-NPN for which CAG concept was first introduced, i.e., there is no mechanism specified to allow a roaming UE to use a PNI-NPN in the VPLMN if the CAG ID of VPLMN is not provisioned to the UE by HPLMN.

In this solution, the provisioning of a roaming UE and the serving network to enable access control in the VPLMN is provided. As part of this solution, it is assumed that CAG concept as defined for PNI-NPN is re-used for 5G Femto deployments to enable Femto access control in 5G.

6.2.2	Procedures

The following figure represents a high-level procedure of the solution.

Figure 6.2.2-1: Call flow for CAG ID(s) provisioning in the visiting network

1:	AF creates a request on visited CAG information which may include visited allowed CAG list and GPSI via AF. Optionally, the request may also include an expiry time in case the given information is only valid temporarily.

NOTE 2:	GPSI can correspond to UE's MSISDN.

NOTE 3:	Provisioning of visited CAG information to V-UDR can also be performed via OAM. For this case, the given procedure continues to step 5.

2:	The V-NEF receives the AF request and authorizes the request.

3:	V-NEF stores the translated information to V-UDR in a way that visited CAG information and UE identity are mapped and stored together.

4:	V-NEF responses back to AF regarding the request on visited CAG information.

65:	UE performs manual selection of CAG cell.

6:	UE sends initial registration request or mobility registration update message to the visited network.

7:	The AMF establishes AM Policy Association with V-PCF and provides GPSI of the UE. This can be done when a UE is accessing network via CAG cell. The V-PCF subscribes UDR using the GPSI to receive visited CAG information. The received information is provided to the AMF.

8:

The AMF generates Allowed CAG list for the UE by combining Allowed CAG list from the subscription and visited CAG information.

9:	AMF sends registration accept/reject message.

In case the registration request is rejected due to UE is not allowed to access this particular CAG cell of 5G Femto, then AMF sends the registration reject message to the UE along with a list of allowed CAG IDs. UE may use the received list of allowed CAG IDs e.g., to manually select another CAG cell from the list.

10:	If UE receives a registration accept, UE sends registration complete message.

* * * * End of Changes * * * *