SA WG2 Meeting #S2-162	S2-2405240

15 – 19 April, 2024, Changsha, China	(rev of S2-2404373)

Source:	InterDigital Inc.

Title:	KI#3, New Solution, NTZ enforcement in 5GC

Document for:	Approval

Agenda Item: 	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a solution on NTZ enforcement in 5GC.

# Discussion

This contribution proposes a solution for NTZ enforcement in 5GC addressing key issue #3.

The solution proposes to configure NTZ information dynamically per interested UAV’s expected location or route.

The solution proposes a method to enforce NTZ policy when UE is detected to be in the NTZ or nearby at Idle state and Connected state.

# 2	Proposal

It is proposed to adopt the following new solution in TR 23.700-59.

* * * * Start of Changes * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |
| --- | --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> | <Key Issue #3> |
| #1 | X |  |  |
| #2 | X |  |  |
| #3 | X |  |  |
| #4 |  | X |  |
| #5 |  | X |  |
| #6 |  | X |  |
| #7 |  |  | X |
| #8 |  |  | X |
| #9 |  |  | X |
| #X |  |  | X |

* * * * Next Changes (All Next Text) * * * *

## 6.X	Solution #X: NTZ Enforcement in 5GC

### 6.X.1	Key Issue mapping

This solution addresses KI#3, “Support of No Transmit Zones”.

### 6.X.2	Description

This solution assumes that NTZ enforcement is provided to the 5GC from USS dynamically. This solution is intended for 5GC to enforce NTZ information for the UAV dynamically so that UAV does not transmit in the NTZ after receiving NTZ enforcement.

In this solution, the NTZ enforcement request is initiated from the USS/UTM and sent to the UAS-NF in 5GC. The request includes the designated NTZ information (areas, spectrum, etc.) and target UAV identifiers (or all UAVs). The UAS NF maps the NTZ areas to network areas and locates the serving AMFs of the affected UAVs, and forwards the NTZ enforcement request to the serving AMFs.

The serving AMF may perform the following actions:

- Use the location service to determine whether the UAV is in or out of the NTZs.

- For those in the NTZs and in Connected mode, inform the SMF to deactivate the PDU sessions of the UAV (C2 communication may be exempted), and inform the RANs to not schedule UL transmissionsc for those UAVs.

Editor's note:	It is FFS whether and how to enforce NTZ for UAVs in Idle state.

### 6.X.3	Procedures

Figure 6.X.3-1: NTZ enforcement in 5GC

- The UAS NF receives the NTZ enforcement request from the USS/UTM. The request include the NTZ information and affected UAV Identifiers (or all UAVs). The NTZ information includes one or multiple NTZ areas, frequency bands that UAVs are not allowed to transmit, etc.

- The UAS NF query UDM and may use the UAV tracking mechanism to determine the location of the affected UAVs, whether they are in NTZs, and locate their serving AMFs.

Editor’s Note: It is FFS whether UAS NF needs to use the UAV tracking mechanism to determine the location of the affected UAV.

NOTE : This solution assumes that the affected UAVs have successfully registered at the network.

The following steps are repeated for each affected UAV.

- The UAS NF forwards the NTZ enforcement request to the serving AMFs.

- The AMF may further use location service to determine the location of the affected UAVs.

If the affected UAV is in Connected mode,

5a. The AMF may request the SMF to deactivate the resource for the PDU Sessions of the affected UAV.

6a. The AMF also notifies the affected UAV of the NTZ enforcement.

NOTE : If the affected UAV is in NTZ zone, step 6a performes only when downlink data reception in NTZs is allowed given the regulatory requirements are not violated. And the affected UAV shall not respond to the notification.

7a. The AMF may also send the serving RAN a NTZ enforcement request with NTZ info (e.g., NTZ area and restricted freq. band).

8a. Based on RAN’s discretion, the RAN may perform NTZ enforcement for the affected UAV (e.g. release the RRC connection of the UAV, etc).

Editor’s Note: RAN aspect is not in scope of SA2 and will be based on RAN decision.

### 6.X.4	Impacts on services, entities and interfaces

UAS NF:

- Supports handling of NTZ enforcement request, including forwarding the request to other NFs (e.g. AMF).

AMF

- Supports notification of NTZ enforcement/restriction to the affected UAVs using NAS signalling.

- Supports requesting the SMF to enforce the NTZ (e.g., deactivate PDU Sessions)

- Supports requesting the RAN to enforce the NTZ (e.g., not scheduling UL transmission)

- Reject service request for the reason of NTZ control.

RAN

- Supports NTZ enforcement (e.g., not scheduling UL transmission, release RRC)

UAV

-   Supports NTZ enforcement

* * * * End of Changes * * * *