3GPP TSG-WG SA2 Meeting #162	S2-2405299

# Changsha, China, April 15- April 19, 2024

Source: 	China Mobile

Title: 	Update on Solution #1: Edge computing handling by I-SMF

Document for: 	Approval

Agenda Item: 	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract of the contribution: Propose a update on Solution #1: Edge computing handling by I-SMF.

1	Discussion

This paper is to propose update on Solution #1.

It is propose to solve the Editor’s notes on Solution #1 in the following ways:

Editor's note:	How to enhance the I-SMF selection (compared with ETSUN) to satisfy the local handling of Edge Computing is FFS.

I-SMF selection takes the visited application into consideration compared with ETSUN. It is proposed to add related clarification and remove the Editor’s note.

Editor's note:	How to update the step 3 to support the PCF and I-SMF communication is FFS.

New procedure is added to address this issue. Please see clause 6.1.3.b.

Editor's note:	How to support the DNS message handling in this solution is FFS.

SMF sends the EASDF IP address to I-SMF, to help the direct interaction between I-SMF and EASDF.

2	Proposal

It is proposed to include the below changes into TR 23.700-49 v0.2.0.

- FIRST CHANGE

6.1	Solution #1: Edge computing handling by I-SMF

6.1.1	Key Issue mapping

This solution is related to the KI#1.

6.1.2	Functional Description

The architecture of this solution is described as Figure 6.1.2-1.

Figure 6.1.2-1: Architecture assumption

In this solution, I-SMF is used to handle the edge computing related traffic to be offloaded into local DN, EDI management, DNS message handling rules configuration or update, and UL CL/BP or L-PSA insertion. Therefore, the impact on central SMF is reduced.

In this solution, the I-SMF, ULCL/BP UPF and Local PSA UPF are introduced with reusing mechanism described in clause 5.34 of TS 23.501 [2]. The difference from the ETSUN mechanism is that, SMF also sends the address of I-SMF to PCF. When traffic offload policy or edge computing related policy is updated, the PCF sends the policy update to I-SMF, with SMF forwarding transparently, which also reduces the edge computing handling impact to central SMF.

6.1.3	Procedures

6.1.3.a	I-SMF Insertion Procedure

UL CL

UE

(R)AN

AMF

SMF

UPF

I-SMF

PCF

UDM

4. Npcf_SMPolicyControl_Update

5. Npcf_PolicyAuthorization_Update

6. Nsmf_PDUSession_Update

EASDF

NEF

1. Steps 1-10 of Addition of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF procedure, clause 4.23.9.1 of TS 23.502

2. I-SMF obtains EDI for local network, clause 6.1.3.e

3. Neasdf_DNSContext_Update

Figure 6.1.3.a-1: Procedure of I-SMF Insertion

- I-SMF invoke the SMF service to sends the I-SMF supported DNAI(s) to SMF in Node Level.

1.	Steps 1-10 of Addition of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF procedure is performed as described in clause 4.23.9.1 of TS 23.502 [3], with the following changes:

-	During the PDU Session Establishment, the SMF selects EASDF, and then SMF sends the EASDF IP address as DNS server address to the UE via PCO.

-	As described in clause 6.2.3.2.2 of TS 23.548[5], based on EAS information received from the EASDF, SMF may determine the DNAI, and trigger the I-SMF insertion. In order to avoid the impact, the SMF may send the target DNAI information to AMF and trigger the AMF to select the I-SMF as described in clause 4.23.5.4 of TS 23.502[3].

-	SMF sends the EASDF IP address to I-SMF, to help the direct interaction between I-SMF and EASDF.

2.	I-SMF obtains EDI for local network, as described in clause 6.1.3.c.

3.	I-SMF invokes Neasdf_DNSContext_Update  with the updated DNS message handling rules based on the EDI for local network.

4.	SMF sends the address of I-SMF to PCF via Npcf_SMPolicyControl_Update service, as well as the indication of supporting local handling of Edge Computing from SM subscription data.

The following steps 5-6 will be performed when traffic offload policy (e.g. forwarding local traffic to the L-PSA) is to be updated.

5.	When PCF gets the I-SMF address and the indication of supporting local handling of Edge Computing, PCF based on the DNN/S-NSSAI, or local configuration, may send policy container to SMF by invoking Npcf_PolicyAuthorization_Update service. The traffic offload policy or edge computing related policy is encapsulated in the policy container, as described in clause 6.1.3.b.

6.	The SMF forwards the policy container transparently to I-SMF via Nsmf_PDUSession_Update service. The I-SMF performs EAS (re)discovery and UPF (re)selection based on the traffic offload policy or edge computing.

UL CL

UE

(R)AN

AMF

SMF

UPF

I-SMF

PCF

UDM

EASDF

NEF

1. Steps 1-9 of EAS discovery procedure, clause 6.2.3.2.2of TS 23.548

2. Steps 2-6 of SMF triggered I-SMF selection, clause 4.23.5.4 of TS 23.502

3. Steps 2-10 of Addition of PDU Session Anchor and Branching Point or UL CL controlled by I-SMF, clause 4.23.9.1 of TS 23.502

4. Steps 2-6 of I-SMF insertion during PDU Session Establishment, clause 6.3.1.a

6.1.3.b	Processing AF requests for offloading policy create/update for I-SMF

UDR

PCF

NEF

AF

2.Nnef_TrafficInfluence_Create/Update

1. Creation of the AF request

SMF

I-SMF

3a. Storing/Updating the information

3b.Nnef_TrafficInfluence_Create/Update Response

4.Nudr_DM_Notify

5.Npcf_PolicyAuthorization_Update

6.Nsmf_PDUSession_Update

Figure 6.1.3.b-1: Procedure of processing AF requests for offloading policy create/update for I-SMF

1-4. Steps 1-4 of the procedure is the same as steps 1-4 of clause 4.3.6.2 of TS 23.502 [3], with the following change:

-	AF provides requested policy related parameters with the indication for Local DN (in step 1-2). And the requested policy related parameters and the indication for L-DN is stored in UDR(in step 3a) and sent to PCF(in step 4).

5.	If received the indication for L-DN, the PCF puts the traffic offload policy into a container and send to SMF.

6.	The SMF forwards the policy container transparently to I-SMF via Nsmf_PDUSession_Update service, in order to achieve that the L-DN related traffic offloading policy do not impact the SMF.

6.1.3.c	EDI for local network Management in the I-SMF

The EDI for local network provisioned from AF to 5GC, i.e. UDR, is same as the procedure defined in clause 6.2.3.4.2 of TS 23.548 [5].

The I-SMF subscribes to EDI for local network Change Notification from the NEF using the procedure same as one defined in clause 6.2.3.4.3 of TS 23.548 [5].

NEF

2.Nnef_EASDeployment_Subscribe Request

SMF

I-SMF

1.Nsmf_EventExposure Subscribe Request

3.Nnef_EASDeployment_Subscribe Response

4.Nsmf_EventExposure Subscribe Response

5.Nnef_EASDeployment_Notify Request

6.Nsmf_EventExposure Notify Request

7.Nsmf_EventExposure Notify Response

8.Nnef_EASDeployment_Notify Response

6.1.4	Impacts on existing services, entities and interfaces

I-SMF:

-	Handling the edge computing related traffic to be offloaded into local DN, EDI management, DNS message handling rules configuration or update, and UL CL/BP or L-PSA insertion. and control the edge computing handling for ULCL/BP,

-	Receiving the policy from PCF.

-	Receiving EDI for local network directly from NEF.

SMF:

-	Determining the target DNAI and triggering AMF to do the I-SMF selection and insertion based on the EAS information received from the EASDF.

-	Forwarding traffic offload policy or edge computing related policy transparently to I-SMF.

PCF:

-	Providing traffic offload policy or edge computing related policy in policy container and sending to I-SMF.

NEF:

-	Sending EDI for local network to I-SMF.

- End of CHANGE