3GPP SA WG2 Meeting #162	S2-2405310

Changsha, CHINA, April 15 – 19, 2024	(revision of S2-2404601)

Source:	LG Electronics

Title:	KI#2, Solution Update: EAS and local UPF (re)selection based on the N6 delay and the EAS load information

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a solution update to address KI#2.

6	Solutions

It is proposed to accept the following changes to TR 23.700-49.

* * * * Start of 1st Change * * * *

6.15	Solution #15: The local EASDF assist for the EAS and local UPF (re)selection based on the AF provided N6 delay and EAS load information.

6.15.1	Description

The 5GC not considering the network metrics(e.g. N6 delay) and the compute metrics(e.g. EAS load) has the possibility to provide suboptimal EAS address to the UE and result in redundant EAS rediscovery or Edge relocation.

The N6 delay for each DNAI which is not constant and the EAS load status information subject for frequent update needs consideration for the signaling and computation overhead aspect to be managed by the SMF having heavy impact for the centralized control in the current design.

This solution proposes the local EASDF to support EAS and local UPF (re)selection based on the N6 delay and the EAS load information from the AF. This assumes the AF can provide the N6 delay between candidate local PSA UPF(s) and EAS in the local DN and the EAS load information to be taken into account for the SMF to perform EAS and local UPF (re)selection with following enhancement:

-	The AF registers its available data to the EASDF via OAM configuration at NEF including the N6 delay and the EAS load information and notifies the information conditionally or periodically to the EASDF via NEF.

-	The local EASDF is configured to manage the N6 delay and the EAS load information and subscribes to the AF to get notified with the N6 delay and the EAS load information and may also be pre-configured or instructed by the SMF with condition to request EAS re-discovery to the SMF upon the updates from the AF.

6.15.2	Procedures

Figure 6.15.2.1: EAS re-discovery based on the N6 delay and the EAS load information

1.	The AF registers its available data (i.e. N6 delay and the EAS load information) to the NEF via OAM and the NEF updates its NF Profile to the NRF to register that it supports exposure of AF available data (i.e. N6 delay and EAS load information).

2-4.	The local EASDF is configured to manage the metric information and with necessary information to subscribe to the AF for the N6 delay and the EAS load information.

The local EASDF discovers the AF available data using the AF ID and the NEF to which the AF available data can be provided and subscribes to the AF via NEF to be notified for the N6 delay and the EAS load information.

5-6.	Based on the subscription in the step 3, the AF notifies the local EASDF conditionally or periodically for the event related to the N6 delay and the EAS load and provides the following information:

-	N6 delay information per DNAI.

-	EAS load information:

-	Indication for the EAS in overload status.

-	IP address of the EAS.

The AF is configured with the information needed for the measurement or has the knowledge for the EAS load and the N6 delay of candidate DNAI(s) with the existing traffic flows between candidate L-PSA(s) and the EAS in the local part of the DN and provides information to the local EASDF.

7-9.	During PDU Session Establishment, the SMF selects the local EASDF and EAS Discovery Procedure with EASDF is performed taking into account the N6 delay and the EAS load information available in the local EASDF.

Upon reception of the DNS response message from the UE, the EASDF reports EAS information to the SMF together with N6 delay and EAS load information if available to be taken into account for EAS discovery.

10-11.	The AF may update the local EASDF for the event related to the N6 delay and the EAS load information based on the subscription in the step 3.

12-13.	The EASDF can be pre-configured or instructed from the SMF to request for the EAS re-discovery jointly considering the updated N6 delay and EAS load information from the AF using Neasdf_DNSContext_Notify with extension to include target UE information.

The SMF sends PDU Session Modification Command to the UE for the EAS re-discovery.

6.15.3	Impacts on Existing Nodes and Functionality

AF:

-	Registers its available data at the NEF including the N6 delay and the EAS load information.

-	Sends notification to the EASDF via NEF for the event with the N6 delay and the EAS load information.

SMF:

-	Instructs the local EASDF to subscribe and manage the N6 delay and the EAS load information to be taken into account for EAS (re)discovery and provides necessary information for AF discovery and subscription.

-	Initiates the EAS re-discovery procedure and local UPF (re)selection by the request from the EASDF.

EASDF:

-	Discovers the AF available data via NEF to which the AF available data can be provided.

-	Subscribes to the AF via NEF to be notified for the N6 delay and the EAS load information as instructed by the SMF.

-	Receives notification from the AF with the N6 delay and the EAS load information and manage the information locally to be kept updated.

-  Requests the SMF to perform the EAS re-discovery for the UEs served by the EAS based on pre-configuration or instruction with conditions upon updates from the AF for the N6 delay and EAS load information.

NEF:

-	Subscribes for the N6 delay and EAS load information to the AF based on the EASDF subscription information.

-	Provisions the N6 delay and EAS load information provided by the AF to the EASDF.

* * * * End of Changes * * * *