3GPP TSG-WG SA2 Meeting #162	S2-2405600

April 15 – 19, 2024, Changsha, P.R.China

Title:	LS on Support of Regenerative-based Satellite Access

Response to:	-

Release:	Release 19

Work Item:	FS_5GSAT_ARCH_Ph3

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact Person:

#### Name:	Amy Zhang

E-mail Address:	amy.zhang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

As part of the Rel-19 study on 5G System enhancements for satellite access (FS_5GSAT_ARCH_Ph3), Key Issue #1 of TR 23.700-29 considers the support of Regenerative-based satellite access. SA2 has identified the following candidate mechanisms and assumptions, which need to be verified by RAN3:

- A procedure to handle the N2 and S1 connections when the eNB/gNB leaves the service area of an AMF/MME (e.g. when setting over the horizon) should be supported. Options e.g. disconnecting/suspending/performing configuration update of the N2/S1 connections are considered in SA2. It is up to RAN3 to determine the final option about whether to reuse existing or new mechanisms/procedures.

- If the eNB/gNB IP address changes due to soft feeder link switch, SA2 assumes that this case can be supported using the existing procedures.

- SA2 assumes that AMF/MME can treat the Mapped Cell ID as per rel-17.

2. Actions:

To RAN3:

ACTION: 	SA2 kindly asks RAN3 to consider the above and provide feedback.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#163			27 - 31 May 2024			Jeju, KR

3GPP SA2#164			19 - 23 August 2024			Maastricht, NL