SA WG2 Meeting #162	S2-2404054

Changsha, China, April 15 –19, 2024 	(revision of S2-2402254 merged with 2755 and 2369)

Source:	China Mobile, LG Electronics, CATT

Title:	New Solution on USS/UTM relocation

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes a new solution for Key Issue #1 "Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions".

1. Discussion

This contribution is related to KI#1: Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions, which including one detail requirement considering "The scenario of multiple USS/UTM serving different geographical areas corresponding to the UAV flight path". It is to be studied the interaction between multiple USS/UTMs and UAV (e.g. USS/UTM relocation) if multiple USS/UTMs are serving for different geographical areas which are along the same flight path for one UAV.

Therefore, there is a scenario where USS/UTM relocation is required due to the change in UAV mobility: for the location movement of UAV, the current USS/UTM may not be able to provide flight-related service after UAV move to the area out of its control. Considering the above scenario, the 5GS suggests to define solution to support USS/UTM relocation to guarantee the continuity of UAV flight.

2. Text Proposal

It is proposed to agree the following solution into TR 23.700-59.

Main changes from S2-2402254 are as below:

①	UUAA revocation procedure performed by old USS/UTM  Removed because UE deregistration or PDU Session release is not considered desirable for the UE in flight.

②	At UUAA Re-authorization procedure, the new USS/UTM informs the UAS NF that the UAV Re-authorization is due to USS/UTM relocation so that the UAS NF can progress this procedure initiated by the other USS/UTM than the USS/UTM serving the UAV previously.  Description added to "Step 4" (This info from new USS/UTM to UAS NF is considered necessary because of not performing UUAA revocation procedure.)

③	The USS/UTM subscribes to UAV presence monitoring events  "Step 1b" added based on S2-2402755 (CATT).

④	New USS/UTM may determine to trigger USS/UTM relocation and migrate the UAV context from the old USS/UTM  "Step 2b" added and "Step 3" modified based on S2-2402755 (CATT).

⑤	The UAS NF may inform the new USS/UTM that USS/UTM relocation is needed for the UAV if the UAV has left or is about to leave the service area of the old USS/UTM. This can be achieved based on configuration about which USS serves which geographical area  Description related to "Step 1a/1b" added to provide an alternative for making ④ work. The configuration is based on S2-2402369 (LGE).

# * * * * First change * * * *

6.X	Solution #X:  USS/UTM relocation

6.X.1	Key Issue mapping

This solution resolves KI#1.

6.X.2	Description

This solution addresses Key Issue #1 "Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions".

Considering the scenario of multiple USS/UTM serving different geographical areas corresponding to the UAV flight paths, there are some cases that the serving USS/UTM may need to be changed during the UAV flight due to UAV mobility. USS/UTM can trigger the re-selection of the USS/UTM based on the location change of the UAV. After the re-selection, the USS/UTM can migrate the UAV context to the new USS/UTM. The UAV Context includes e.g., subscribed events, subscriber addresses, UAS NF address, and the Notification Endpoint. For the subscribed events, new USS/UTM may re-initiate the subscription after the relocation.

NOTE:	Multiple USSs can serve overlapped area.

6.X.3	Procedures

Figure 6.X.3-1: Procedure for USS/UTM relocation

1a.	The USS/UTM (i.e. old USS/UTM or new USS/UTM) subscribes to UAV location reporting events to AMF/GMLC/LMF through UAS NF. The event reporting type could be periodic reporting. See clause 5.3.2 Procedure for UAV location reporting in TS 23.256 [2].

1b.	The USS/UTM (i.e. old USS/UTM or new USS/UTM) subscribes to UAV presence monitoring events whether the UAV is moving in or moving out of the service area of the USS/UTM that is set as monitoring area. See clause 5.3.3 Procedure for UAV presence monitoring in TS 23.256 [2].

NOTE 1:	It is assumed that if the new USS/UTM has knowledge of the UAV flight route (e.g. waypoints within its service area at specific time intervals), the new USS/UTM can subscribe to UAV location reporting events and/or UAV presence monitoring events for the UAV.

In step 1a or step 1b, if the UAV has left or is about to leave the service area of the old USS/UTM, the UAS NF may determine a new USS/UTM that can serve the UAV based on configuration about which USS serves which geographical area. In this case, the UAS NF may inform the new USS/UTM that USS/UTM relocation is needed for the UAV, so the UAV context needs to be retrieved from the old USS/UTM.

Step 2a or step 2b is executed.

2a.	If the old USS/UTM detects that the UAV has left or is about to leave its service area based on the location information provided in step 1, the old USS/UTM may determine to trigger USS/UTM relocation and find a new USS/UTM to serve the UAV.

2b.	If the new USS/UTM detects that the UAV has moved into its own service area based on the information provided in step 1, the new USS/UTM may determine to trigger USS/UTM relocation.

3.	After step 2a, the old USS/UTM triggers the process of migrating the UAV context to the new USS/UTM. After step 2b, the new USS/UTM triggers the process of migrating the UAV context from the old USS/UTM.

The UAV context includes e.g., subscribed events, subscriber addresses, UAS NF address, and the Notification Endpoint. For subscribed events, new USS/UTM may re-initiate the subscription to the subscriber address.

NOTE 2:	Interaction between old USS/UTM and new USS/UTM is out of scope of this specification.

4.	The new USS/UTM initiates UUAA Re-authorization procedure via UAS NF. For details, see clause 5.2.4.3 UUAA Re-authorization procedure in TS 23.256 [2]. In this procedure, the new USS/UTM needs to inform the UAV with the new USS/UTM address. In addition, the new USS/UTM needs to inform the UAS NF that the UAV Re-authorization is due to USS/UTM relocation so that the UAS NF can progress UUAA Re-authorization procedure initiated by the other USS/UTM than the USS/UTM serving the UAV previously.

5.	The new USS/UTM executes UUAA Re-authentication procedure, which will trigger the UAV to report the CAA level UAV ID. For details, see clause 5.2.4.1 UUAA Re-authentication procedure in TS 23.256 [2].

6.X.4	Impacts on Existing Nodes and Functionality

USS/UTM:

-	Supports USS/UTM relocation during the UAV flight due to UAV mobility.

UAS NF

-	Supports USS/UTM relocation.

# * * * * End of changes * * * *