SA WG2 Meeting #161	S2-2404151

April 15th – 19th, 2024, Changsha, China                            (revision of S2-2403668)

Source:	Apple, ETRI, China Telecom, Nokia, Nokia Shanghai Bell, LG Electronics, OPPO, NEC, KDDI

Title:	FS_MASSS Architectural Assumptions and Requirements for DualSteer

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes architectural assumptions and requirements for DualSteer for the FS_MASSS TR 23.700-54.

# 1	Discussion

This paper proposes architectural assumptions and requirements for DualSteer based on content of the FS_MASSS SID (SP-231802) and the DualSteer requirements in TS 22.261.

The objectives of the FS_MASSS SID, SP-2401315, include the following statements:

- The following scenarios are considered:

1.	Two NR/5GC accesses in a single PLMN (HPLMN or VPLMN) with each access being NR TN or NR NTN;

2.	Two NR/5GC accesses in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN) with each access being NR TN or NR NTN;

3.	NR/5GC access and E-UTRA/EPC access in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

4.	NR/5GC access and E-UTRA/EPC access in a single PLMN (HPLMN or VPLMN);

5.	PNI-NPN (integrated with the HPLMN or integrated with the VPLMN) and PLMN access (TN/NTN plus TN or NTN). This scenario assumes only non-simultaneous transmission.

- The study assumes there is no coordination in RAN between the two 3GPP access networks where the DualSteer Device is accessing simultaneously.

- For the PNI-NPN scenario, the subscriber is assumed to be a subscriber of the PNI-NPN.

- Each subscription / SUPI of the DualSteer Device is used to connect to only one of the 3GPP access at any given point in time.

- No impacts on network slicing features, i.e. network slicing features apply to each subscription / SUPI separately.

The Traffic steering and switching over two 3GPP access networks from TS 22.261, clause 6.50, include the following statements:

- Scenarios may also include traffic steering and/or switching across LTE/EPC and NR/5GC, with anchoring in 5GC.

- Traffic policies are intended to be in full control of the home network operator.

- The requirements can apply to different DualSteer device types (e.g., smartphones, IoT, UAV, VSAT devices).

- The requirements apply to a subscriber with two subscriptions/SUPIs, sharing one subscription profile from the same operator. For simultaneous transmission over two networks, a DualSteer device is assumed to include two separate UEs.

- Inter-PLMN requirements can apply also to PLMN-NPN scenarios assuming a PLMN-integrated NPN (NPN hosted by a PLMN or offered as a slice of a PLMN).

- For any particular service, at any given time, the DualSteer device shall transmit all traffic of that service using only a single 3GPP access network.

- Charging information should be collected for both 3GPP access networks; in case the two 3GPP access networks belong to different PLMNs, or a PLMN and NPN, a proper business/roaming agreement among network operators is assumed.

- The NPN is hosted by a PLMN or offered as a slice of a PLMN, data anchoring in the NPN, and a business/roaming agreement between the PLMN and the NPN operator (if different).

# 2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.2.0.

* * * * Start of Changes * * * *

### 4.1.1	Architectural Assumptions for DualSteer

-	The 5GC architecture as specified in TS 23.501 [3], TS 23.502 [4], and TS 23.503 [5] are regarded as the baseline for this study.

-	It is assumed that no coordination is required between the two 3GPP RAN the DualSteer Device may be connected to.

-	Each subscription (i.e. SUPI) is used to connect to only one of the 3GPP access networks at any given point in time.

-	No impacts are assumed for network slicing features, i.e. network slicing features apply to each subscription / SUPI separately.

-	Solutions are expected to demonstrate not to impact PLMNs that do not support DualSteer functionality.

-	Traffic policies are controlled by the home network operator.

-    For any particular service, at any given time, it is assumed that the DualSteer Device transmits all traffic of that service using only a single 3GPP access network.

-	Splitting functionality is not supported for DualSteer in any scenario.

* * * * Next Changes * * * *

### 4.2.1	Architectural Requirements for DualSteer

Service Requirements for DualSteer are specified in clause 6.50 of TS 22.261 [2].

The following scenarios shall be supported:

1.	Two NR/5GC accesses in a single PLMN (HPLMN or VPLMN) with each access being NR TN or NR NTN;

2.	Two NR/5GC accesses in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN) with each access being NR TN or NR NTN;

3.	NR/5GC access and E-UTRA/EPC access in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

4.	NR/5GC access and E-UTRA/EPC access in a single PLMN (HPLMN or VPLMN);

5.	PNI-NPN (integrated with the HPLMN) and PLMN access (TN/NTN plus TN or NTN). This scenario assumes only non-simultaneous transmission.

The following architectural requirements are applicable to this study:

-	Scenario 5) requires the subscriber to be a subscriber of the PNI-NPN. In this scenario the PNI-NPN is hosted by a HPLMN. For sessions subject to traffic steering and/or to traffic switching, this scenario requires data anchoring in the PNI-NPN.

-	The subscriber shall have two subscriptions/SUPIs from the same operator.

-	In case the two 3GPP access networks belong to different PLMNs, or a PLMN and PNI-NPN, a proper business/roaming agreement among network operators is required.

-	The 5GS shall be able to identify and link the two SUPIs that are part of the subscription used for DualSteer services.

-	The 5GS shall be able to differentiate between the two connections of the DualSteer device.

-	The 5GS shall be able to minimize impacts to CN and OAM.

-	The 5GS shall be able to support mechanisms to minimize service interruption when switching a DualSteer Device's user data, for one or multiple services, between two 3GPP access networks.

* * * * End of Changes * * * *