SA WG2 Meeting #162	S2-2404309

15-19, April 2024, Changsha, P. R. China

Source:	OPPO

Title:	FS_UIA_ARC KI#4, New Sol: devices behind UE or 5G-RG

Document for:	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC /Rel-19

Abstract of the contribution: This paper proposes a new solution for KI#4 of the FS_UIA_ARC.

# 1	Discussion

This key issue will study whether and how 5GC identifies individual non-3GPP devices connecting behind a UE or 5G-RG and whether and how to provide policy control for the traffic associated with individual non-3GPP devices.

The use case that is associated with this key issue is the case where non-3GPP devices behind a UE or 5G-RG need to be identified.

The objective of this key issue is how an identifier is used by the network to control the traffic to/from UE or 5G-RG when the traffic is associated with the non-3GPP devices. This objective differs from existing support for AUN3 devices in TS 23.316 [7] because the objective is to enable the non-3GPP devices to be identified and to use only the subscription of the UE or 5G-RG to access the 5GC (i.e. the UE or 5G-RG should have to maintain only a NAS Context itself and not for each non-3GPP device).

Solutions to this key issue will address:

-	whether and how the 5GC identifies individual non-3GPP devices connecting behind a UE or 5G-RG. (e.g. in order to charge the individual non-3GPP devices),

-	whether and how to provide policy control for the traffic of individual non-3GPP devices connecting behind a UE or 5G-RG. Including whether and how to trigger policy control for the traffic of individual non-3GPP devices via PCF and NEF APIs, including whether and how to support concurrent services with differing QoS requirements launched by the different non-3GPP devices, and

-	whether and how the operator can optionally restrict the number of simultaneously active User Identifiers per UE or 5G-RG.

NOTE 0: The User Identifiers in the above bullet means the identifiers of the devices behind gateway UE or 5G-RG.

NOTE 1:	Changes to the layer 1 or layer 2 protocols of non-3GPP devices are not in scope of this study. It is assumed that the non-3GPP devices do not support 5G-AKA authentication procedures nor separate NAS connections with the 5GC for each non-3GPP device (e.g. like for AUN3 devices).

NOTE 2:	Conclusions related to an 5G-RG should be shared with the Broadband Forum (BBF) and CableLabs.

NOTE 3:	Solutions for Ethernet PDU Session should work in the presence of randomized MAC addresses.

KI#4 of FS_UIA_ARC has been agreed and documented in TR 23.700-32, captured in the text box below. This contribution proposes a solution for KI#4 of FS_UIA_ARC.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-32 V0.2.0.

* * * * Start of Changes * * * *

## 6.X	Solution #X: Non-3GPP Devices Behind UE or 5G-RG support

### 6.X.1	Key Issue mapping

This is a solution for KI#4.

### 6.X.2	Description

The proposed solution addresses how 5GC identifies individual non-3GPP devices connecting behind a UE or 5G-RG and how to provide policy control for the traffic associated with individual non-3GPP devices. The proposed solution meets the architecture assumptions specified in clause 4.1. In this solution, the following architecture principles apply.

Non-3GPP Device Behind UE/5G-RG (N3DBU) is a subscribed service, and a subscriber needs to coordinate with the operator to subscribe for N3DBU service. When a user subscribes for the N3DBU service, the N3DBU ID and associated subscription data may be allocated by the operator. The N3DBU-capable UEs/5G-RG may be provisioned with appropriate URSP rules to enable the non-3GPP devices to route the non-3GPP device traffic based on URSP rules.

N3DBU ID is introduced as a unique identifier within the home PLMN. It is provisioned by 5GS operators and is recognized by 5GS. An N3DBU ID is only allowed to be actively used by one Non-3GPP device at any time.

Each N3DBU ID is associated with pre-defined QoS and policy rules.

Upon N3DBU-capable UE or 5G-RG registration with the 5GC, the list of subscribed N3DBU IDs is populated from the AMF to the UE or 5G-RG. Additionally, N3DBU IDs can be stored/updated/removed based on UDM decisions.

Upon connecting a non-3GPP device that is intended to be associated with an N3DBU ID to the UE or 5G-RG, the UE or 5G-RG should provide means to bind the connected non-3GPP devices with N3DBU ID. This binding process can be performed either manually by a user or automatically by the UE or 5G-RG with subscription to the N3DBU service.

NOTE 1: 	The binding process is implementation specific. For instance, subscribers could specify the binding by themselves through a GUI provided by the UE, which allow them to select the desired non-3GPP device to be bound with the N3DBU ID.

Alternatively, if the N3DBU ID is already associated with specific non-3GPP devices, identified by certain physical parameters or identifiers, the UE may automatically bind the non-3GPP device with the N3DBU ID upon its connection to the UE or 5G-RG.

N3DBU ID is introduced as one of Traffic Descriptors. And N3DBU-capable 5G-RG or UE can recognize URSP rules including N3DBU ID.

It is proposed to store information related to the subscribed N3DBU IDs, such as the slice info or PDU session info, QoS settings, and a list of subscribed N3DBU ID, etc. within the subscriber’s subscription data in UDR.

The N3DBU IDs may be associated with subscriber category identifiers (e.g., gold, silver, premium etc.).

URSP rules should be provided to the 5G-RG or UE to indicate how to map N3DBU IDs to the parameters of the PDU Session, DNN, and S-NSSAI, used to carry the traffic of corresponding non-3GPP devices.

Differentiation of charging and QoS may be provided via PCC rules (for different service flows) related with dedicated PDU Sessions for non-3GPP devices.

### 6.X.3	Procedures

### 6.X.3.1	Identification

Figure 6.X.3.1-1 depicts high level procedures to show how 5GC identifies individual non-3GPP devices connecting behind a UE or 5G-RG.

Figure 6.X.3.1-1: Identification of non-3GPP devices connecting behind a UE or 5G-RG

- The 5GS operators provision a list of N3DBU IDs.

- In Steps 1a-1c, the UE or 5G-RG follows general registration procedures specified in clause 4.2.2.2.2 of TS 23.502, except for UEs that indicate N3DBU ID support and are subscribed to N3DBU services, in which case the AMF may provide the UE with a list of subscribed N3DBU IDs.

AMF may retrieve the N3DBU IDs and their related subscription data using Nudm_SDM_Get service operation.

- AMF selects a PCF that supports N3DBU ID related policies. During the UE Policy Association Establishment procedure, the UE may receive UE context policy control subscription information related to the N3DBU ID.

- When a subscriber connects a non-3GPP device intended for association with a configured N3DBU ID to the UE or 5G-RG, the UE or 5G-RG is responsible for binding the connected non-3GPP device with the configured N3DBU ID.

The UE should maintain the associations between the configured N3DBU ID and the connected non-3GPP devices.

- UDM may decide to store/update/remove one or more N3DBU IDs associated with the UE/5G-RG's subscription. It may update information related to N3DBU IDs stored in UDR using Nudr_DM service operations.

Based on updates received from the UDM, the UE will store, update, or remove N3DBU IDs and their related configurations locally. Additionally, the PCF, AMF, and SMF will be informed to update their records with the latest N3DBU ID-related information.

### 6.X.3.2	N3DBU Admission Control

An N3DBU Admission Control Function (N3ACF) is introduced to keep track of the current number of simultaneously active N3DBU ID per UE or 5G-RG. The N3ACF also maintains a list of N3DBU IDs actively used by the UE/5G-RG that is subject to N3DBU Admission Control (N3AC). An N3DBU ID is only allowed to be actively used by one N3DBU device at any time.

The N3DBU Admission Control Function (N3ACF) is provisioned with parameters specifying the maximum number of simultaneously active N3DBU ID allowed per UE or 5G-RG.

The AMF triggers a request to N3ACF when the number of simultaneously active N3DBU ID used by the UE or 5G-RG is changing.

In scenarios where the limit of maximum simultaneously active N3DBU ID is reached, the UE may be notified to deny further non-3GPP device connections requesting N3DBU ID. This could be enforced on UE through various means, such as withholding IP address allocation to the additional non-3GPP devices being associated with a N3DBU ID.

Figure 6.X.3.2-1 describes high-level procedures for N3ACF service operation.

Figure 6.X.3.2-1: N3DBU Admission Control Procedures

- The AMF is informed if the number of simultaneously active N3DBU ID used by the UE or 5G-RG is changing.

- If the AMF is not aware of which N3ACF to communicate, it may perform N3ACF discovery. The AMF triggers a check and update for the number of simultaneously active N3DBU ID.

- The AMF sends an Nn3acf_N3AC_NumberofN3DBUUpdate_Request message to the N3ACF, including the N3DBU ID and UE/5G-RG ID in the message.

- The N3ACF determines whether the number of simultaneously active N3DBU IDs has reached the limit and updates the list of N3DBU IDs actively used by the UE/5G-RG.

- The N3ACF returns an Nn3acf_N3AC_NumberofN3DBUUpdate_Response message, including a Result indication per UE/5G-RG.

- The UE/5G-RG is notified of the admission control result and take actions to either deny or allow N3DBU connections.

### 6.X.3.3	Policy Aspects

To send PDU of a non-3GPP device associated with an N3DBU ID, the upper layers of the 5G-RG or UE require information on the PDU session (e.g., PDU address) via which to send the PDU of the non-3GPP device.

The mapping of local network resources and 5GC QoS is configured in the 5G-RG or UE. If N3QAI together with QoS rule information is provided, the 5G-RG or UE may reserve resources in the non-3GPP network behind the 5G-RG or UE.

N3DBU ID in the Traffic Descriptor of the URSP rule is matched against the N3DBU ID that the non-3GPP device is associated with.

IP descriptors are matched against header information contained in IP packets sent by non-3GPP devices bound with the N3DBU Device ID.

Non-IP descriptors are matched against header information contained in Ethernet frames sent by the non-3GPP devices associated with the N3DBU ID. It is further assumed that the UE or 5G-RG maintains a mapping between the randomized MAC and the real physical MAC of the non-3GPP device.

When a URSP rule is determined to be applicable for a given N3DBU ID, the UE should select a Route Selection Descriptor within the URSP rule in the order of the Route Selectin Descriptor Precedence.

The UE/5G-RG may re-evaluate the URSP rules, to check if the change of the association of N3DBU ID to a PDU session is needed following the principles defined in TS 24.526.

Differentiation of charging and QoS may be provided via PCC rules (for different service flows) related with dedicated PDU Sessions for non-3GPP devices.

Figure 6.X.3.3-1 describes high-level procedures for N3DBU service operation.

Figure 6.X.3.3-1: N3DBU Policy Aspects Procedures

- UE or 5G-RG with subscription to N3DBU service has completed registration and got a list of configured N3DBU IDs.

- For every newly detected non-3GPP device by N3DBU-capable UE or 5G-RG, the association between the target non-3GPP device and N3DBU ID is established.

- UE or 5G-RG evaluates the URSP rules associated with this N3DBU ID in the order of Rule Precedence and determines if the N3DBU ID is matching the Traffic Descriptor of any URSP rules.

- The UE or 5G-RG determines whether to establish a new PDU session or use one of the existing PDU session(s) based on the URSP rules which include S-NSSAIs, if any, or based on UE local configuration, as described in clause 4.2.2.3 of TS 24.526.

If UE determine to establish a new PDU session, it should include N3DBU ID in N1 SM container for Nsmf_PDUSession_CreateSMContext Request.

- In Steps 4a-4d, based on the N3DBU ID, SMF might retrieval/subscription for updates from N3DBU ID policy control subscription data from UDR.

If frame route is used, SMF get frame route info data from Framed Route information.

SMF may perform an SM policy Association Establishment procedure to establish an SM policy association with the PCF and get the default PCC rules for the PDU Session.

When one or several of the QoS parameters exchanged between the UE and the network associated with the N3DBU ID are modified, PDU Session modification procedures will be applied.

UE or 5G-RG then continue PDU Session procedures as described in clause 4.3 of TS 23.502.

- If a specific resource is required to be provisioned to support N3DBU service by using AF, e.g., to meet the non-3GPP device’s requested subscriber category, the procedure for clause 4.15.6.6 or clause 4.15.6.7a of TS 23.502 may be used.

The AF provides guidance for URSP generation to the 5GC. The AF uses a UE/5G-RG identifier (i.e., GPSI) as the target UE. The AF request contains DNN, S-NSSAI allocated to the N3DBU ID to meet the non-3GPP devices’ requested subscriber category. The NEF authorizes the request received from the AF and stores the information in the UDR as “Application Data” service specific subscription info sub date type.

The procedures specified in clause 4.15.6.7a of TS 23.502 may be used to authorize the AF request by the UDM.

### 6.X.4	Impacts on services, entities and interfaces

UE or 5G-RG

Support Registration Request with indication of N3DBU support

Handle N3DBU ID related URSP rules

AMF

UE Policy Association Establishment with UE context subscription information related to N3DBU IDs

SMF

SM Policy Association Establishment or SMF initiated SM Policy Association Modification for N3DBU ID relate polices.

UDM

Make decision to store/update/remove N3DBU IDs in the subscription.

UDR

Store/update/remove N3DBU IDs in the subscription data.

PCF

Support of N3DBU ID related URSP rules.

N3ACF

New NF to provide N3DBU ID admission control.

* * * * End of Changes * * * *