SA WG2 Meeting #162	S2-2404589

Changsha, China, April 15 – 19, 2024	(revision of S2-240xxxx)

Source: 	LG Electronics

Title: 	KI#2, New Sol: Inference procedure for the Vertical Federated Learning between NWDAF(s) and AF(s)

Document for: 	Approval

Agenda Item: 	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This contribution proposes to New Solution to KI#2 of FS_AIML_CN.

1	Discussion

Regarding Key Issue #2: 5GC Support for Vertical Federated Learning captured in clause 5.2 of TR 23.700-84, this paper proposes a new solution. Related use cases are use cases #4 and #5.

2 Text Proposal

It is proposed to agree the following changes into TR 23.700-84.

* * * * Start of 1st Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues and Use Cases

|  | Key Issues |  |  |  | Use cases (optional) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 5 | 6 |
| #1 | X |  |  |  |  |  |  |  |  |  |
| #2 | X |  |  |  |  |  |  |  |  |  |
| #3 | X |  |  |  |  |  |  |  |  |  |
| #4 | X |  |  |  |  |  |  |  |  |  |
| #5 | X |  |  |  |  |  |  |  |  |  |
| #6 | X |  |  |  |  |  |  |  |  |  |
| #X |  | X |  |  |  |  |  | X | X |  |

* * * * Start of 2nd Change * * * *

!! All New Texts !!

6.X	Solution #X: Inference procedure for the Vertical Federated Learning between NWDAF(s) and AF(s)

6.X.1	Description

This solution resolves KI#2: 5GC Support for Vertical Federated Learning, focusing on the procedures for VFL Inference.

While each party in HFL uses the trained global model to make inferences, the parties in VFL have to collaborate to make inferences and each party may have only a sub-model.

The ML Model Training for VFL can be conducted by the NWDAF containing MTLF and/or AF(s), but the inferences are conducted by the NWDAF(s) containing AnLF and/or AF(s). Therefore, to collaborative VFL Inference, the NWDAF(s) containing AnLF and/or AF(s) need to be aware whether the collaboration is needed to make inferences, and a coordinator need to discover and select participants (i.e., NWDAF(s) and/or AF(s)) for VFL Inference, and aggregate the intermediate inference results from other participants. Also, each NWDAF containing AnLF participating the inference needs to provision ML Model for VFL Inference from NWDAF containing MTLF.

The inferences can be initiated by AF or NWDAF containing AnLF. For both cases, we assume that the AF itself provisions its own ML Model for VFL Inference.

To support this procedure, this solution proposes the following VFL Inference Capabilities in NF profile for NWDAF containing AnLF and AF:

-	VFL Inference Coordinator (or Server) and/or

-	VFL Inference Participant (or Client)

In Inference phase, the capabilities do not need to consider whether the participants have labels or not, we only consider two capabilities that one is the VFL Inference Coordinator (VFL Server) which can aggregate the intermediate inferences from participants and make final inference result, and the other is VFL Inference Participant (VFL Client) which can provide local inference result to VFL Inference Coordinator.

Also, VFL ML Model may have different ML Model according to its role (VFL Inference Coordinator or Participant) (e.g. split-VFL). When provision ML Models for VFL Inference, the NWDAF containing AnLF may include VFL Inference Coordinator or VFL Inference Participant Indication when request ML Model provisioning to NWDAF containing MTLF. NWDAF containing MTLF take into account this information when provisioning ML Model for VFL Inference Coordinator or VFL Inference Participant.

In addition to the VFL Inference capabilities for NWDAF containing AnLF, it is proposed to use new service operations which can be used for collaborative VFL Inference and preparation before the VFL Inference between NWDAF(s) containing AnLF and AF(s).

When the VFL Inference Coordinator discovers the VFL Inference Participants, it considers required features of the VFL Inference Participants such as NF Set IDs or NF types, or Area of Interest.

Before the training, VFL Inference Coordinator check whether the VFL Inference Participants can meet VFL Inference requirement including Data Availability requirement such as a list of Event IDs of the local data. The Data Availability requirement can be used by the Coordinator to determine the participants which have different feature spaces.

6.X.2	Procedures

6.X.2.1	Inference Procedure Initiated by the NWDAF

Figure 6.X.2.1-1: Inference procedure for the VFL initiated by NWDAF

0.	Each NWDAF containing AnLF and AF which support VFL Inference is registered to the NRF with its NF Profile, which includes Analytics ID(s), Address information of NWDAF, Service Area, VFL Inference capability and Time interval supporting VFL Inference.

NOTE 1:	When the NWDAF initiates the VFL, multiple AFs can be involved as described in use case#4.

- NF Consumer request Analytics to NWDAF containing AnLF which have VFL Inference Coordinator capability.

NOTE 2:	The NF consumer can be another AnLF which does not support VFL Inference capability or VFL Inference capability with only VFL Inference Participant.

2.	The NWDAF containing AnLF determines that VFL Inference is needed with other participants (NWDAF(s) and/or AF(s)) based on the Analytics ID, supported features, or local configuration. If the NWDAF containing AnLF does not have trained ML Model for VFL Inference for the requested Analytics ID, NWDAF containing AnLF need to retrieve ML Model from the NWDAF containing AnLF for the requested Analytics ID.

NWDAF containing AnLF (VFL Inference Coordinator) may need to discover and select NWDAF containing MTLF before the ML Model provisioning for the requested Analytics ID.

The ML Model Provisioning request may additionally include VFL Inference Coordinator Indication. The MTLF containing AnLF considers this indication to determine ML Model for VFL Inference Coordinator and provide ML Model to the AnLF in the ML Model Provisioning Response (MLModelProvision Notify or MLModelInfo Response).

3.	The NWDAF containing AnLF discovers candidate participants (NWDAF(s) and/or AF(s)) by querying the NRF. The discovery request includes Analytics ID, VFL Inference capability for AnLF (VFL Inference Participant) including the support of VFL Inference service operation, and The NF Set ID or NF Type of a data source. The data source information can be used by the coordinator to determine the participants for required features of the analytics.

4.	Before initiating the inference with discovered participants, the Coordinator NWDAF containing AnLF checks whether the discovered candidate participants (NWDAF(s) and/or AF(s)) can meet the VFL Inference requirement (e.g. Analytics ID, ML Model Interoperability information, Data Availability requirement, VFL Inference Availability time requirement (time span needed for the VFL Inference process), etc.). Data Availability requirement such as a list of Event IDs of the local data can be used by the Coordinator to determine the participants which have different feature spaces. A new service operation is used for this procedure and include VFL Inference Correlation ID, VFL Inference preparation flag.

If the AF is untrusted, NEF perform the translation and NEF need to support new service operation for VFL Inference.

5.	When the candidate participant AnLF or AF receives the VFL Inference preparation request, it checks whether it can meet the VFL Inference requirement and/or successfully provision the ML Model for VFL Inference and decide whether to join the VFL Inference process. If the participant AnLF or AF cannot provide Intermediate Inference, it can reject to join the VFL Inference process in the response at step 6.

If the candidate NWDAF containing AnLF participating the inference does not have ML Model for VFL Inference, it retrieves ML Model for VFL Inference from NWDAF containing MTLF. NWDAF containing AnLF (VFL Inference Participant) may need to discover and select NWDAF containing MTLF before the ML Model provisioning for the request Analytics ID.

The ML Model Provisioning request include Inference Input Data information and VFL Inference Participant Indication. The Inference Input Data information is related with the supported feature spaces of the NWDAF containing AnLF and may also be used for the NWDAF containing MTLF to select the ML Model for the NWDAF containing AnLF. The NWDAF containing MTLF considers VFL Inference Participant Indication to determine ML Model for VFL Inference Participant and provide ML Model to the NWDAF containing AnLF in the ML Model Provisioning Response (MLModelProvision Notify or MLModelInfo Response).

6.	VFL participants (NWDAF(s) containing AnLF and/or AF(s)) response/notify to the inference preparation request/subscribe whether they can join the VFL Inference or not.

7.	The Coordinator NWDAF finally determines VFL participants for inference.

8.	The Coordinator NWDAF sends Inference request/subscribe with new service operation including VFL Inference Correlation ID.

9.	The participants (NWDAF(s) and AF) perform inference and response/notify with intermediate inference results.

10.	The Coordinator NWDAF aggregate the intermediate inference results from the NWDAF(s) containing AnLF and/or AF(s) and generate the final inference result.

11.	The Coordinator NWDAF provides the analytics to the NF consumer with Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify.

6.X.2.2	Inference Procedure Initiated by the AF

If the AF wants to know the analytics and the AF can act as a coordinator, the AF can initiate the VFL Inference procedure.

If the AF cannot act as a Coordinator, the procedure of clause 6.X.2.1 applies and the AF acts a NF consumer.

Figure 6.X.2.2-1: Inference procedure for the VFL initiated by AF

0.	Each NWDAF containing AnLF which support VFL Inference registered to NRF with its NF Profile, which includes Analytics ID(s), Address information of NWDAF, Service Area, VFL Inference capability and Time interval supporting VFL Inference.

NOTE 1:	When an AF initiates the VFL, then only one AF is involved as described in use case#4.

1.	The AF determines that VFL Inference is needed with other participants (NWDAF(s) containing AnLF) based on the Analytics ID, supported features, or local configuration. We assume that the AF itself provisions its own ML Model for VFL Inference.

The AF discovers candidate participants by querying the NRF. The discovery request includes Analytics ID, VFL Inference capability for AnLF (VFL Inference Participant) including the support of VFL Inference service operation, and The NF Set ID or NF Type of a data source. The data source information can be used by the coordinator to determine the participants for required features of the analytics.

2-4.	The same procedure of step 4 to step 6 in Figure 6.X.2.1-1 applies without the interaction with participant AF.

5.	The Coordinator AF finally determines VFL participants for inference.

6.	The Coordinator AF sends Inference request/subscribe with new service operation including VFL Inference Correlation ID to the participants.

7.	The participants (NWDAF(s) containing AnLF) perform inference and response/notify with intermediate inference results.

8.	The Coordinator AF aggregate the intermediate inference results from the NWDAF(s) containing AnLF and generate the final inference result.

6.X.3	Impacts to Services, Entities and Interfaces

NWDAF:

-	Support new Service operation between NWDAF(s) containing AnLF and/or AF(s) for VFL Inference (e.g., Nnwdaf_MLInferenceInfo_Request/Response, Nnwdaf_MLInference_Subscribe/Notify)

-	New capability indication for AnLF (VFL server or VFL coordinator, VFL client or VFL participant)

-	MTLF can provide the ML Model for VFL Inference with additional consideration of the VFL Inference capabilities.

NEF:

-	Support new Service operation with AF(s) for VFL Inference (e.g., Nnef_MLInferenceInfo_Request/Response, Nnef_MLInference_Subscribe/Notify).

AF:

-	Provision ML model for VFL training and/or inference.

-	Support new service operation for VFL Inference provided by NEF.

* * * * End of Changes * * * *