SA WG2 Meeting S2#162	S2-2404597

April 15 – 19, 2024, Changsha, PR China	revision of S2-24xxxxx

Source:	Nokia

Title:	KI#8: New solution on Support of IMS Avatar Communication without using data channel

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes a new solution to KI#8 without using data channel.

# Introduction

This contribution proposes a solution to KI#8 on “Support of IMS avatar communication” including architectural aspects.

# 2	Proposal

It is proposed to add following text to TR 23.700-77.

# * * * * First change* * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  |  |
| 2 | X |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |  |
| 6 |  | X |  |  |  |  |  |  |
| 7 | X | X |  |  |  |  |  |  |
| 8 |  |  | X |  |  |  |  |  |
| 9 |  |  | X |  |  |  |  |  |
| 10 |  |  |  | X |  |  |  |  |
| 11 |  |  |  | X |  |  |  |  |
| 12 |  |  |  | X |  |  |  |  |
| 13 |  |  |  | X |  |  |  |  |
| 14 |  |  |  |  |  | X |  |  |
| 15 |  |  |  |  |  | X |  |  |
| 16 |  |  |  |  |  | X |  |  |
| 17 |  |  |  |  |  |  |  | X |
| Y |  |  |  |  |  |  |  | X |

# * * * * Next change* * * *

## 6.X	Solution #Y: Solution for support of IMS avatar communication without using data channel

DAC

MF

IMS AS

S-CSCF

P-CSCF

IMS-AGW

IBCF

TrGW

UE-B

Terminating IMS network

DAC1

DC2

ISC

Mw

Gm

Iq

Mx

Ix

Mb

IMS HSS

N71

UE-A

DAC

MF

IMS AS

S-CSCF

P-CSCF

IMS-AGW

IBCF

TrGW

UE-B

Terminating IMS network

DAC1

DC2

ISC

Mw

Gm

Iq

Mx

Ix

Mb

IMS HSS

N71

UE-A

Originating IMS network

Originating IMS network

6.X.1	Description

Figure 6.X.1-1: Architecture to support IMS avatar communication without DC

To support of IMS avatar communication, the architecture is enhanced as follows:

Editor's note:	Whether existing DC2 can be re-used (e.g. by adding a new service) or a new DC2’ is needed is FFS.

Digital Asset Container (DAC):

-	The Digital Asset Container is used for storing and retrieving the avatar metadata (e.g. including avatar representation) from 5GC. DAC can be internal to the PLMN, e.g., a new network function, or part of a network function (HSS, UDM, IMS AS), a XR application server or a web server within the IMS and/or 5GC; or DAC can be external to the PLMN, e.g., a webserver.

Editor's note:	The definition and nature of the reference point DAC1 between DAC and MF (e.g., to download the avatar representation) is FFS.

New information elements are defined as follows:

Avatar metadata:

-	Avatar metadata includes the avatar representation stored in a file, which can be accessed for example via a URL. The avatar metadata, or a reference of the avatar metadata (e.g., identifier), is associated per subscriber and/or per subscriber’s user identifier. Avatar representation corresponds to a 2D/3D model of a face, full body, or a portion of the body. Additional metadata can be associated with the avatar representation as part of the avatar metadata, e.g., format, resolution, access/usage rights & conditions, date of creation, licensing, etc.

Avatar-id:

-	Avatar metadata is addressable via an Avatar-id. It can be an integer, string, UUID, or a URI (URN or URL). Other means such as APIs, or via a predefined server (e.g. DAC) can be used to retrieve the avatar metadata based on the Avatar-id.

-	It is assumed that Avatar-id are associated per subscriber and/or per associated subscriber’s User Identity in the IMS HSS (i.e., assuming multiple User Identities per subscriber, e.g., personal and/or professional).

Avatar media:

-	Avatar media is the user/media plane data related to an avatar animation transmitted between two UEs. It can be only representative of real-time face/gesture movements or a combination of face/gesture movement and the avatar metadata and is used to render the animated avatar.

Editor's note:	The avatar-related terminology will be updated once agreed upon.

This solution assumes that Avatar-id is provided in SIP/SDP and Avatar metadata are downloaded to UE without using data channel. STIR/SHAKEN mechanisms can be used to protect such information.

This solution covers three scenarios with different assumptions:

1.	Network-centric avatar rendering

2.	UE-A centric avatar rendering

3.	UE-B centric avatar rendering

Scenarios 1 and 2 assume that regular Audio/Video media is exchanged via RTP with UE-A and UE-B, i.e. UE-A and UE-B do not need to support any new media types. However, an indication in SIP/SDP that UE-A wants to use avatars (even if not specifying a specific Avatar ID) is assumed in such scenarios.

Scenario 1 further assumes that MF provides face detection functionality and can detect and replace a face in a video stream by an avatar. It also assumes that UE-A indicates the need for network-based rendering.

Scenario 3 assumes that UE-A and UE-B support avatar media as described above. When avatar media is used in SIP/SDP, an avatar call is assumed, even if no specific Avatar ID is indicated by UE-A.

### 6.X.2	Procedures

#### 6.X.2.1.	Scenario 1: Network centric IMS avatar call flow using SIP/SDP

UE-A

P/I/S-CSCF/IMS-AGW

IMS AS

MF

DAC

Terminating IMS Core

UE-B

1. Decide to start avatar session with network rendering based on its status + select Avatar-id

2. INVITE (Audio, Video, “Avatar” network rendering indication + UE-A Avatar ID?)

6. Request Avatar representation With Avatar ID, or download info

12. Send Audio/Video media over RTP

13. Rendering of avatar ID (e.g. replacing face)

14. Send Audio/Video media of UE-A over RTP (e.g. with avatar replacing face)

15. Send Audio/Video media of UE-A over RTP (e.g. with avatar replacing face) [UE-A’s feedback of own avatar]

9. INVITE (Audio, Video, “Avatar” indication, (with UE-A Avatar ID?))

7. Response with Avatar representation content

10. 200 OK (Audio, Video [, “Avatar” indication ])

11. 200 OK (Audio, Video , “Avatar” network rendering indication)

5. Request Avatar netw rendering (UE-A Avatar id, Session info incl. download info)

Originating Side

Terminating Side

IMS HSS

8. Confirm avatar network rendering

3. Avatar ID retrieval/validation req (UE A IMPU/IMPI ID, UE-A Avatar ID?)

4. Avatar ID retrieval/validation resp (incl. download info, default UE-A avatar id?)

UE-A

P/I/S-CSCF/IMS-AGW

IMS AS

MF

DAC

Terminating IMS Core

UE-B

1. Decide to start avatar session with network rendering based on its status + select Avatar-id

2. INVITE (Audio, Video, “Avatar” network rendering indication + UE-A Avatar ID?)

6. Request Avatar representation With Avatar ID, or download info

12. Send Audio/Video media over RTP

13. Rendering of avatar ID (e.g. replacing face)

14. Send Audio/Video media of UE-A over RTP (e.g. with avatar replacing face)

15. Send Audio/Video media of UE-A over RTP (e.g. with avatar replacing face) [UE-A’s feedback of own avatar]

9. INVITE (Audio, Video, “Avatar” indication, (with UE-A Avatar ID?))

7. Response with Avatar representation content

10. 200 OK (Audio, Video [, “Avatar” indication ])

11. 200 OK (Audio, Video , “Avatar” network rendering indication)

5. Request Avatar netw rendering (UE-A Avatar id, Session info incl. download info)

Originating Side

Terminating Side

IMS HSS

8. Confirm avatar network rendering

3. Avatar ID retrieval/validation req (UE A IMPU/IMPI ID, UE-A Avatar ID?)

4. Avatar ID retrieval/validation resp (incl. download info, default UE-A avatar id?)

Figure 6.X.2.1-1: Network centric IMS avatar call flow using SIP/SDP

The Avatar ID is transmitted as part of SIP/SDP negotiation.

NOTE 1: The following flow relates only to UE-A’s avatar. Whether UE-B also wants to use an avatar in the same session is not considered.

1.	UE A decides to request an avatar session with network-based avatar rendering, e.g., based on its internal capabilities, and can select an Avatar ID to be used for the session.

2.	UE A initiates an IMS Audio/Video session indicating in the SIP/SDP it would like to use an avatar, using network-based rendering. It can further provide to the IMS network the Avatar ID of the avatar to be used in the call. For example, Avatar ID can be e.g., sent within a m- or a- line in the SDP.

NOTE 2: How to indicate an “avatar” session and how to transfer avatar-id in the SIP/SDP is up to SA4 to define.

3.	Based on the UE-A associated IMPU/IMPI, the IMS AS requests the IMS HSS to validate whether the Avatar ID can be used by UE-A, or to retrieve the “default” avatar-id associated to the subscription or the current registration if no Avatar ID was provided by UE-A at step 2.

4.	The IMS HSS responds to the IMS AS. If the avatar ID usage for this session was validated, then the IMS HSS can indicate additional information such as a download link to retrieve avatar metadata (incl. avatar representation). If no Avatar ID was provided in step 3, the IMS HSS can respond with a “default” avatar-id associated to the subscription or the current registration, and related additional information to retrieve the avatar. If the usage of the avatar ID requested by UE-A was not validated, an error response is generated towards UE-A and the session is terminated.

5.	IMS AS requests MF to prepare for network-based rendering, including information about the session including the information to download Avatar metadata, e.g. Avatar representation, from DAC.

6.	MF requests Avatar metadata from DAC using the information provided by IMS AS (e.g. Avatar ID, download link).

7.	DAC provides Avatar metadata (including avatar representation) to MF.

8.	The MF confirms to IMS AS that it can perform network-based rendering of the avatar and provides the necessary information to anchor the media streams.

9.	The IMS AS forwards the INVITE to UE-B via the terminating IMS network including avatar session indication (and modified SDP anchored at MF). Optionally, the avatar-id of UE-A can be forwarded to UE-B as additional information. UE-B has the option to reject the avatar (but accept the regular A/V session) or terminate the session.

10.	UE B and terminating IMS network send 200 OK to IMS AS including avatar session indication acceptance.

11.	The IMS AS forwards the 200 OK to UE-A including avatar session indication (and modified SDP anchored at MF) to confirm that this avatar can be used.

12.	UE A sends regular audio/video stream to MF.

13.	The MF receives the audio/video stream from UE-A and renders the avatar (e.g. replaces the face/body from the incoming video media from UE-A with the selected Avatar).

NOTE 3: How MF performs the rendering of the avatar is up to SA4.

14.	The resulting stream is sent as regular video to UE-B.

15.	(Optional) The rendered Avatar video can be sent back to UE-A as feedback (same content as the one sent to UE-B in step 14), e.g., to display a thumbnail view of the Avatar to UE-A.

#### 6.X.2.2	Scenario 2: UE-A centric IMS avatar call flow using SIP/SDP

UE-A

P/I/S-CSCF/IMS-AGW

IMS HSS

IMS AS

DAC

Terminating IMS Core

UE-B

1. Decide to start avatar session with local rendering based on its status

2. INVITE (Audio, Video, Avatar local rendering indication + Avatar ID?)

10. Send Audio/Video media including avatar of UE-A over RTP

9. Local rendering of avatar id of UE-A

5. INVITE (Audio, Video, avatar indication, Avatar ID?)

6. 200 OK (Audio, Video , avatar indication)

7. 200 OK (Audio, Video, Avatar local rendering indication, avatar id, download info)

Originating Side

3. Avatar ID retrieval/validation req (UE A IMPU/IMPI ID, UE-A Avatar ID?)

4. Avatar ID retrieval/validation resp (incl. download info, default UE-A avatar id?)

8. UE-A downloads UE-A avatar from DAC, unless already locally available

UE-A

P/I/S-CSCF/IMS-AGW

IMS HSS

IMS AS

DAC

Terminating IMS Core

UE-B

1. Decide to start avatar session with local rendering based on its status

2. INVITE (Audio, Video, Avatar local rendering indication + Avatar ID?)

10. Send Audio/Video media including avatar of UE-A over RTP

9. Local rendering of avatar id of UE-A

5. INVITE (Audio, Video, avatar indication, Avatar ID?)

6. 200 OK (Audio, Video , avatar indication)

7. 200 OK (Audio, Video, Avatar local rendering indication, avatar id, download info)

Originating Side

3. Avatar ID retrieval/validation req (UE A IMPU/IMPI ID, UE-A Avatar ID?)

4. Avatar ID retrieval/validation resp (incl. download info, default UE-A avatar id?)

8. UE-A downloads UE-A avatar from DAC, unless already locally available

Figure 6.X.2.2-1: UE A centric IMS avatar call flow using SIP/SDP

The Avatar ID is exchanged with the IMS network for getting the avatar representation. The Avatar ID is negotiated within SIP/SDP.

1.	UE A decides to request an avatar session with local avatar rendering, e.g., based on its internal capabilities, and can select an Avatar ID to be used for the session.

2.-	UE A initiates an IMS Audio/Video session indicating in the SIP/SDP it would like to use avatar, using local rendering. It can further provide to the IMS network the Avatar ID of the avatar to be used in the call. For example, Avatar ID can be e.g., sent within a m- or a- line in the SDP.

NOTE 1: How to indicate an “avatar” session and how to transfer avatar-id in the SIP/SDP is up to SA4.

3.	Based on the UE-A associated IMPU/IMPI, the IMS AS requests the IMS HSS to validate whether the Avatar ID can be used by UE-A, or to retrieve the “default” avatar-id associated to the subscription or the current registration if no Avatar ID was provided by UE-A at step 2.

4.	The IMS HSS responds to the IMS AS. If the avatar ID usage for this session was validated, then the IMS HSS can indicate additional information such as a download link to retrieve avatar metadata (incl. avatar representation). If no Avatar ID was provided in step 3, the IMS HSS can respond with a “default” avatar-id associated to the subscription or the current registration, and related additional information to retrieve the avatar. If the usage of the avatar ID requested by UE-A was not validated, an error response is generated towards UE-A and the session is terminated.

5.	The IMS AS forwards the INVITE to UE-B via the terminating IMS network including avatar session indication. Optionally, the avatar-id can be forwarded to UE-B as additional information. UE-B has the option to reject the avatar (but accept the regular A/V session) or terminate the session.

6.	UE B and terminating IMS network send 200 OK to IMS AS including avatar session indication acceptance.

7.	The IMS AS forwards the 200 OK to UE-A including avatar session indication to confirm that this Avatar can be used. IMS AS further provides information to download Avatar metadata, e.g. Avatar representation, from DAC.

8.	UE A downloads Avatar metadata (incl. avatar representation) from DAC using the information provided by IMS AS, unless locally available.

NOTE 2:	Security implications about how UE A requests and receives the Avatar representation from DAC is defined by SA3.

9.	UE A starts avatar media rendering process (e.g. replacing the face from the incoming media from the camera).

10.	UE A sends rendered media to UE B, e.g. as regular A/V media or as avatar media depending on SDP negotiation.

#### 6.X.2.3	Scenario 3: UE-B centric IMS avatar call flow using SIP/SDP

UE-A

P/I/S-CSCF/IMS-AGW

IMS HSS

IMS AS

DAC

Terminating IMS Core

UE-B

1. Decide to start avatar session with remote rendering based on its status

2. INVITE (Audio, Avatar media + Avatar ID?)

9. Send Audio + Avatar media of UE-A over RTP

10. Local rendering of avatar id of UE-A

5. INVITE (Audio, avatar media, Avatar ID, download info)

7. 200 OK (Audio, avatar media, avatar ID)

8. 200 OK (Audio, avatar media, avatar ID)

Originating Side

6. UE-B downloads UE-A avatar from DAC, unless already locally available

3. Avatar ID retrieval/validation req (UE A IMPU/IMPI ID, UE-A Avatar ID?)

4. Avatar ID retrieval/validation resp (incl. download info, default UE-A avatar id?)

UE-A

P/I/S-CSCF/IMS-AGW

IMS HSS

IMS AS

DAC

Terminating IMS Core

UE-B

1. Decide to start avatar session with remote rendering based on its status

2. INVITE (Audio, Avatar media + Avatar ID?)

9. Send Audio + Avatar media of UE-A over RTP

10. Local rendering of avatar id of UE-A

5. INVITE (Audio, avatar media, Avatar ID, download info)

7. 200 OK (Audio, avatar media, avatar ID)

8. 200 OK (Audio, avatar media, avatar ID)

Originating Side

6. UE-B downloads UE-A avatar from DAC, unless already locally available

3. Avatar ID retrieval/validation req (UE A IMPU/IMPI ID, UE-A Avatar ID?)

4. Avatar ID retrieval/validation resp (incl. download info, default UE-A avatar id?)

Figure 6.X.2.3-1: UE B centric IMS avatar call flow using SIP/SDP

The Avatar ID is exchanged with UE-B for getting the avatar representation. The Avatar ID is negotiated within SIP/SDP.

1.	UE A decides to request an avatar session with remote UE based avatar rendering, e.g., based on its internal capabilities, and can select an Avatar ID to be used for the session.

2.-	UE A initiates an IMS session indicating in the SIP/SDP it would like to use avatar, using remote UE rendering. For example, the SDP indicates audio and avatar media (instead of video media) as m-line. It can further provide to the IMS network the Avatar ID of the avatar to be used in the call. For example, Avatar ID can be e.g., sent within a m- or a- line in the SDP.

Editor’s Note: How to indicate an “avatar” session and how to transfer avatar-id in the SIP/SDP is up to SA4.

3.	Based on the UE-A associated IMPU/IMPI, the IMS AS requests the IMS HSS to validate whether the Avatar ID can be used by UE-A, or to retrieve the “default” avatar-id associated to the subscription or the current registration if no Avatar ID was provided by UE-A at step 2.

4.	The IMS HSS responds to the IMS AS. If the avatar ID usage for this session was validated, then the IMS HSS can indicate additional information such as a download link to retrieve avatar metadata (incl. avatar representation). If no Avatar ID was provided in step 3, the IMS HSS can respond with a “default” avatar-id associated to the subscription or the current registration, and related additional information to retrieve the avatar. If the usage of the avatar ID requested by UE-A was not validated, an error response is generated towards UE-A and the session is terminated.

5.	The IMS AS forwards the INVITE to UE-B via the terminating IMS network including avatar session indication and the avatar-id to be rendered. IMS AS further provides information to download Avatar metadata, e.g. Avatar representation, from DAC. UE-B has the option to reject the avatar media (i.e. only establish an audio call) or terminate the session.

6.	UE B downloads Avatar metadata (incl. avatar representation) from DAC using the information provided by IMS AS, unless locally available.

NOTE:	Security implications about how UE B requests and receives the Avatar representation from DAC is defined by SA3.

7.	UE B and terminating IMS network send 200 OK to IMS AS including avatar session indication acceptance.

8.	The IMS AS forwards the 200 OK to UE-A including avatar session indication to confirm that this Avatar can be used, and that avatar media can be received by UE B.

9.	UE A sends audio and avatar media to UE B.

10.	UE B starts the avatar rendering process (e.g. animating the avatar representation from UE A with the information provided in the incoming avatar media from UE A).

### 6.X.3	Impacts on existing nodes and functionality

Impacted nodes and functionality are listed as follows:

-	UE: Support indicating avatar-id when establishing IMS sessions. Can support local avatar download from DAC, and rendering. Can support avatar media to allow remote rendering. Can support receiving feedback media including avatar from MF in case of network-based rendering.

-	IMS AS: Support avatar indications in SIP/SDP. Interacts with HSS to validate the avatar-id for the session and retrieve the information to enable avatar representation download from DAC. Insert avatar id and avatar download information (e.g. URL) provided by HSS into SIP/SDP. Interacts with MF to request network-based avatar rendering.

-	MF: Support network-based avatar rendering. Receives request from IMS AS to download avatar representation and confirm rendering capability. Performs avatar rendering based on incoming video media (e.g. replacing face) prior to sending video media to terminating UE. Sends feedback media including avatar to originating UE.

-	HSS: Support associating avatar-id(s) with IMPU/IMPI, including “default” avatar-id. Validates requests from IMS AS to use avatar-id with an IMPI/IMPU and provides information (e.g. download link) to enable avatar representation download from DAC.

-	DAC: Provides avatar metadata upon request from MF or UE.

# * * * * End of changes * * * *