3GPP TSG-WG SA2 Meeting #162	S2-2404822

# Changsha, China, April 15- April 19, 2024

Source: 	ZTE

Title:	KI#1, solution update for solution#34

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract: Propose solution update for solution#34.

1. Discussion

This proposal proposes solution update for solution#34.

2. Proposal

It is proposed to capture the following changes in TR 23.700-29.

FIRST CHANGE

6.34	Solution #34: Enhancement on AMF discovery for Regenerative-based satellite

6.34.1	Description

This is a solution addressing Key Issue#1.

For LEO/MEO deployment, the gNB deployed on a satellite moves around the earth, it is not preferable to locally configure the IP addresses of all AMFs on the earth that can be connected by the satellite gNB. This solution provides mechanism for Satellite/gNB to dynamically discover the AMFs that is allowed to be connected by the Satellite/gNB.

In this solution, an AMF agent is deployed by the satellite operator and closed to the NTN-GW. The AMF agent supports N2 interface towards Satellite/gNB via the NTN-GW. The AMF Agent provides address information of all AMFs within its service area that can be connected by the Satellite/gNB, so the Satellite/gNB can establish direct SCTP connection and NGAP connection towards these AMFs. The AMF Agent also provides the supported TAI list that can be broadcast by the Satellite/gNB.

The Satellite/gNB is configured with the address of AMF Agent based on the ephemeris information of the Satellite/gNB, and this information may be updated by the satellite operator.

The Satellite/gNB may be shared by multiple PLMNs. The AMF Agent may determine which PLMNs are allowed to share the Satellite/gNB in this service area, and provide address information of the AMFs from different PLMNs to the Satellite/gNB.

6.34.2	Procedures

The following procedures shows how to enhance the AMF discovery Regenerative-based satellite.

Figure 6.34.2.1-1: AMF selection enhancement for Regenerative-based satellite

0.	The Satellite/gNB is configured with the address information of AMF Agent based on the ephemeris information of the Satellite/gNB.

1.	When the Satellite/gNB moves into the service area of the AMF Agent, it initiates SCTP connection establishment towards the AMF Agent via an NTN-GW.

2.	After the SCTP connection is established, the Satellite/gNB initiates NG Setup procedure by sending NG Setup Request message to the AMF Agent. The message includes relevant configuration information e.g. Global RAN Node ID, list of supported TA(s), list of supported PLMN(s), supported S-NSSAI(s) per TA and so on

3.	The AMF Agent may use the Global RAN Node ID to identify the Satellite/gNB and determine the AMFs within the AMF Agent's service area that are allowed to be connected by the Satellite/gNB. The AMF Agent returns NG Setup Response message towards the Satellite/gNB, including new AMF information. The new AMF information may include at least the SCTP address information of the AMF. The AMF Agent may also provide the supported TAI list that can be broadcast by the Satellite/gNB in the NG Setup Response message.

4-9.	Based on local configuration, the Satellite/gNB starts to establish SCTP connection and NGAP connection towards the AMFs using the AMF information as received in the step 3.

6.34.3	Impacts on services, entities and interfaces

NGAP interface:

-	The NG Setup response message is enhanced to include the address information of all AMFs within AMF agent's service area that is allowed to be connected by the Satellite/gNB

-	The NG Setup response message is enhanced to include the supported TAI list that can be broadcast by the Satellite/gNB.

AMF Agent:

-	AMF with enhancement on the NGAP interface as described in clause 6.34.2.

Satellite/gNB:

-	gNB with enhancement on the NGAP interface as described in clause 6.34.2.

# * * * * End of changes * * * *