SA WG2 Meeting #162	S2-2404944

April 15 – April 19, 2024, Changsha, China

Source:	InterDigital Inc.

Title:	KI#1, New Solution, Handling of Direct AIML Positioning

Document for:	Approval

Agenda Item: 	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This paper proposes a new solution for Key Issue #1.

# 1	Discussion

.

# 2	Proposal

It is proposed to adopt the proposed new solution in TR 23.700-84.

* * * * Start of Changes * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues and Use Cases

|  | Key Issues |  |  |  | Use cases (optional) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 5 | 6 |
| #X | X |  |  |  |  |  |  |  |  |  |

* * * * Next Change (All new text) * * * *

## 6.X	Solution #X: Handling of Direct AIML Positioning

### 6.X.1	Description

When the 5GC network receives a location service request, the network needs to determine whether traditional positioning method or AIML-based positioning method should apply. This may be based on a few considerations, such as the required positioning performance, availability of AIML models, etc. If it is determined to proceed with AIML-based positioning, the 5GC network needs to further select a positioning strategy for the current location service request. This includes:

- determining whether a traditional positioning method or Direct AIML-based positioning method is used.

-	the entity that provides the trained AIML models.

-	the entity that performs the model inference for the positioning results or enhancing the positioning measurements.

-	the AIML models that can be used considering the applicable area/scenario, performance requirements, etc.

-	The data input that are required by the selected models and how to collect the data input.

In this solution, it is proposed that a Positioning Strategy Execution Function (PSEF) be introduced to handle the AIML-based positioning selection and execution. The PSEF may be co-located with a proper NF, e.g., AMF. The PSEF possesses (by local configuration or NRF query) the information of the NF capabilities of AIML-based positioning support, e.g., the LMFs that support AIML-based positioning and the Model IDs that they support. The PSEF also possesses the information of available models for AIML-based positioning, including their applicable area/scenario, estimated performance, required dataset, etc. It is assumed the trained AIML models are stored in ADRF, and the information can be retrieved using NWDAF/MTLF services.

When a location service request for a target UE is received, the AMF/PSEF performs the following actions:

-	The AMF/PSEF determines whether the AIML-based positioning applies, or the traditional method is used instead.

-	If AIML-based positioning is used, the AMF/PSEF determines what AIML models are applicable and which entity (e.g. LMF) performs the model inference based on NF capabilities to support AIML-based positioning.

-	the AMF/PSEF selects the model inference NF (e.g. LMF) and informs it that AIML-based positioning is used, and the source of AIML models (e.g. NWDAF/MTLF address)

### 6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

Figure 6.X.2-1: AIML-based positioning in MT-LR procedure

- An external LCS client sends an LCS request to the GMLC for a target UE location.

- The GMLC obtains the serving AMF address and invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF to request the current location of the UE.

- The AMF/PSEF determines that an AIML-based positioning procedure may apply to this request based on a few factors. For example, whether there are necessary NFs (NWDAF, LMF, etc.) available supporting AIML-based positioning, there are AIML models available and suitable for the location service request, etc. The AMF/PSEF also selects the suitable AIML models and locate the NWDAF/MTLF that host the models.

- The AMF selects the LMF that supports AIML-based positioning and the required models, e.g., through NRF (not shown in figure 6.X.2-1), based on the NF capability, e.g., whether AIML-based positioning is supported.

- The AMF invokes the Nlmf_Location_DetermineLocation service operation towards the selected LMF to request the current location of the UE. In the request, the AMF may indicate that AIML-based positioning is expected and include the NWDAF/MTLF address that may provide required AIML models, and model IDs to be retrieved.

- The LMF sends a Nnwdaf_MLModelInfo_Request to the received NWDAF address to retrieve the models.

- The LMF stores the models in its local repository.

- The LMF collects positioning measurements required as the AIML model input, by interacting with the UE and the RAN node.

- The LMF performs the model inference to obtain AIML-based positioning result.

- The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF to return the current location of the UE.

- The AMF returns the Namf_Location_ProvidePositioningInfo Response towards the GMLC/LRF to return the current location of the UE.

- GMLC respond to the LCS client service request.

6.X.3	Impacts on services, entities and interfaces

AMF/PSEF:

- Support determination on traditional positioning or AI-based positioning

- Support selection of NFs involved in AI-based positioning for inference, model acquisition, input collection, etc.

LMF:

- Support model inference for AI-based positioning

- Support model acquisition from NWDAF/MTLF

- Support measurement input collection for AI-based positioning

* * * * End of Changes * * * *