3GPP TSG-WG SA2 Meeting #162	S2-2405158

Changsha, China, April 15 – 19, 2024

Source: 	China Mobile

Title: 	New solution for KI#2: Usage information exposure by UPF

Document for: 	Approval

Agenda Item: 	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract of the contribution: New solution for KI#2.

1 Discussion

It is needed for industry to aware of traffic usage information of employees in real-time. They require for traffic usage monitoring, e.g. accessing specific application/service/DNN/network slice during specific period, especially for industry network.

2 Proposal

It is proposed to add a new solution of KI#2 to TR 23.700-63 FS_UPEAS_Ph2.

FIRST CHANGE “all the text is new”

6.X	Solution #X: Usage information exposure by UPF

6.X.1	Key Issue mapping

This solution is for KI#2.

6.X.2	Description

AF may need to know the traffic usage information of specific users in the local Network Slice or local NPN in real-time. The main intention is that the AF or the Vertical Campus owner can have a clear idea of how much traffic the local network has consumed within a short time. It is proposed to expose this information through API interface to alleviate the complex in AF side(Since currently, some verticals they do not have the tool to monitor the Network Slice and NPN on-going traffic information).So in this solution, it is proposed to let UPF can directly expose the traffic usage information for specific UE  when the UE accesses to DNN/Network Slice/NPN during specific period.

6.X.3	Procedures

UPF(s)

AF

NEFF

NRF

1.Nnef_UsageReport_Request

2.Nnrf_NFDiscovery_Request

3.Nnrf_NFDiscovery_Request response

4.Nupf_EventExposure_Subscribe

5. Usage Report Generation

7.Nnef_UsageReporting_Response

6.Nupf_EventExposure_Notify

Figure 6.X.3-1: Usage report exposure to AF by UPF

- The AF invokes Nnef_UsageReport_Request service operation with UE IP address, DNN, S-NSSAI,Traffic Description, Usage aggregation period, reporting frequency, and the indication for direct information exposure..

- NEF authrizes the AF request, and NEF invokes the Nnrf_NFDiscovery service operation to obtain the address of the UPF.

- NRF response with the UPF address to NEF..

4. NEF invokes the Nupf_EventExposure Subscribe service operation with Usage monitoring assisted information including UE IP address, Traffic Description for the target traffic (e.g. Application Id), Usage aggregation period, reporting frequency.

5-7. UPF starts the Usage monitoring based on the assistance information and report the Usage information to NEF/AF.

6.x.4	Impacts on services, entities and interfaces

UPF:

-	Support NEF as a service consumer for direct subscription of UPF Event Exposure service.

AF:

-	Supports to subscribe Nnef_UsageReport service operation.

NEF:

-	Support to invoke Nnrf_NFDiscovery service operation with UE IP address to discover the serving UPF(s) for the requested UPF event.

-	Supports to directly subscribe UPF Event Exposure service for usage report with UE IP address and Traffic Description.

End of CHANGE