3GPP TSG-WG SA2 Meeting #162	S2-2405308

Changsha, China, April 15th – 19th, 2024	(revision of S2-2404081)

Source:	Huawei, HiSilicon, CATT

Title:	KI#2, Sol #9: Update to remove ENs

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3/ Rel-19

Abstract: It is proposed to update Solution#9 to remove ENs.

1. Discussion

1.1	Overview

The following sections resolve Editor’s notes in solution#9.

1.2    Solution to editor’s notes

Editor’s Note: Whether End to end delay requirement is sent within EDI or Application Function influence on traffic routing related information as described in clause 5.6.7 of TS 23.501 is FFS.

Observation#1: The newly introduced parameter End-to-end delay requirement is to indicate the value of delay between UE and EAS for specific application and is used for EAS selection. Based on current specs, the AF influence on traffic routing procedure is normally used for including the requirement or indication for EAS selection (e.g. User Plane Latency Requirement, EAS Correlation indication). So the end to end delay requirement is more suitable to be included in the Application Function influence related information.

Proposal#1: It’s proposed to remove this Editor’s Note.

Editor’s Note: The details of the conversion is FFS.

Observation#2: If the AF provides EAS load information with the format of the EAS processing delay directly, there is no need to perform conversion. Please see details in Observation #3.

Proposal#2: It’s proposed to remove this Editor’s Note and add more description after step 4 in 6.x.3.2.

Editor’s Note: Whether the processing delay can be provided by the AF or not is FFS.

Observation#3: Actually, the application service provider may decide the specific format of the provided EAS load information. For example, the application service provider can decide whether to provide percentage of the load or EAS processing delay based on configuration. The EAS itself supports the ability to monitor the percentage of EAS load in real time, and application server can simply send this information to 5GC via AF. However, for EAS processing delay provisioning, the delay statics function needs to be added to the EAS during application running, which may introduce additional system overhead for the EAS.

Proposal#3: It’s proposed to remove this Editor’s Note and update the solution that the application server is able to decide the specific format of the provided EAS load information (e.g., percentage of the load or EAS processing delay).

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-49.

# * * * * First change * * * *

6.9	Solution #9: local UPF and EAS (re)selection jointly considering N6 delay and EAS load

6.9.1	Description

Per existing design of TS 23.548[5], the (re)selection of the EAS and local PSA by 5GC is based on the UE location and EAS Deployment Information, which only considers the topology information of UE, UPF and EAS. Nevertheless, dynamic information especially the N6 delay between the local PSA and EAS, and EAS load, which contributes to the End to end delay (i.e. between the UE and EAS), are also important factors that should be taken into account.

Some consideration that EAS load has been considered when the DNS server returns the DNS response back to the client. However, these parameters are considered separately. It is better to have one NF jointly consider these parameters and also get the real time measurement.

Comparing to the solution discussed for KI#1, this solution focus on how the N6 delay and EAS load is used for L-PSA/EAS (re)selection.

6.9.3	Procedures

6.9.3.1	Enhancement on EAS Deployment Information Management

The EAS deployment information provisioned from AF to 5GC as defined in clause 6.2.3.4 of TS 23.548 [5] is used for local UPF and EAS (re)selection. It is proposed to enhance this information as following:

Table 6.9.3.1-1 Additional information of EAS Deployment Information

| Parameters | Description |
| --- | --- |
| … | …. |
| Delay-sensitive indication | the application demands end to end delay requirement [optional]. |
| AF/NEF address information | Target Address for event subscription [optional]. (see NOTE 5). |
| …. NOTE 4: It includes End to end delay, i.e. the sum of N6 delay between local PSA and EAS, delay between UE and local PSA and processing delay for a specific application. NOTE 5: If the AF is trusted, the AF provides AF address information during the EDI provision process. Otherwise, NEF provides the NEF address information to replace the address information provided by the AF. |  |

6.9.3.2	EAS/L-PSA discovery procedure with EASDF

Normally the EAS load can be converted to the processing delay (e.g. https://ieeexplore.ieee.org/abstract/document/9253665). Hence when the EAS reports how much percentage of load EAS reaches, based on the local configuration, the percentage of load can be converted to the how much processing delay the EAS needs.

The procedure of EAS/L-PSA discovery procedure with EASDF is defined as in Figure 6.9.3.2-1. The AF requests End-to-end delay requirement of the application for EAS discovery as described in step 1-5 of figure 4.3.6.2-1 in TS 23.502 [3] before the following procedure.

UE

SMF

ULCL/BP

Local PSA

PSA

NEF

AF

EASDF

1. step 1-step 12 in Figure 6.2.3.2.2-1 of TS 23.548

DNS Server

2. DNS response

3. Neasdf_DNSContext_Notify Request/Response

4a. Nnef_EventExposure_Subscribe

4b. Naf_EventExposure_Subscribe

4c. Naf_EventExposure_Subscribe

4d. Naf_EventExposure_Notify

4f. Naf_EventExposure Notify

4e. Nnef_EventExposure_Notify

5. SMF performs EAS load-Processing delay conversion

6. SMF informs local PSAs to measure the real-time N6 delay

Immediate Notification

7. SMF determines suitable EAS and L-PSA based on the end-to end delay requirement or/and N6 delay requirement

8. Perform step 16-step 19 in Figure 6.2.3.2.2-1 of TS 23.548

Figure 6.9.3.2-1: EAS discovery procedure with EASDF

- Same as step 1 – step 12 of clause 6.2.3.2.2 in TS 23.548[5].

- same as step 13 as described in clause 6.2.3.2.2 of TS 23.548[5].

- same as step 14/15 as described in clause 6.2.3.2.2 of TS 23.548[5].

NOTE 1: The SMF can be changed to other NF per the discussion of the KI#1.4a/4b. If the matched DNS response message corresponding to the delay sensitive application, i.e. which has provisioned the Delay-sensitive indication as part of EDI, SMF subscribe to the EAS load information (e.g., percentage of the load or EAS processing delay) from AF by sending the received EAS IP (s)/FQDN(s), subscribed event to AF through NEF. The event subscription also includes the Immediate reporting flag and reporting threshold, e.g. load level or specific delay value.4c. SMF subscribe to the EAS load information (e.g., percentage of the load or EAS processing delay) from AF by sending the received EAS IP (s)/FQDN(s), subscribed event directly. The event subscription also includes the Immediate reporting flag and reporting threshold, e.g. load level or specific delay value.

4d/4e. AF notifies SMF of EAS load information (e.g., percentage of the load or EAS processing delay) corresponding to the received EAS IP (s)/FQDN(s) through NEF immediately. The EAS load is reported as percentage of EAS Instance load.

4f. AF notifies SMF of EAS load information (e.g., percentage of the load or EAS processing delay) corresponding to the received EAS IP (s)/FQDN(s) immediately. The EAS load is reported as the percentage of EAS Instance load.

NOTE 2:  The application service provider may decide the specific format of the provided EAS load information. For example, the application service provider can decide whether to provide percentage of the load or EAS processing delay based on configuration. The EAS itself supports the ability to monitor the percentage of EAS load in real time, and application server can simply send this information to 5GC via AF. However, for EAS processing delay provisioning, the delay statics function needs to be added to the EAS during application running, which may introduce additional system overhead for the EAS.

OTE 3: The AF can also send a notification when a condition is met, e.g., the load level of the EAS (e.g., percentage of the load or EAS processing delay) meets the reporting threshold. The details are described in clause 6.9.3.3. It is assumed the AF is willing to provide EAS load information (e.g., percentage of the load or EAS processing delay to use the optimization from 5GC.

- SMF performs EAS load-Processing delay conversion, i.e. convert the percentage of EAS load to the processing delay.

NOTE 4: The relationship between percentage of EAS load and processing delay can be pre-configured on SMF. Step 5 can be skipped if the AF provides EAS load information with the format of the EAS processing delay directly.

6. 	SMF informs candidate local PSA(s) to measure the real-time N6 delay between the L-PSA and the reported EAS IP(s). In details, SMF determine the candidate Local PSA(s) base on the reported EAS IP(s) and Local PSA information. Then, the candidate local PSA(s) obtain the N6 delay by using certain Layer 3 measuring mechanisms respectively, e.g. via ICMP as defined in RFC 792[7].

NOTE 4: Step 6 can be executed in parallel with step 4 or 5.

7. SMF determines suitable EAS and L-PSA based on the end-to end delay requirement.

NOTE 5: The delay between UE and PSA UPF is controlled by the PDB in the 5QI. Thus only N6 delay and processing delay are measured for EAS and PSA selection.

8. Same as step 16 – step 19 as described in clause 6.2.3.2.2 of TS 23.548[5].

For EAS/L-PSA discovery procedure with local DNS server mechanism, the L-PSA is inserted either per DNS query or per UE mobility before DNS query. For the L-PSA inserted per DNS query, same procedure (i.e. step 4 -7) as described above can be executed.

6.9.3.3	EAS/L-PSA rediscovery at Edge Relocation

It is assumed that the SMF, which re-discovers the EAS/L-PSA, can be other NF per the discussion of KI#1.

7. SMF reselects a L-PSA or triggered EAS discovery as the end-to end delay requirement are not satisfied

Case 2：

5.SMF subscribes N6 delay

Local PSA 2

PSA 2

Case 1：

2a. Naf_EventExposure_Notify

2c. Naf_EventExposure_Notify

2b. Nnef_EventExposure Notify

UE

SMF

ULCL/BP

Local PSA1

PSA 1

PSA

NEF

EASDF

6. SMF informs candidate local PSA(s) to measure the real-time N6 delay

4. SMF triggered EAS discovery as the end-to end delay requirements are not satisfied

1. Perform EAS/L-PSA discovery

AF

3. SMF perform EAS load-Processing delay conversion

Notification when the condition is met

Case 3：8.AF trigger EAS rediscovery as EAS load balance or maintenance, etc.

9. Step 3b-11b in Figure 4.3.3.2-1 of TS23.502

Figure 6.9.3.3-1: EAS/L-PSA rediscovery procedure at Edge relocation

This procedure is used to trigger the EAS (or L-PSA) rediscovery procedure when a new connection between EAS and L-PSA need to be established. This rediscovery can be triggered in three cases:

- Case 1: SMF triggered EAS rediscovery based on reported EAS load (e.g., percentage of the load or EAS processing delay) as the end-to end delay requirement is not satisfied

- Case 2: SMF triggered L-PSA or EAS rediscovery based on measured N6 delay as end-to end delay requirement is not satisfied.

- Case 3: AF triggered EAS rediscovery as EAS load balance or maintenance, etc.

1. Perform EAS and L-PSA discovery as described in 6.9.3.2.

Case 1: EAS load trigger EAS rediscovery2. AF notifies SMF of EAS load information (e.g., percentage of the load or EAS processing delay) corresponding to the EAS IP (s) through NEF or directly when the event condition is met, e.g. the EAS load (e.g., percentage of the load or EAS processing delay) exceed the load threshold set by SMF. The SMF sets load threshold based on the end to end delay requirement in the EDI.

3. The same as step 5 of clause 6.9.3.2.

NOTE 1: The step 3 can be skipped if the AF provides EAS load information with the format of the EAS processing delay directly.

4. SMF triggers EAS discovery as the end-to end delay requirement are not satisfied.

Case 2: N6 delay trigger L-PSA or EAS rediscovery

5. The SMF subscribes the N6 delay measurement event to L-PSA. This event subscription can be per event threshold or periodically. The UPF report the measured real-time N6 delay to the SMF by using certain Layer 3 measuring mechanisms respectively, e.g. via ICMP as defined in RFC 792[7]. The SMF judges that the N6 delay exceed the related threshold per end-to-end delay requirement provisioned in EDI.

6. 	If there are candidate L-PSA(s), e.g. different L-PSA to the same DNAI, the SMF informs the candidate L-PSA(s) to measure the N6 delay to the indicated EAS instance.

7. 	If the calculated end-to end delay of the candidate L-PSA(s) can fulfil the end-to end delay requirement, the SMF triggers the L-PSA reselection and following steps are skipped, i.e. step 8-9 are skipped. Otherwise SMF trigger EAS rediscovery as the end-to end delay requirement are not satisfied.

Case 3: AF trigger EAS rediscovery

8. AF trigger EAS rediscovery as EAS load balance or maintenance, etc.

9. Same as step 2 of clause 6.2.3.3.3 in TS 23.548[5].

6.9.3	Impacts on services, entities and interfaces

AF/NEF:

-  Provide enhanced EDI information as described above

-	Provide the EAS load information (e.g., percentage of the load or EAS processing delay) to SMF per event subscription

-  Provide AF influence related information as described above.

SMF:

-	Get enhanced EDI information and EAS load information (e.g., percentage of the load or EAS processing delay) from AF

-	Perform EAS load - processing delay conversion

-	Iinstructs UPF to measure the real-time N6 delay

-	Determine suitable EAS and L-PSA based on the above information