/SA WG2 Meeting #162	S2-2405540

# April 15 – April 19, 2024, Changsha, China	(revision of S2-2404245)

Source: 	vivo, CMCC, CATT, Huawei, Samsung, China Unicom, KDDI

Title: 	Interim evaluation and conclusion for KI3

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution tries to provide the Interim evaluation and conclusion for KI3.

- Introduction

This paper is trying to evaluate the solutions captured for KI#3(5GS enhancements for network energy saving and efficiency), provide the conclusions for the KI#3 based on the captured solutions.

2	Proposal

It is proposed to capture the following solution in TR 23.700-66.

Start of Changes

8.X	Conclusion for Key Issue #3: 5GS enhancements for network energy saving and efficiency

Following interim conclusions for KI#3 are proposed for normative work:

Editor's note:	The conclusions need to be further revised.

- For Enhancements on NF (re-)selection based on energy related information :

-	NF profile is extended (e.g., include the new energy related information or reuse existing NF profile parameters) to allow an operator to influence NF discovery and selection based on their energy strategy.

Editor's note: It’s FFS if the information in NF profile is abstract and static information or more dynamic energy related information.

-  NF (re-)selection is enhanced to consider the energy related information from the NF profiles.

- Enhancements on existing operations and procedures for energy saving and energy efficiency:

-	Access management, session management, UP path adjustment may be enhanced taking the energy related information into account.

-  For Network slice which is subject to energy control, e.g.,

determination of accept/reject of requested S-NSSAIs or PDU session, and modification/release of PDU session may consider energy related information.

- Maximum number of UEs and maximum number of PDU sessions for a network slice maybe adjusted by considering energy related information when it is subject to NSAC

Editor's note:	Whether all the NSAC architecture options are applicable to this feature is FFS.

- AMF may retrieve energy related subscription information (e.g., the Green Subscription Indication (GSI)) from the UDM during registration and provide it to RAN.

NOTE:	The energy related decision will also consider operator’s policy.

Editor’s note: Whether the enhancement on existing operations and procedures can assist saving energy and energy efficiency is FFS.

- For NWDAF-Based Energy Analytics for network energy saving and efficiency:

Editor’s note: Whether the NWDAF can assist saving energy and energy efficiency (e.g. considering the EC of generating analytics) is FFS.

Editor's note: Energy related information required and obtained to support 5GS enhancement for energy saving and energy efficient is dependent on KI#1 conclusions.

End of Changes