SA WG2 Meeting #S2-162	S2-2405517

15 - 19 April, 2024, Changsha, China	(revision of S2-2404376/S2-2403829)

Source:	ZTE

Title:	KI#1, new solution on reusing the existing mechanism for UE configuration and idle mobility

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract of the contribution: This paper proposes a new solution to Key issue #1, to resolve the CSG-CAG interworking.

1. Proposal

It is proposed to accept the change to 23.700-45.

>>>> Start of Change <<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[x]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

>>>> Next Change <<<<

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> |
| #1 |  |  |
| #2 |  |  |
| #X | x | x |

>>>> Next Change <<<<

## 6.X	Solution #X: Reusing the existing mechanism for UE configuration and idle mobility

### 6.X.1	Description

In the Architectural Assumptions, the CAG concept is used for the 5G femto as following:

-	It is assumed that the existing CAG concept defined for PNI-NPN is re-used for Femto access control without impacts to PNI-NPN.

In this solution, to access the 5G Femto cell, the UE may be pre-configured or (re)configured with the following CAG information:

-	an Allowed CAG list i.e. a list of CAG Identifiers the UE is allowed to access; and

The HPLMN may configure or re-configure a UE with the above CAG information using the UE Configuration Update procedure for access and mobility management related parameters described in clause 4.2.4.2 of TS 23.502 [3].

The above CAG information is provided by the HPLMN on a per PLMN basis.

The UE may be locally pre-configured with the allowed 4G CSG list or following the CSG handling defined in the TS 23.401 [x].

Editor's note:	If the UE is not configured with the allowed 4G CSG list, how it works is FFS.

During the idle mobility from 4G to 5G femto cell:

-	The UE uses the Allowed CAG list to perform the network and cell selection as described in clause 5.30.3.4 of TS 23.501 [2].

During the idle mobility from 5G to 4G CSG cell:

-	The UE perform the network and cell selection as described in TS 23.401 [x].

### 6.X.2	Procedures

There is no new procedure defined

### 6.X.3	Impacts on services, entities and interfaces

There is no change to the existing mechanism on UE configuration, provisioning and idle mobility.

>>>> End of Change <<<<