3GPP TSG SA WG2 Meeting #164	S2-2407458

# Maastricht, Netherlands, August 19 – August 23, 2024                  (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1401 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3: DC interworking with MTSI UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on KI#3 conclusion in TR 23.700-77, DC interworking with MTSI UE needs to be specified in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Three types of DC interworking with MTSI UE are described. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DC interworking with MTSI UE cannot be supported completely. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text is new)* * * *

AC.X	DC interworking with MTSI UE

AC.X.1	General

For supporting data channel interworking with MTSI UE, there are three cases of IMS DC session as below:

- When the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE, the IMS DC session is set up between the originating DCMTSI UE and the originating DCSF/MF and the data channel media is transcoded to MMTEL media at the MF, and the MMTEL media is transferred to the terminating MTSI UE.

- When the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE, the P2A IMS DC session is set up between the originating DCMTSI UE, the originating DCSF/MF, and DCAS. When the DC AS notices that the DC interworking with terminating MTSI UE, the DC AS may take action to provide DC application specific information to the terminating MTSI UE.

- When the originating MTSI UE initiates an IMS session to the terminating DCMTSI UE, the IMS AS may send event notification to the DCAS that the IMS session that requires DC interworking is established. The event may be the non-subscriber specific event, such as a call is requested for a service number which requires DC interworking. When the DC AS notices that the DC interworking with originating MTSI UE, the DC AS may take action to provide DC application specific information to the originating MTSI UE.

# * * * * End of changes * * * *