3GPP TSG SA WG2 Meeting #164	S2-2407478

# Maastricht, Netherlands, August 19 – August 23, 2024                  (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4828 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPSI handling at the UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on LS reply from CT3 (S2-2407444/C3-243450), the misalignment between list of PSIs provisioned at the UE and those sent by the PCF can be avoided if the PCF first removes all the UE Policies in the UE as per the list of UPSI(s) stored in UDR/PCF and then provide to the UE all determined UE Policies. And a NOTE is added in TS 29.525. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new NOTE in clause 4.16.11.1 to describe the potential removal and then provisioning of UE Policies in case the PCF detects a possible misalignment between the UE Policies stored in UDR and the UE Policies stored in the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential misalignment cannot be clarified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * *First change * * * *

4.16.11.1	General

The UE Policy Association Establishment procedure, which may be performed for a UE registered in the same AMF or different AMFs for 3GPP access and non-3GPP access, concerns the following scenarios:

1.	UE initial registration with the network.

2.	The AMF relocation with PCF change in handover procedure and registration procedure.

3.	UE registration with 5GS when the UE moves from EPS to 5GS and there is no existing UE Policy Association between AMF and PCF for this UE.

4.	A change of the value of the UE Policy Association indicator received from UDM from "disabled" to "enabled" while the UE is registered.

The H-PCF may interact with the CHF in HPLMN to make a decision about UE Policies based on spending limits.

Figure 4.16.11-1: UE Policy Association Establishment

This procedure concerns both roaming and non-roaming scenarios. The AMF stores the UE Policy Container in the UE Context when received from the UE.

In the non-roaming case the V-PCF is not involved and the role of the H-PCF is performed by the PCF. For the roaming scenarios, the V-PCF interacts with the AMF and the H-PCF interacts with the V-PCF:

1.	The AMF establishes UE Policy Association with the (V-)PCF considering the value of the UE Policy Association Indicator from UDM, the reception of the UE Policy Container from the UE and the AMF local configuration as defined in TS 23.503 [20].

NOTE 1:	In roaming scenario, the AMF local configuration can indicate whether UE Policy delivery is needed based on the roaming agreement with home PLMN of the UE as defined in TS 23.503 [20].

2.	The AMF sends a Npcf_UEPolicyControl Create Request with the following information: SUPI, may include Access Type and RAT, PEI, ULI, UE time zone, Serving Network (PLMN ID, or PLMN ID and NID, see clause 5.34 of TS 23.501 [2]), the Internal-Group-ID-list and UE Policy Container (the list of stored PSIs, operating system identifier, Indication of UE support for ANDSP, indication of UE capability of reporting URSP rule enforcement to network, indication of support of URSP delivery in EPS). In roaming scenario, based on operator policies, the AMF may provide to the V-PCF the PCF ID of the selected H-PCF. The V-PCF contacts the H-PCF. In roaming case, steps 3 and 4 are executed, otherwise step 5 follows.

If the AMF, based on configuration, is aware that the UE is accessing over a gNB using satellite backhaul, the AMF includes the Satellite backhaul category as described in clause 5.43 of TS 23.501 [2].

If during UE initial registration the AMF receives from the UE the indication of support of N3IWF selection based on the slices the UE wishes to use over untrusted non-3GPP access and/or of UE support for TNGF selection based on the slices the UE wishes to use over trusted non-3GPP access, the AMF shall indicate to the PCF that the UE supports N3IWF selection based on the slices the UE wishes to use over untrusted non-3GPP access and/or UE support for TNGF selection based on the slices the UE wishes to use over trusted non-3GPP access. The AMF may request to the PCF to receive a notification when the provisioning in the UE of ANDSP/WLANSP with slice information has finished.

3.	The V-PCF forwards the information received from AMF in step 2 to the H-PCF. When a UE Policy Container is received at initial registration, the H-PCF may store the PEI, the OSId, indication of UE capability of reporting URSP rule enforcement to network, indication of support of URSP delivery in EPS or the indication of UE support for ANDSP in the UDR using Nudr_DM_Create including DataSet "Policy Data" and Data Subset "UE context policy control data".

The H-PCF may invoke the Nudr_DM_Query service to the UDR. And the UDR may response with the requested policy control subscription data as in clause 6.2.1.3 of TS 23.503 [20] and/or application data as in clause 6.2.1.6 of TS 23.503 [20].

The V-PCF may retrieve the Application guidance on URSP Rule for inbound roamers of the PLMN of the SUPI, if not available, using Nudr_DM_Query or Nudr_DM_Subscribe including the Data Set "Application Data" and Data Subset "Service Specific Information" and DataKey set to "PLMN ID(s) of inbound roamers".

4.	The H-PCF sends a Npcf_UEPolicyControl Create Response to the V-PCF. The H-PCF may provide the Policy Control Request Trigger parameters in the Npcf_UEPolicyControl Create Response. Before sending the response, the H-PCF may determine that the decision about UE policy control depends on the status of the policy counters available at the CHF and if such reporting is not established for the subscriber, the H-PCF initiates an Initial Spending Limit Report Retrieval as defined in clause 4.16.8.2. If policy counter status reporting is already established for the subscriber and the H-PCF determines that the statuses of additional policy counters are required, the H-PCF initiates an Intermediate Spending Limit Report Retrieval as defined in clause 4.16.8.3.

NOTE 2:	The Nudr_DM_Query in step 3 may include the Spending Limit Information, i.e., the policy counters and their latest status. Thus the PCF can provide the UE policy to the AMF before contacting the CHF. The PCF may need to update the AMF depending on the statuses of the policy counters provided by the CHF.

NOTE 3:	Potential inconsistencies between the policy counter and its status in the UDR and in the CHF can happen given that the CHF may update the policy counter and its status at any time, as such it is recommended that the PCF contacts the CHF if the policy counters and its status stored in the UDR is used, to be able to receive updated information from the CHF.

NOTE X:	Potential misalignment between the UE Policies determined by the PCF and the UE Policies stored at the UE (e.g. in AMF relocation with PCF change scenario), and there is a possiblity that UE receives removal of PSIs that are not present at the UE. To avoid this, when the PCF, based on configuration and implementation specific means, detects this possibility, the PCF can avoid it if the PCF first removes all the UE Policies in the UE as per the list of PSI(s) stored in UDR/PCF and then provides to the UE all determined UE Policies.

The (H-)PCF in roaming and the PCF in non-roaming may register to the BSF as the PCF serving this UE, if not already registered at the AM Policy Association establishment. This is performed by using the Nbsf_Management_Register operation, providing as inputs the UE SUPI/GPSI and the PCF identity.

5.	The (V-) PCF sends a Npcf_UEPolicyControl Create Response to the AMF. The (V-)PCF relays the Policy Control Request Trigger parameters in the Npcf_UEPolicyControl Create Response.

The (V-)PCF also subscribes to notification of N1 message delivery of policy information to the UE using Namf_Communication_N1N2MessageSubscribe service which is not shown in this figure.

6.	The (H-)PCF gets policy subscription related information and the latest list of PSIs from the UDR using Nudr_DM_Query service operation (SUPI, Policy Data, UE context policy control data, Policy Set Entry) if either or both are not available and makes a policy decision. The (H-)PCF may get the PEI, the OSId, indication of UE capability of reporting URSP rule enforcement to network or the indication of UE support for ANDSP in the UDR using Nudr_DM_Query including DataSet "Policy Data" and Data Subset "UE context policy control data" if the AMF relocates and the PCF changes. In the roaming scenario, the H-PCF may provide the indication of UE support for ANDSP to the V-PCF, if the indication was not present in the Npcf_UEPolicyControl Create request from V-PCF and the H-PCF gets this information from the H-UDR. The (H-)PCF may get the 5G VN group data and 5G VN group membership for each Internal-Group-ID received from the AMF using Nudr_DM_Query (Internal-Group-Id, Subscription Data, 5G VN Group Configuration). The (H-)PCF may store the 5G VN group data and 5G VN group membership for later use for other SUPIs that belong to the same Internal-Group-ID. The (H-)PCF may request notifications from the UDR on changes in the subscription information by invoking Nudr_DM_Subscribe (Policy Data, SUPI, DNN, S-NSSAI, Notification Target Address (+ Notification Correlation Id), Event Reporting Information (continuous reporting), UE context policy control data) service. The (H-)PCF may request notifications from the UDR on changes in the 5G VN group data or 5G VN group membership associated to each of the Internal-Group-Id provided to the PCF by invoking Nudr_DM_Subscribe (Subscription Data, 5G VN Group Configuration, Internal Group ID, Notification Target Address (+ Notification Correlation Id), Event Reporting Information (continuous reporting)) service. The (H-)PCF creates the UE policy container including UE policy information as defined in clause 6.6 of TS 23.503 [20] and in the case of roaming H-PCF provides the UE policy container in the Npcf_UEPolicyControl_UpdateNotify Request. In the non-roaming case, the PCF may subscribe to Analytics from NWDAF as defined in clause 6.1.1.3 of TS 23.503 [20].

7.	The V-PCF sends a response to H-PCF using Npcf_UEPolicyControl_UpdateNotify Response.

NOTE 2:	Step 6 (and step 7) can be omitted. Then the (H-)PCF creates the UE policy container including UE polices in step 2 (in the case of non-roaming) or step 3 (in the case of roaming). This means that the potential interactions with UDR as in step 6 will have to be executed in step 2 (non-roaming) or step 3 (roaming).

8.	The (V-)PCF triggers UE Configuration Update Procedure in clause 4.2.4.3 to sends the UE policy container including UE policy information to the UE. The (V-)PCF checks the size limit as described in clause 6.1.2.2.2 of TS 23.503 [20].

9.	If the V-PCF received notification of the reception of the UE Policy container then the V-PCF forwards the notification response of the UE to the H-PCF using Npcf_UEPolicyControl_Update Request.

If the V-PCF is notified by the V-UDR about the Service Specific Information applicable to inbound roamers from the HPLMN of the UE as specified in clause 4.15.6.10, the V-PCF provides the Service Parameters to the H-PCF.

10.	The H-PCF sends a response to the V-PCF. If the V-PCF received a UE Policy Container step 8 will follow.

# * * * * End of changes * * * *