3GPP TSG-SA WG2#163	S2-2407485

Maastricht, Netherlands, 19 – 23 August, 2024	(was S2-2406279)

Source:	Qualcomm Incorporated

Title:	KI 1.2: Conclusion

Document for:	Discussion/Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution proposes some principles for the conclusion of KI#1.2.

# 1.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-54:

# >>>>BEGINNING OF CHANGES<<<<

## 8.1.	Conclusions for DualSteer

### 8.1.xx	KI#1.2 Registration and mobility management for DualSteer

To support DualSteer the following principles are concluded for the normative work:

-	Two separate registration procedures, one for each SUPI of the DualSteer device, are performed. The association of these two SUPIs is provided by the UDM to the H-PCF.

-	The DualSteer Device includes two PEIs, each one associated to a USIM.

-	No changes on the VPLMN (i.e., AMF, V-PCF) will be standardized.

-	The DualSteer device initiates a registration procedure using either SUPI as identifier and the associated PEI.

-   	The UDM ensures that the subscription information provided to the AMF is consistent with the subscription information provided later when the DualSteer Device registers with the other SUPI.

-	The UE Policy association establishment/modification may trigger the PCF to initiate UE Configuration Update procedure for transparent UE Policy delivery (see TS 23.502 [4] clause 4.2.4.3).

-	The registration procedure is performed as per current specifications. If the DualSteer Device receives UE policies for this SUPI, it provides them to the DualSteer Control Function.

-	For the registration of the other SUPI:

-	If DualSteer is enabled and the initial SUPI registration was successful, the DualSteer Control Function may activate the other SUPI of the DualSteer Device.

-	If the other SUPI is activated, the DualSteer Device initiates the registration procedure towards a second 3GPP access network using the mentioned SUPI and the associated PEI.

-	During the registrations of the two SUPIs, extended URSP rules for DualSteer are provided to the UEs associated with the two SUPIs (see clause 8.1.xz for conclusions on policy enhancements for DualSteer).

# >>>>END OF CHANGES<<<<