3GPP TSG-WG2 Meeting #164	S2-2407488

19th – 23rd August 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0532 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 - LMF enhancements for UE positioning using a ML model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR proposes to specify the conclusions in the TR 23.700-84 for KI#1 for the option when LMF performs both training of the ML Model and inference, as such principle #2 and #5 are the main ones listed below for reference, although other principles are also applicable to this option. Principle #2: LMF is enhanced to perform model training for AI/ML based Positioning, the trigger for data collection and for model training in LMF is up to implementation. Principle #5: Data used for model training, inference and model performance monitoring for AI/ML based positioning will be decided by RAN WGs, and SA WG2 will align with RAN WGs. The related procedures for data collection will be coordinated with RAN WGs in the normative phase. NOTE 3: The user's authorization/consent of collecting UE related training data is needed as specified in TS 23.288 [5] and TS 23.273 [7]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | LMF functional description is extended to indicate that UE positioning may be based on ML model. LMF retrieves user consent for data collection for the purpose of model training from UDM. The LMF collects channel measurements from the UE using a N1 container that is sent transparently via AMF. The N1 container transfer a LCS Periodic-Triggered Invoke Request/Response to request channel measurements and location estimates. The procedure for data collection is applicable for training an ML Model in LMF, however any differences with the procedure for data collection when the training of the ML Model is not in LMF should be minimized. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusion in the TR 23.700-84 are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 4.2b, 4.2.1, 4.3.5, 4.4.10, 4.4.x (new), 5.x(new), 6.x.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[5]	3GPP TS 43.059: "Functional Stage 2 description of Location Services in GERAN".

[6]	Void.

[7]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[8]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[9]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[10]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[12]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[13]	OMA MLP TS: "Mobile Location Protocol", [http://www.openmobilealliance.org].

[14]	Void.

[15]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[16]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[17]	3GPP TS 25.305: "Stage 2 functional specification of User Equipment (UE) positioning in UTRAN".

[18]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[19]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[20]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[21]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[22]	Void.

[23]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[24]	3GPP TS 23.222: "Common Application Programming Interface (API) framework for 3GPP northbound APIs".

[25]	RFC 2396: "Uniform Resource Identifiers".

[26]	RFC 3261: "SIP: Session Initiation Protocol".

[27]	3GPP TS 23.228: "IP multimedia subsystem (IMS)".

[28]	3GPP TS 23.003: "Numbering, addressing and identification".

[29]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[30]	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

[31]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[32]	Void.

[33]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[34]	Void.

[35]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[36]	3GPP TS 24.571: "5G System (5GS); Control plane Location Services (LCS) procedures; Stage 3".

[37]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[38]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[39]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[40]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[41]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[42]	3GPP TS 23.632: "User data interworking, coexistence and migration; Stage 2".

[43]	3GPP TS 29.563: "Home Subscriber Server (HSS) services for interworking with Unified Data Management (UDM); Stage 3".

[44]	3GPP TS 33.536: "Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

[45]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[46]	3GPP TS 33.533: "Security aspects of ranging based services and sidelink positioning".

[47]	3GPP TS 38.355: "Sidelink Positioning Protocol (SLPP)".

[48]	3GPP TS 24.572: " User Plane Location Services (LCS) Protocols And Procedures; Stage 3".

[49]	OMA-AD-SUPL-V2_0: "Secure User Plane Location Architecture Approved Version 2.0".

[50]	3GPP TS 33.501: " Security architecture and procedures for 5G system".

[xx]	3GPP TS 38.305: " NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

********** Next Change **********

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

APN	Access Point Name

APN-NI	APN Network Identifier

EDT	Early Data Transmission

E-SMLC	Evolved Serving Mobile Location Centre

GMLC	Gateway Mobile Location Centre

HGMLC	Home GMLC

LCS	LoCation Services

LDR	Location Deferred Request

LIR	Location Immediate Request

LMF	Location Management Function

LOS	Line-Of-Sight

LPI	LCS Privacy Indicator

LPHAP	Low Power and High Accuracy Positioning

LRF	Location Retrieval Function

ML	Machine Learning

MBSR	Mobile Base Station Relay

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

NLOS	Non-Line-Of-Sight

NI-LR	Network Induced Location Request

PMD	Pseudonym mediation device functionality

POI	Privacy Override Indicator

PRU	Positioning Reference Unit

SDT	Small Data Transmission

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TWAP	Trusted WLAN Access Point

VGMLC	Visited GMLC

SL-MO-LR	Sidelink Mobile Originating Location Request

SL-MT-LR	Sidelink Mobile Terminating Location Request

SUPL	Secure User Plane Location

********** Next Change **********

## 4.2b	Positioning methods

The LCS feature utilises one or more positioning methods in order to determine the location of user equipment (UE). The positioning methods may use a trained ML Model available at the LMF.

Determining the position of a UE involves two main steps:

-	Radio signal measurements or non-RAT measurements; and

-	Position estimate computation based on the measurements.

The positioning methods for 3GPP access are described in clause 5.2.

The positioning methods for non-3GPP access are described in clause 5.3.1.

********** Next Change **********

### 4.2.1	Non-roaming reference architecture

Figure 4.2.1-1 shows an architectural reference model for 5GS LCS for a non-roaming UE and PRU in reference point representation.

Figure 4.2.1-1: Non-roaming reference architecture for Location Services in reference point representation

NOTE 1:	(R)AN represents NG-RAN, trusted non-3GPP access or untrusted non-3GPP access.

NOTE 2:	Reference point interface related to charging functionality is not shown in this specification.

NOTE 3:	PRU refers to a Positioning Reference Unit realized by a UE as defined in clause 5.4.5 of TS 38.305 [9].

Figure 4.2.1-2 shows an architectural reference model for 5GS LCS for a non-roaming UE and PRU in SBI representation.

Figure 4.2.1-2: Non-roaming reference architecture for Location Services in SBI representation

********** Next Change **********

### 4.3.5	UE

A target UE may support positioning according to four different modes:

-	UE assisted mode (the UE obtains location measurements and sends the measurements to another entity (e.g. an LMF) to compute a location);

-	UE based mode (the UE obtains location measurements and computes a location estimate making use of assistance data provided by serving PLMN);

-	standalone mode (the UE obtains location measurements and computes a location estimate without making use of assistance data provided by serving PLMN);

-	network based mode (a serving PLMN obtains location measurements of signals transmitted by a target UE and computes a location estimate).

NOTE:	The transmission of UE signals for network based mode may or may not be transparent to the UE.

Positioning procedures used by a UE for NG-RAN access are described in TS 38.305 [9].A limited set of UE positioning capabilities and UE user plane positioning capabilities can be transferred to the 5GCN in the 5GMM capability during registration of the UE as described in TS 24.501 [11]. Some of these positioning capabilities may be transferred subsequently to an LMF as described in TS 29.572 [12]. UE positioning capabilities may also be transferred directly to a location server (e.g. LMF).

Additional functions which may be supported by a UE to support location services include the following.

-	Support location requests received from a network for 5GC-MT-LR, 5GC-NI-LR or a deferred 5GC-MT-LR for periodic or triggered location.

-	Support location requests to a network for a 5GC-MO-LR.

-	Support privacy notification and verification for a 5GC-MT-LR or deferred 5GC-MT-LR for periodic or triggered location.

-	Send updated privacy requirements to a serving AMF (for transfer to a UDR via UDM).

-	Support periodic or triggered location reporting to an LMF.

-	Support change of a serving LMF for periodic or triggered location reporting.

-	Support cancelation of periodic or triggered location reporting.

-	Support multiple simultaneous location sessions.

-	Support the reception of unciphered and/or ciphered assistance data broadcast by NG-RAN.

-	Support the reception of ciphering keys for the assistance data from the AMF.

-	Support handling of 5GC-MT-LR, 5GC-NI-LR, 5GC-MO-LR and deferred 5GC-MT-LR for periodic or triggered location over a user plane connection between UE and LMF.

-	Support reporting of location events for a periodic or triggered 5GC-MT-LR over a user plane connection to an LCS Client or AF with periodic cumulative event reports being sent over control plane to the LMF, H-GMLC and LCS Client or AF.

-	Support channel measurements requests received from the network and report channel measurements to the network. The reports may be provided periodically.

********** Next Change **********

### 4.3.8	Location Management Function, LMF

The LMF manages the overall co-ordination and scheduling of resources required for the location of a UE that is registered with or accessing 5GCN. It also calculates or verifies a final location and any velocity estimate and may estimate the achieved accuracy. The LMF receives location requests for a target UE from the serving AMF using the Nlmf interface. The LMF interacts with the UE in order to exchange location information applicable to UE assisted and UE based position methods and interacts with the NG-RAN, N3IWF or TNAN in order to obtain location information.

The LMF shall determine the result of the positioning in geographical co-ordinates as defined in TS 23.032 [8] and/or in local co-ordinates as defined in TS 23.032 [8]. If requested and if available, the positioning result may also include the velocity of the UE. The coordinate type(s) is determined by LMF when receiving a location request, based on LCS Client type and supported GAD shapes. If the location request indicates regulatory LCS Client type the LMF shall determine a geographical location and optionally a location in local coordinates. For location request indicates a value added LCS Client type, the LMF may determine the UE location in local coordinates or geographical co-ordinates or both. If the supported GAD shapes is not received or Local Co-ordinates is not included in the supported GAD shapes, the LMF shall determine a geographical location.

NOTE 1:	Some RAT independent position methods (e.g. GNSS based position methods) can only determine a UE location in geographical co-ordinates. In such a case, the LMF may translate a UE location in geographical co-ordinates into a location in local co-ordinates when an origin for the local co-ordinates has known global coordinates. When an origin for the local co-ordinates does not have known global coordinates, position methods that can only determine a UE location in geographical co-ordinates cannot be used to determine a UE location in local co-ordinates.

Additional functions which may be performed by an LMF to support location services include the following.

-	Support a request for a single location received from a serving AMF for a target UE.

-	Support a request for periodic or triggered location received from a serving AMF for a target UE.

-	Determine type and number of position methods and procedures based on UE and PLMN capabilities, QoS, UE connectivity state per access type, LCS Client type, co-ordinate type and optionally service type and indication of requiring reliable UE location information.

-	Report UE location estimates directly to a GMLC for periodic or triggered location of a target UE.

-	Support cancelation of periodic or triggered location for a target UE.

-	Support the provision of broadcast assistance data to UEs via NG-RAN in ciphered or unciphered form and forward any ciphering keys to subscribed UEs via the AMF.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	Support of receiving stored UE Positioning Capability from AMF and support of providing updated UE Positioning Capability to AMF.

-	Map the UE location to a geographical area where the PLMN is or is not allowed to operate based on the request from AMF.

-	Support determination of a UE location at a scheduled location time.

-	Support determination of indoor or outdoor for a location estimate.

-	Determine whether to use user plane or control plane for positioning.

-	Determine whether to use an ML Model for UE positioning. If yes, trigger data collection for training of the ML Model and for monitoring of the performance of the ML Model. The LMF retrieves user consent for data collection for the purpose of ML model training from UDM as defined in TS 23.502 [19].

NOTE x:	The trigger for data collection for training and monitoring the performance of the ML Model is upto implementation.

Editor´s Note: Whether LMF can also retrieve a trained model from OAM or not is FFS and needs to coordinate with SA5.

-	Support handling of 5GC-MT-LR, 5GC-NI-LR, 5GC-MO-LR and deferred 5GC-MT-LR for periodic or triggered location over a user plane connection between UE and LMF over TLS.

NOTE 3:	How the LMF uses the received UE user plane positioning capability for SUPL [49] is left to the implementation.

-	Support collection of GNSS assistance data from AFs.

-	Support service level PRU Association, PRU Association update or PRU Disassociation.

-	LMF supports verification of a PRU initiated Association or Disassociation by checking whether there is an PRU verified indication from AMF.

-	LMF stores the received PRU information contained in service level PRU Association message and removes the PRU information after PRU Disassociation.

-	LMF keeps PRU information for PRUs which are in OFF state.

-	LMF may indicate support of PRU function to NRF via NF profile and may further send the PRU indication via NF profile update if PRU is stationary PRU.

-	LMF may request a PRU to associate to a new LMF by returning a Routing ID of the new LMF.

PRU ON/OFF states indicate temporarily availability of the PRU functionality of a UE at the serving LMF (e.g. PRU OFF due to other high priority tasks/energy saving at the UE, or the UE temporarily loses network coverage).

-	Support selection of a PRU based on stored PRU information if the LMF needs to obtain the location measurements from the PRU to assist positioning of a target UE.

-	Support to obtain PRU location measurements as described in clause 5.4.5 of TS 38.305 [9] by triggering the procedure in clause 6.11.

-	Support to obtain PRU location measurements from other PRU serving LMF(s).

-	As a serving LMF of target UE(s), support discovery and selection of other PRU serving LMF(s) by querying the NRF and support to request PRU location measurements from the selected LMF(s).

-	As a serving LMF of PRU(s), support to provide PRU location measurements to other LMF(s) after receiving a request from other LMF(s).

-	Support to determine UE location by considering obtained PRU location measurements.

NOTE 4:	Country, area within a country, or an international area can be supported as different types of geographical area.

-	Support a request for user plane reporting from a UE to an LCS Client or AF for a periodic or triggered 5GC-MT-LR. Subsequently, support the transfer of cumulative event reports from the target UE via control plane back to the H-GMLC and LCS Client or AF. Also support any request for assistance data received in a cumulative event report.

-	Determine UE location for a UE connecting to a MBSR based on location and velocity of the MBSR and the timing of the location estimations for the target UE and MBSR.

-	For a regulatory location service, support reporting of multiple INTERMEDIATE location estimates to GMLC.

********** Next Change **********

### 4.4.10	NL1 Reference Point

The NL1 reference point supports location requests for a target UE sent from a serving AMF for the target UE to an LMF. Location requests are supported for immediate location and for deferred location for periodic or triggered location events.

The NL1 reference point also supports the transfer from an LMF to an AMF of ciphering keys and associated data that enable deciphering by suitably subscribed UEs of ciphered broadcast assistance data.

The NL1 reference point also supports requests for transparent delivery of both the request for channel measurements from the LMF to the UE via AMF, and the provided channel measurements from the UE to the LMF.

Messages for the NLI reference point are defined in in TS 29.518 [16] and TS 29.572 [12].

********** Next Change **********

### 4.4.x	NL11 Reference Point

The NL11 reference point is used to retrieve user consent for data collection for training purposes from UDM using Nudm_SDM service defined in TS 23. 502 [19]. Depending on local policy or regulations, to protect the privacy of user data, the data collection for a user may be subject to user consent.

.

********** Next Change **********

## 5.x	Support for UE positioning based on a ML Model at the LMF

The LMF may perform UE positioning based on an ML Model that may be trained and monitored at the LMF. The ML Model may be part of the LMF logic. The input data for training ML model is generated at the UE (PRU or non-PRU UE) or at the gNB, in both cases, these are channel measurements, quality indicator and time stamp of the channel measurements that are provided to LMF. In order to monitor the performance of the ML Model, the LMF may obtain the ground truth label and related positioning measurements generated at gNB or the UE.

The procedure for data collection for ML model training triggered by LMF is defined in clauses 6.x.1.

The procedure for performance monitoring for ML model at LMF is based on the LMF obtaining ground truth data using a different positioning method such as PRU as defined in clause 6.17 or network assisted GNSS as defined in 4.3.2 of TS 38.305 [xx].

********** Next Change **********

##### 6.X.1	Data collection at LMF to train the AI/ML model to perform positioning based on UE measurements

The LMF needs to obtain channel measurements and may also request the UE to provide the UE location. The ML Model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Figure 6.x.1-1: Data collection by LMF to train the AI/ML model using UE measurements.

- The LMF starts data collection for the purpose to train a ML Model for UE Positioning. The LMF subacribes to AMF to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = ”any UE”, Event ID = “UEs in/out area of interest”.

- The AMF send Namf_EventExposure_Subscriber_Response (list of SUPIs in the area of interest”).

- The LMF checks the user consent for model training using Nudm_SDM_Get (SUPI, User consent (purpose is set to ML model training)), if user consent is granted, the LMF subscribes to UDM to change on user consent using Nudm_SDM_Subscribe (SUPI, User consent (purpose is set to ML model training) and the procedure continues in step  for this SUPI, otherwise the SUPI in the area of interest is skipped no data is collected for this SUPI.  The LMF also checks that the UE supports the capability to report channel measurements.

For each SUPI that provided consent to data collection and for a UE that can report channel measurements for on a model training steps 4 to 9 are performed:

- The LMF requests to transfer a LCS supplementary service to provide channel measurements to the UE. The LMF sends the LCS Periodic-Triggered Invoke Request to provide channel measurements encapsulated in a N1 container. The LMF contacts AMF using Namf_Communication_N1N2MessageTransfer (SUPI,  N1 container (LCS Periodic-Triggered Invoke Request (routing indetifier, required channel measurements, required location measurements). . If the UE is not currently reachable, the AMF waits for the UE to become reachable.	Once the UE is reachable, if the UE is then in CM IDLE state, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.3 of TS 23.502 [19] to establish a signalling connection with the UE. The AMF forwards the N1 container to the UE that includes the LCS Periodic-Triggered Invoke Request. The AMF includes an routing identifier in the NAS transport message containing an LCS Correlation identifier - e.g. according to clause 6.11.1.

- The UE returns a supplementary services acknowledgment to the LMF, which is transferred via the serving AMF using the routing identifier and delivered to the LMF using an Namf_Communication_N1MessageNotify service operation.

- The UE collects the channel measurements and the UE location estimates that were requested in step 4.

- The UE reports to the LMF the positioning measurements as indicated in the frequency. The performs a UE triggered service request as defined in clause 4.2.3.2 of TS 23.502 [19] if in CM-IDLE state in order to establish a signalling connection with the AMF. The UE sends a supplementary services event report message to the LMF which is transferred via the serving AMF and is delivered to the LMF using an Namf_Communication_N1MessageNotify service operation, The event report includes any channel measurements, location estimate and the timestamp of the channel measurements and location estimate if available. The UE also includes the routing identifier received in step 6  in the NAS Transport message used to transfer the event report from the UE to the AMF. The AMF then forwards the event report to either the serving LMF based on the routing identifier indicates a particular LMF. The LMF trains the AI/ML model using the data collected from the UE.

- If a location estimate is needed for event reporting, the LMF may perform one or more of the positioning procedures described in clauses 6.11.1, 6.11.2, 6.11.3 and 6.11.4 and as described for step 8 in clause 6.1.1 and step 12 in clause 6.1.2. The LMF then determines the UE location using the location measurements and/or location estimate(s) obtained at this step and/or received at step 25. The LMF may also determine the timestamp of the location estimate.

- The UE continues to monitor for further periodic or trigger events as in step 6 and then step 7 to step 9 are performed each time a trigger event is detected.

At the time the LMF decides that channel measurements for a UE are not needed, e.g. the user consent for data collection for model training is revoked, the LMF sends a LCS Periodic-Triggered Invoke Request to cancel the reporting.

********** End of changes **********