3GPP TSG-SA WG2 Meeting #164	S2-2407490

# August 19th – 23rd 2024, Maastricht, Netherlands

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1095 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4 - Updates in input/output for signalling storm analytics report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Conclusions in TR 23.700-84 has been agreed and this paper only proposes data to include in input and output for the new AnalyticsID as well as an example of enforcement to mitigate/prevent signalling storm. The changes are aligned with the correct functionality and responsibility of data sources, i.e., NF related data shall be collected from OAM, while UE related data can be collected from NF(s) in the core network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes are proposed in the tables for input and output data and the table for example actions. Comparing with the conclusions in the TR, only the following changes are included, as the more updates in the tables are FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsistencies with the type of the data that shall be collected from different data sources. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change (all text is new) * * *

## 6.X	Signalling Storm

### 6.X.1	General

### 6.X.2	Input data

Table 6.X.2-1: Input data from 5GC related to Control Plane Signalling Abnormality

| Information (NOTE 1) | Source | Description |
| --- | --- | --- |
| NFs ID | Service Producer and Consumer entities | The NF instance ID of the service producer and consumer entities. |
| Service Operation information | Service Producer and Consumer entities | The information of the reference point and corresponding service operation. |
| > Service Operation Request Counters | Service Producer entity / OAM (NOTE 2) | The counters related to the requests received by the service producer entity |
| >> Number of requests |  | The number of requests received in a time window. |
| >> Rate of requests |  | The rate of the service requests. |
| >> Unhandled requests |  | The number of requests received but not responded. |
| >> Timing of request |  | Max/min/variance of inter-request periods. |
| >> Rejected requests |  | The number of rejected requests. |
| >> Redundant requests |  | The number of redundant requests. |
| > Service Operation Response Counters | Service Consumer entity / OAM (NOTE 2) | The counters related to the requests sent by the service consumer entity. |
| >> Number of requests |  | The number of requests sent in a time window. |
| >> Rate of requests |  | The rate of the service requests. |
| >> Unhandled requests |  | The number of requests sent but not replied. |
| >> Timing of request |  | Max/min/variance of inter-request periods. |
| >> Rejected requests |  | The number of timeout requests. |
| >> Redundant requests |  | The number of redundant requests. |
| NF Context data | Service Producer and Consumer entities | The information about the current UE context, or network context and NF’s load. |
| > Number of UEs | NF | Total number of UEs that is served by the NF. |
| > Number of network context | NF | Total number of network context (e.g., interface context) that is maintained by the NF. |
| > NF load | NRF | The load of specific NF instance(s) in their NF profile. |
| > NF status & resource usage | OAM | The usage of assigned virtual resources currently in use for specific NF instance(s) (mean usage of virtual CPU, memory, disk) as defined in clause 5.7 of TS 28.552 [8]. |
| NOTE 1: All input data are optional. NOTE 2: The already supported counters/KPIs by OAM are collected from OAM; other remaining counters/KPIs are collected from the corresponding NFs. |  |  |

Editor´s Note: Whether Table 6.X.2-1 needs further updates is FFS.

### 6.X.3	Output Analytics

Table 6.X.3-1: Signalling Storm Statistics

| Information | Description |
| --- | --- |
| Source NF(s) ID | The NF ID of the source network entities. |
| Destination NF ID | The NF ID of the destination network entity. |
| Reference Point & Service Operation(s) | The information of the reference point and service operation(s) impacted. |
| Exceptions ID (NOTE 1) | The IDs of detected abnormality: - Too many requests - Too many parallel requests (without response) - Too less responses - Too late responses - Too inconsistent responses time - Too many requests reject - Too many requests timeout - Unexpected messages pattern |
| NOTE 1: The process of detecting abnormality and identifying the exception ID is left to the NWDAF’s implementation. |  |

Editor´s Note: Whether Table 6.X.3-1 needs further updates is FFS.

Table 6.X.3-2: Signalling Storm Predictions

| Information | Description |
| --- | --- |
| Source SUPI(s)/NF(s) ID | The NF ID(s) or SUPI(s) of the source network entities or UE(s). |
| Destination NF ID | The NF ID of the destination network entity. |
| Exceptions ID (NOTE 1) | The IDs of detected abnormality: - Too many requests - Too many parallel requests (without response) - Too less responses - Too late responses - Too inconsistent responses time - Too many requests reject - Too many requests timeout - Unexpected messages pattern |
| Confidence | Confidence of the prediction. |
| NOTE 1: The process of detecting abnormality and identifying the exception ID is left to the NWDAF’s implementation. |  |

Editor´s Note: Whether Table 6.X.3-2 needs further updates is FFS.

NFs and/or OAM as service consumers of the signalling storm analytics report shall have freedom to decide what action to take to mitigate or prevent signalling storm based on the internal logic. Table 6.X.3-3 lists examples of enforcement that can be done by NF service consumers to prevent signalling storm.

Table 6.X.3-3: Example mechanisms to mitigate and prevent the signalling storm

| Abnormality ID | Example Actions of NFs |
| --- | --- |
| Massive UE access | AMF sets MM NAS related timer (e.g. back-off, T3512) with suggested time range for a selected set of UEs. |
| Massive UE access | SMF sets SM NAS related timer (e.g. back-off) with suggested time range for a selected set of Sessions. |
| Massive UE access | AMF/SMF sets suggested N1/N2 interface related ingress/egress threshold, or AMF triggers RAN to initiate overload control for a selected set of UEs in specific slice or priority as defined in clause 8.7.7 of TS 38.413 [8] to start overload control. |
| Massive UE access | PCF may update PCC/URSP rules for a selected set of UEs to e.g., create PDU sessions, etc., to mitigate/prevent signalling storm in the network. |
| NF abnormal signalling | NRF configures the local policy to prevent the source NF/SCP with abnormal signalling from being discovered or discovering others. |
| NF abnormal signalling | Source NF/SCP configures to (re)select other NFs instead of NF with abnormal signalling and may unsubscribes the NF with abnormal signalling. |
| NF abnormal signalling | Source NF/SCP configures to deprioritize the NFs/Services with abnormal signalling from being selected. |
| NF abnormal signalling | Source NF triggers UE Reregistration and/or Session Reestablishments to avoid NF with abnormal signalling. |

* * * End of Change * * *