SA WG2 Meeting#164	S2-2407504

19 – 23 August 2024, Maastricht, Netherlands

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1308 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the handling of the UE and the AM Policy association |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_MINPA |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Additional clarifications are added that the operator sets the value of the AM or UE Policy Association Indication in UDM. The provisoning of AM Policies by the AF is aleady described in clause 6.1.2.6. and in TS 23.502 clause 4.15.6.9, no additional references are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additional clarifications are added that the operator sets the value of the AM or UE Policy Association Indication in UDM. Removal of incorrect text that refers to NEF authorization for provisoning of AM Policies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect description of the AM and UE Policy Control Associations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.1.2, 6.1.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First Change *****

##### 6.1.2.1.2	Handling of the AM Policy Association

In the non-roaming scenario, the establishment of the AM Policy Association is determined as follows. First, the AMF checks the AM Policy Association indicator received from UDM and establishes the AM Policy Association if its value is "enabled". If its value is "disabled" or no AM Policy Association indicator is received from the UDM then the AMF determines whether to establish the AM Policy Association or not according to local configuration.

In the roaming scenario, only the AMF local configuration is considered for the determination of the establishment of the AM Policy Association.

NOTE 1:	In roaming, the AMF does not consider the value of the AM Policy Association indicator, if received.

The AM Policy Association indicator for a SUPI is part of Access and Mobility Subscription data in UDM as described in clause 5.2.3.3.1 of TS 23.502 [3] and can be set by the operator to "enabled" or "disabled".

NOTE 2:	The operator can set the AM Policy Association indicator to "enabled" when it is required to provide AM Policies to the AMF or to "disabled" when there is no AM Policies to be provided to the AMF.

NOTE 3:	A possible trigger to set the AM Policy Association indicator to "enabled" is that either there are "AM influence information" for e.g. a SUPI in UDR or AM Policies are available for e.g. a SUPI provisioned in the PCF. The "AM influence information" is not provided to the AMF when the AM Policy Association Indicator is set to "disabled".

***** Next change *****

##### 6.1.2.2.5	Handling of the UE Policy Association

In the non-roaming scenario, the establishment of the UE Policy Association is determined as follows. First, the AMF checks the UE Policy Association indicator received from UDM and establishes the UE Policy Association if its value is "enabled". If its value is "disabled" or no UE Policy Association indicator is received from the UDM then the AMF establishes the UE Policy Association when the UE Policy Container is received from the UE. If no UE Policy Association indicator is received from the UDM and no UE Policy Container is received from the UE, then the AMF determines whether to establish the UE Policy Association or not as indicated by the local configuration.

In the roaming scenario, the AMF local configuration is considered first for the determination of the establishment of the UE Policy Association then the remaining parameters described above are applied.

NOTE 1:	It is assumed that the AMF local configuration is based on agreement between operators about whether UE policies can be delivered in the roaming scenario.

The UE Policy Association indicator for a SUPI is part of Access and Mobility Subscription data in UDM as described in clause 5.2.3.3.1 of TS 23.502 [3] and can be set by the operator to "enabled" or "disabled".

NOTE 2:	The operator can set the UE Policy Association indicator to "enabled" when it is required to provision UE Policies to the UE or to "disabled" when the UE is no longer subscribed to any service that requires sending UE policies to the UE.

NOTE 3:	A possible trigger to set the UE Policy Association indicator to "enabled" is that either there are service specific parameters for e.g. a SUPI in UDR or UE Policies are available for e.g. a SUPI provisioned in the PCF. Before setting the indicator to "disabled", the operator needs to ensure that there are no UE Policies stored for any of these services in the UE, by revoking the authorization for the service specific information, for Service Type "AF guidance for URSP", for all of the AFs with active authorizations, as defined in clause 4.15.6.7a of TS 23.502 [3] and by triggering the PCF to remove the UE Policies in the UE if any or both.

***** End of Changes *****