3GPP TSG-SA WG2#164	S2-2407509

Maastricht, Netherlands, 19 – 23 August, 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	KI 2.2: Update of evaluation

Document for:	Discussion/Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution:

# 1.	Discussion

In the last SA2#164 meeting the discussion on Key Issue 2.2 lead to some interim conclusions stating that there is no consensus to proceed with related normative work in Release 19. At the same time, some evaluation of the solutions for that key issue was captured in clause 7.2.2. The authors of this paper believe that the evaluation text contains some inconsistencies:

-	The evaluation in clause 7.2.2 is incomplete. For example, no security analysis was captured for Solutions 2.12 and 2.13. For these solutions (as well as for other solutions that are not relying on IPSec encryption to protect the confidentiality of the UP data exchanged between UE and UPF) the problem is that by relying purely on MPQUIC security the inner IP header which includes the IP addresses of UE and UPF is exposed in the clear over the UE-N3IWF communication path. This exposes not only the ingress point of the UPF, but potentially also the internal topology of the UP part of the 5GC. In addition, this enables also an attacker to potentially track a UE. This issue was not captured in the evaluation text agreed.

-	It includes opinions of the authors of the original paper proposing the evaluation (see S2-2406674), which are not agreed by other companies.

# 2.	Proposal

It is proposed to include the following changes in TS 23.700-54:

# >>>>BEGINNING OF CHANGES<<<<

### 7.2.2	Evaluation for Key Issue #2.2: Simplified ATSSS architecture over non-3GPP access

The following solutions have been proposed for simplifying the overall architecture over non-3GPP access:

Table 7.2.2-1

| Solution | Brief summary of the solution | Evaluation |
| --- | --- | --- |
| #2.2 | The UPF has a Public IP address, UE establishes an IPSec with the ePDG collocated with UPF and perform authentication via AAA server. UE establishes the non-3GPP access leg with PSA UPF over the IPSec tunnel. | This solution eliminates both N1 and N2 signaling over non-3GPP access. The solution uses the ePDG architecture and it co-locates the ePDG with the PSA UPF. Pros: 1. It eliminates the encryption over non-3GPP access by using no encryption for IPSec tunnelling. Drawbacks: 1. Limited deployment options assuming colocation of ePDG with UPF. 2. Exposes UPF through the use of a Public IP address. 3. No N1 interface over non-3GPP access. 4. No clean CP/UP split, i.e. UPF is concerned with CP for Non-3GPP access 5. If the idea is to use IKE between UE and ePDG for UE authentication and authorization, then why not use existing ePDG solution. Colocation of ePDG with UPF can always be an implementation or deployment choice and does not need to be specified. 6. This solution requires operators to support EPS nodes, which is a burden for 5G SA only operators. |
| #2.6 | UPF supports IPSec functionality. The UPF has a Public IP address and operates as an EAP Proxy between UE and AUSF UE establishes an IPSec with UPF based on the IP address received from 3GPP access and is authenticated by AUSF via UPF/SMF. | This solution eliminates both N1 and N2 signaling over non-3GPP access. Compared with Sol#2.2, the solution does not rely on interworking architecture for EPS and 5GS. Instead of using the SWm interface for device authentication, it proposes the authentication to be done by AUSF through UPF/SMF via a new interface. The solution has similar drawbacks as Sol#2.2 and Sol#2.7. It also introduces an interface for authentication between SMF and AUSF. |
| #2.7 | The UPF has a Public IP address, UE establishes MPQUIC with UPF based on the IP address received from 3GPP access. The security is performed e2e between UE and PSA UPF at MP QUIC layer based on TLS 1.3 | This solution eliminates both N1 and N2 signaling as well as IKE2v/IPSec over non-3GPP access. The solution does not use primary (AKA based) authentication via non-3GPP access and relies on that the UE has been authenticated via 3GPP access. Only MPQUIC functionalities can be applied to this solution. Drawbacks: 1. The solution assumes all the control of the terminal device being done only over 3GPP access. 2. The UE device cannot operate when it is outside of 3GPP coverage. 3. The solution exposes UPF through the use of a Public IP address. 4. The solution does not support all the steering functionalities, e.g. ATSSS-LL or MPTCP cannot be supported. 5. It is unclear how some of the steering modes may work as the Non-3GPP access connectivity is dependent on the availability of 3GPP access e.g. If Active/Standby mode is used with 3GPP access as the active and Non-3GPP access as the standby access, when 3GPP access is not available the UE cannot send traffic through the standby access (i.e. Non-3GPP access) also. 6. This solution does not simplify the UE implementation either because for accessing other services (e.g. voice etc) through Non-3GPP access the UE shall use the existing architecture. On the contrary this will add to the UE implementation complexities as for some services (including for some ATSSS uses cases e.g. ATSSS-LL) the UE has to perform NAS registration over Non-3GPP access and for a few ATSSS use cases (i.e. for MPQUIC) the UE has to implement this new architecture. |
| #2.8 | The UPF has a Public IP address, UE establishes MPQUIC with UPF based on the IP address received from 3GPP access. The security is performed at MP QUIC via the MP QUIC path validation mechanism. This solution addresses the temporary loss of 3GPP access coverage for a MA PDU Session. It specifies a timer-based approach in which the timer indicates how long the MA PDU Session can be active via non-3GPP access without the UE being available in 3GPP access. | This solution eliminates both N1 and N2 signaling as well as IKE2v/IPSec over non-3GPP access. The solution does not use primary (AKA based) authentication via non-3GPP access and relies on that the UE has been authenticated via 3GPP access. Only MPQUIC functionalities can be applied to this solution. This solution has the same drawbacks as Solution#2.7 although it provides a timer-based mechanism to address temporary loss of 3GPP access coverage for the MA PDU session. |
| #2.12 | The 5GC notifies N3IWF when a MA PDU session using QUIC based steering is established. N3IWF negotiates with the UE the use of NULL encryption for all IPSec SAs between UE and N3IWF associated with this MA PDU session | The solution addresses the double encryption for the user plane traffic over non-3GPP access for a MA PDU session using QUIC based steering. The solution has no impacts on either non-3GPP access as well as over 5GC with the exception of providing the notification to N3IWF. The solution has no impacts on the single access PDU session over non-3GPP access and a UE device can access 5G services when only non-3GPP access is available. The solution is only applicable for MA PDU sessions using QUIC based steering. Drawbacks: - The system architecture for ATSSS and the protocol stacks for the Non-3GPP Access leg are not simplified. It still requires the UE cellular modem to send NAS signaling over the non-3GPP modem which results in complex UE implementation. - The solution exposes the UPF IP address as well as the UE IP address, which may lead to exposure of the 5GC inner topology as well as to UE tracking by an attacker. |
| #2.13 | This solution uses similar notification mechanism as Solution #2.12. However, it proposes to completely eliminate the IPSec based tunnelling for user plane for MA PDU sessions using QUIC based steering. | Comparing with solution #2.12, this solution has impact on the untrusted non-3GPP access. Eliminating the IP tunnelling mechanism between UE and N3IWF, it is FFS how this solution forwards IP packets to/from UPF. Drawbacks: - Same drawbacks as in Solution 2.12 |

Four of the proposed solutions (2.2, 2.6, 2.7 and 2.8) eliminate both N1 and N2 signalling over non-3GPP access and two of them (2.7 and 2.8) also eliminate IKEv2 and IPSec tunnelling over non-3GPP access. As a result, each of these solutions assume the need of the MA PDU session to always have 3GPP access available, although Solution 2.8 addresses also the case in which for a limited duration (timer based) the MA PDU Session can keep the user plane over non-3GPP access when the 3GPP access is unavailable.

Two solutions (2.12 and 2.13) address the complexity of using double encryption over non-3GPP access for MA PDU session using QUIC based steering. Solution 2.13 proposes to completely eliminate IPSec tunnelling for user plane associated with MA PDU session. However, how the traffic is going to be routed over non-3GPP access as well as the matching between non-3GPP access and the N3 interface with the UPF is FFS.

Solution 2.12 instead proposes only the use of NULL encryption between UE and N3IWF when N3IWF is notified that the associated user plane traffic belongs to a MA PDU session using QUIC based steering.

# >>>>END OF CHANGES<<<<