SA WG2 Meeting #164	S2-2407516

# Maastricht, Aug 19th–Aug 23rd, 2024        											(was S2-)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1309 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Misalignment text in AM and UE policy association with respect to local configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_MINPA |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 23.502, section 4.16.1.2 mentions NOTE 1: The AM Policy Association indicator set to "disabled" or "enabled" from UDM takes precedence over the local policies in AMF. NOTE 2: The absence of the AM Policy Association indicator from UDM triggers the AMF to check the local policies. 23.503, section 6.1.2.1.2 has a misaligned text, where in the value of “disabled” is also considered to check the local configuration to determine the policy association, which is incorrect. If its value is "disabled" or no AM Policy Association indicator is received from the UDM then the AMF determines whether to establish the AM Policy Association or not according to local configuration. Similarly, 23.503, section 6.1.2.2.5 has a similar misalignment text. Note that the TEI19_MINPA WID also states “The Access and Mobility Subscription Data Type is extended with an indication of whether the UE Policy Association is enabled, disabled or no control of UE Policies. The UE Policy Association indication is enabled for SUPIs that are subscribed to a service that requires sending UE Policies to the UE. The UE Policy Association indication is disabled for SUPIs that are not subscribed to any service that requires sending UE Policies to the UE. The absence of the UE Policy Association indication means “no control of UE Policies” for a SUPI, the AMF applies the existing local configuration and the presence of the UE Policy Container as defined in previous releases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes as per the reason for change documented above. It is proposed to correct the misalignment by removing the text “If its value is "disabled" or” in section 6.1.2.1.2. Similar text in section 6.1.2.2.5 is proposed to be removed with respect to the UE policy association. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specifications and misalignment between 23.502 and 23.503 exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.1.2, 6.1.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.1.2.1.2	Handling of the AM Policy Association

In the non-roaming scenario, the establishment of the AM Policy Association is determined as follows. First, the AMF checks the AM Policy Association indicator received from UDM and establishes the AM Policy Association if its value is "enabled" and do not establish the AM Policy Association if its value is "disabled". If no AM Policy Association indicator is received from the UDM then the AMF determines whether to establish the AM Policy Association or not according to local configuration.

In the roaming scenario, only the AMF local configuration is considered for the determination of the establishment of the AM Policy Association.

NOTE 1:	In roaming, the AMF does not consider the value of the AM Policy Association indicator, if received.

The AM Policy Association indicator for a SUPI is part of Access and Mobility Subscription data in UDM as described in clause 5.2.3.3.1 of TS 23.502 [3] and can be set to "enabled" or "disabled".

NOTE 2:	The operator can set the AM Policy Association indicator to "enabled" when it is required to provide AM Policies to the AMF or to "disabled" when there is no AM Policies to be provided to the AMF.

NOTE 3:	A possible trigger to set the AM Policy Association indicator to "enabled" is that either there are "AM influence information" for e.g. a SUPI in UDR or AM Policies are available for e.g. a SUPI provisioned in the PCF. If the AF requests to store AM influence information into UDR, the NEF authorizes the AF request when the AM Policy Association Indicator is set to "enabled". The "AM influence information" is not provided to the AMF when the AM Policy Association Indicator is set to "disabled".

# * * * * Next change * * * *

##### 6.1.2.2.5	Handling of the UE Policy Association

In the non-roaming scenario, the establishment of the UE Policy Association is determined as follows. First, the AMF checks the UE Policy Association indicator received from UDM and establishes the UE Policy Association if its value is "enabled" and do not establish the UE Policy Association if its value is "disabled". If no UE Policy Association indicator is received from the UDM then the AMF establishes the UE Policy Association when the UE Policy Container is received from the UE. If no UE Policy Association indicator is received from the UDM and no UE Policy Container is received from the UE, then the AMF determines whether to establish the UE Policy Association or not as indicated by the local configuration.

In the roaming scenario, the AMF local configuration is considered first for the determination of the establishment of the UE Policy Association then the remaining parameters described above are applied.

NOTE 1:	It is assumed that the AMF local configuration is based on agreement between operators about whether UE policies can be delivered in the roaming scenario.

The UE Policy Association indicator for a SUPI is part of Access and Mobility Subscription data in UDM as described in clause 5.2.3.3.1 of TS 23.502 [3] and can be set to "enabled" or "disabled".

NOTE 2:	The operator can set the UE Policy Association indicator to "enabled" when it is required to provision UE Policies to the UE or to "disabled" when the UE is no longer subscribed to any service that requires sending UE policies to the UE.

NOTE 3:	A possible trigger to set the UE Policy Association indicator to "enabled" is that either there are service specific parameters for e.g. a SUPI in UDR or UE Policies are available for e.g. a SUPI provisioned in the PCF. Before setting the indicator to "disabled", the operator needs to ensure that there are no UE Policies stored for any of these services in the UE, by revoking the authorization for the service specific information, for Service Type "AF guidance for URSP", for all of the AFs with active authorizations, as defined in clause 4.15.6.7a of TS 23.502 [3] and by triggering the PCF to remove the UE Policies in the UE if any or both.

# * * * * End of changes * * * *