3GPP TSG-WG SA2 Meeting #164	S2-2407520

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5412 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of identifying non-3GPP Devices Connecting behind a UE or 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to SP-240971 "New WID on User Identities and Authentication Architecture" and conclusions in clause 8.4 of TR 23.700-32, the 5GC may support identifying non-3GPP devices connecting behind a UE or 5G-RG to enable QoS differentiation in handling the traffic of these non-3GPP devices. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the general description for support of identifying non-3GPP devices connecting behind a UE or 5G-RG. - Add the relevant functionalities for the PCF and UDR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of identifying non-3GPP devices connecting behind a UE or 5G-RG by the 5GC is unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.xx (new), 6.2.4, 6.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

5.xx	Support of identifying non-3GPP Devices Connecting behind a UE or 5G-RG

Non-3GPP devices connected behind a UE or 5G-RG to 5GC may be identified with a Device Identifier, which enables the 5GC to associate the traffic from/to each individual non-3GPP device with the Device Identifier and thus provision different QoS settings for the traffic of different non-3GPP devices behind a UE or 5G-RG.

The Device Identifier of a Non-3GPP device connected behind a UE or 5G-RG is a permanent identifier and can take the form of a generic string. The UE or 5G-RG shall bind the Device Identifier to a non-3GPP device.

NOTE 1:	Allocation of the Device Identifier is out of scope of 3GPP.

NOTE 2:	Authentication for non-3GPP device is performed by UE or 5G-RG, which is out of scope of 3GPP.

NOTE 3:	Transmission of the traffic of Non-3GPP devices which are connected behind a 5G-RG and not identified by a Device Identifier is supported using the Connectivity Group ID mechanism as described in clause 4.10b of TS 23.316 [84].

For non-3GPP devices requiring QoS differentiation:

-	Device Identifiers and their corresponding QoS/Policies associated with a UE/5G-RG subscription are provisioned into the UDR by the AF.

-	QoS differentiation can be performed within a PDU Session using different QoS Flows or between PDU Sessions. In the case of non-3GPP devices sharing a PDU Session, QoS Flows are provisioned in the UE/5G-RG using PDU Session Modification procedures.

-	During PDU Session establishment or PDU Session modification procedures, the UE/5G-RG may send the Device Identifier and user plane address of the non-3GPP device to the SMF, which then forwards the received Device Identifier and user plane address to the PCF. Based on the Device Identifier of the non-3GPP device and the Device Identifier information retrieved from the UDR, the PCF makes policy decisions. In addition to the QoS information used by the UE/5G-RG for QoS flow transmission between the UE/5G-RG and the network, the PCF may also provide Non-3GPP QoS Assistance Information (N3QAI) per QoS flow in PCC rules. When providing the UE/5G-RG with QoS flow descriptions, the SMF may additionally signal N3QAI for each QoS flow to the UE/5G-RG based on the Device Identifier. The UE/5G-RG determines the QoS per QoS flow between non-3GPP devices and the UE in the non-3GPP network based on the N3QAI.

-	UE/5G-RG may also send the Device Identifier of the non-3GPP device to the SMF in DHCPv6 message to associate the Device Identifier with the user plane address.

-	When using an IP PDU Session Type, the user plane address is the UE IP address(es) and Port Number(s) that carry the non-3GPP device's traffic. When using an Ethernet PDU Session Type, the user plane address is the MAC address and/or the VLAN tag ID that is associated with the non-3GPP device's traffic.

Optionally, the 5GS can restrict the max number of simultaneously active Device Identifiers per UE/5G-RG for device whose traffic need to be differentiated.

* * * Next Change * * * *

6.2.4	PCF

The Policy Control Function (PCF) includes the following functionality:

-	Supports unified policy framework to govern network behaviour.

-	Provides policy rules to Control Plane function(s) to enforce them.

-	Provides policy rules to UE to guide UE behaviour.

-	Accesses subscription information relevant for policy decisions in a Unified Data Repository (UDR).

-	Support PDU Set Handling as defined in clause 5.37.5.

NOTE:	The PCF accesses the UDR located in the same PLMN as the PCF.

The details of the PCF functionality are defined in clause 6.2.1 of TS 23.503 [45].

* * * Next Change * * * *

6.2.11	UDR

The Unified Data Repository (UDR) supports the following functionality:

-	Storage and retrieval of subscription data by the UDM.

-	Storage and retrieval of policy data by the PCF.

-	Storage and retrieval of structured data for exposure.

-	Storage and retrieval of application data (including e.g. Packet Flow Descriptions (PFDs) for application detection, AF request information for multiple UEs, 5G-VN group information for 5G-VN management).

-	Storage and retrieval of NF Group ID corresponding to subscriber identifier (e.g. IMPI, IMPU, SUPI).

-	Storage and retrieval of non-3GPP device profile associated with a UE or 5G-RG subscription.

The Unified Data Repository is located in the same PLMN as the NF service consumers storing in and retrieving data from it using Nudr. Nudr is an intra-PLMN interface.

NOTE 1:	Deployments can choose to collocate UDR with UDSF.

* * * End of Changes * * * *