3GPP TSG-WG SA2 Meeting #164	S2-2407529

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1101 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF assisted QoS policy generation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, China Mobile, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For KI#3 "NWDAF-assisted policy control and QoS enhancement", clause 8.3 of TR 23.700-84 includes the following conclusions: “Based on PCF request, NWDAF may provide assistance information analytics to PCF to assist the QoS determination and/or modification of QoS parameters. The PCF may take the assistance information into account to determine QoS parameters for a QoS flow, based on the operator's policy. After the PCF determines the QoS parameters, if updated assistance information is received from NWDAF (the updated assistance information may include candidate QoS parameters, the analytics of the duration and usage of established QoS Flows), the PCF may determine to modify the QoS parameters for a QoS flow and provide the modified QoS towards the SMF, based on the operator's policy. In particular: - The PCF may send a analytics request or subscription to NWDAF for QoS and policy assistance information analytics. The PCF may include an analytics ID, Filter Information, one or multiple sets of QoS parameters, the candidate value list(s) for each QoS parameter, requesting expected service experience (e.g. QoE), etc. - Based on the PCF request, the QoS and policy assistance information analytics provided by NWDAF may include: expected QoE can be achieved by the candidate QoS parameters (the values of the QoS parameters are within the candidate value list provided by the PCF). ” Based on the conclusions, the description and procedure that the NWDAF generates QoS policy assistance information analytics based on PCF request are proposed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description and procedure that the NWDAF generates QoS policy assistance information analytics based on PCF request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How the NWDAF generates QoS policy assistance information based on PCF request is unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), 6.x.1 (new), 6.x.2 (new), 6.x.3 (new), 6.x.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

6.x	QoS policy assistance information analytics

6.x.1	General

Based on PCF request, NWDAF may provide QoS policy assistance information analytics to assist the PCF in QoS policy determination and modification. The PCF may take the assistance information into account to determine QoS policy (e.g. QoS parameters for a QoS flow), based on the operator's policy.

The consumer (i.e. PCF) of the analytics may indicate the following parameters in the request:

-	Analytics ID = "QoS policy assistance information";

-	Target of Analytics Reporting: a single UE (SUPI), group of UEs (i.e. a list of Internal Group Ids) or any UE;

-	Analytics Filter Information optionally containing:

-	S-NSSAI;

-	DNN;

-	Area of Interest;

-	Application ID;

-	Optionally, Candidate QoS policy for QoS flow, which includes:

-	Service data flow template as defined in clause 6.3.1 of TS 23.503 [4];

-	One or multiple of the following QoS parameters and/or QoS characteristics as defined in clause 5.7 of TS 23.501 [2], each QoS parameter or QoS characteristic optionally associated with a candidate value list:

-	5QI (standardized or pre-configured);

-	For a pre-configured (i.e. non-standardized) 5QI, the associated 5G QoS characteristics as described in clause 5.7.3 of TS 23.501 [2], including: Resource Type, Priority Level, Packet Delay Budget, Packet Error Rate, Averaging Window, Maximum Data Burst Volume (MDBV).

-	ARP;

-	RQA;

-	GFBR and MFBR for GBR QoS flows;

-	Maximum Packet Loss Rate;

-	Optionally, Target service experience, including service experience type (e.g. voice, video) and expected service experience value(s) (average, variance) over the Analytics target period, as described in clause 6.4;

-	Optionally, Target network performance, including e.g. expected gNB status information, gNB resource usage, gNB resource usage for GBR traffic or for Delay-critical GBR traffic, number of UEs and communication performance over the Analytics target period, as described in clause 6.6;

-	Optionally, preferred level of accuracy of the analytics;

-	Optionally, preferred level of accuracy per analytics subset;

-	Analytics target period: indicates the time period over which the analytics are requested;

-	In a subscription, Notification Correlation Id and Notification Target Address are included.

6.x.2	Input Data

In order to produce QoS policy assistance information analytics, the NWDAF collects the following data:

-	Service data flow information and corresponding QoS parameters, as described in Table 6.x.2-1.

-	Input data for network performance Analytics as defined in clause 6.6.2;

-	Input data for observed service experience analytics as defined in clause 6.4.2.

Table 6.x.2-1 Application service information and corresponding QoS parameters collected by the NWDAF

| Information | Source | Description |
| --- | --- | --- |
| Service data flow information | PCF | For IP PDU traffic: Either a list of service data flow filters or an application identifier of the service data flow. For Ethernet PDU traffic: Combination of traffic patterns of the Ethernet PDU traffic. See clause 6.3.1 of TS 23.503 [4]. |
| QoS parameters | PCF | QoS parameters and QoS characteristics for QoS Flow / service data flow, see clause 5.7 of TS 23.501 [2] and clause 6.1.3.22 of TS 23.503 [4]. |

6.x.3	Output Analytics

The analytics results for QoS policy assistance information analytics provided by the NWDAF are described in Table 6.x.3-1 and Table 6.x.3-2.

Table 6.x.3-1: QoS policy assistance information

| Information | Description |
| --- | --- |
| QoS Policy for QoS flow (NOTE 1) | Optimal QoS policy for the QoS flow. |
| > Service data flow template | Service data flow template as defined in clause 6.3.1 of TS 23.503 [4]. |
| > 5G QoS Identifier (5QI) | The 5QI authorized for the service data flow. |
| > ARP | The Allocation and Retention Priority for the service data flow consisting of the priority level, the pre-emption capability and the pre-emption vulnerability |
| > Reflective QoS Control | Indicates whether or not to apply reflective QoS for the SDF. |
| > UL GFBR | The uplink guaranteed bitrate for the service data flow |
| > DL GFBR | The downlink guaranteed bitrate for the service data flow |
| > UL MFBR | The uplink maximum bitrate for the service data flow |
| > DL MFBR | The downlink maximum bitrate for the service data flow |
| > UL Maximum Packet Loss Rate | The maximum rate for lost packets that can be tolerated in the uplink direction for the service data flow. |
| > DL Maximum Packet Loss Rate | The maximum rate for lost packets that can be tolerated in the downlink direction for the service data flow. |
| > Resource Type (NOTE 2) | Indicates whether the QoS Flow is Non-GBR, GBR or Delay-critical GBR QoS Flow. |
| > Priority Level (NOTE 2) | Indicates a priority in scheduling resources among QoS Flows. |
| > Packet Delay Budget (NOTE 2) | Indicates an upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF. |
| > Packet Error Rate (NOTE 2) | Indicates an upper bound for a rate of non-congestion related packet losses, see clause 5.7.3.5 of TS 23.501 [2]. |
| > Averaging Window (NOTE 2) | Indicates the duration over which the guaranteed and maximum bitrate shall be calculated. |
| > Maximum Data Burst Volume (NOTE 2) | Indicates the largest amount of data that is required to be transferred within a period of 5G-AN PDB. |
| Expected application service experience (1..max) | Expected service experience per application when the optimal QoS policy is applied in the network. The information is the same as the application service experiences defined in Table 6.4.3-2. |
| Expected network performance (1..max) | Expected network performance when the optimal QoS policy is applied in the network. The information is the same as the network performance information defined in Table 6.6.3-2. |
| Spatial validity (NOTE 3) | Area where the QoS policy assistance information applies. |
| Validity period | Time period for which the QoS policy assistance information is valid. |
| NOTE 1: The values of the QoS parameters and QoS characteristics are within the candidate value lists if provided by the PCF. NOTE 2: Optional and applicable only when the 5QI value is pre-configured (i.e. non-standardized). NOTE 3: Spatial validity is present in the output analytics if the consumer provided the Area of Interest as described in clause 6.x.2. |  |

6.x.4	Procedures for Providing QoS Policy Assistance Information Analytics

Figure 6.x.4-1: Procedure for NWDAF providing QoS policy assistance information analytics

1.	The PCF sends Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request (Analytics ID="QoS policy assistance information", Target of Analytics Reporting, Analytics Filter Information, Candidate QoS policy, Target service experience, Target network performance) to the NWDAF.

2a-2b. NWDAF subscribes to the service data from AF for observed service experience analytics as defined in clause 6.4.2 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service operation.

2c-2d.	NWDAF subscribes to the network data from 5GC NF(s) (e.g. AMF, SMF, UPF) for observed service experience analytics as defined in clause 6.4.2 and network performance analytics as defined in clause 6.6.2 by invoking Nnf_EventExposure_Subscribe service operation.

2e-2f.	NWDAF subscribes to the QoS policy information as defined in clause 6.x.2 from the PCF by invoking Npcf_EventExposure_Subscribe service operation.

2g.	NWDAF subscribes to OAM services to collect data for observed service experience analytics as defined in clause 6.4.2 and network performance analytics as defined in clause 6.6.2.

3.	Based on the input data, parameters indicated in the analytics request/subscription (e.g. Target service experience, Target network performance) and local configuration, the NWDAF trains the ML model, derives service experience and network performance analytics information resulted from the enforced QoS policy, and further generates the QoS policy assistance information, i.e. optimal QoS policy, expected service experience and optionally expected network performance information.

4.	The NWDAF provides the QoS policy assistance information to the PCF via Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5.	The PCF determines QoS policy taking into account of the QoS policy assistance information (i.e. adopting or modifying the QoS policy in the QoS policy assistance information to get the QoS policy for enforcement), and provisions the determined QoS policy to the SMF. The SMF interacts with the NG-RAN, UPF and UE to enforce the QoS policy as specified in TS 23.503 [4].

6-9.	The AF, AMF/SMF/UPF, PCF and OAM sends notifications of new data related to service experience and network performance after enforcement of the QoS policy to the NWDAF. The NWDAF may optimize the ML model based on service experience and network performance analytics derived with the new input data, to generate new QoS policy assistance information analytics to provide to the PCF. The PCF may then update the QoS policy based on the new optimal QoS policy in the QoS policy assistance information and provide the updated QoS policy towards the SMF.

* * * End of Changes * * * *