3GPP TSG-WG SA2 Meeting #164	S2-2407531

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1103 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS policy assistance information analytics / ML model accuracy monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, China Mobile, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For KI#3 "NWDAF-assisted policy control and QoS enhancement", clause 8.3 of TR 23.700-84 includes the following conclusions: “Based on PCF request, NWDAF may provide assistance information analytics to PCF to assist the QoS determination and/or modification of QoS parameters. The PCF may take the assistance information into account to determine QoS parameters for a QoS flow, based on the operator's policy. After the PCF determines the QoS parameters, if updated assistance information is received from NWDAF (the updated assistance information may include candidate QoS parameters, the analytics of the duration and usage of established QoS Flows), the PCF may determine to modify the QoS parameters for a QoS flow and provide the modified QoS towards the SMF, based on the operator's policy.” To monitor the performance (i.e. accuracy) of the QoS policy assistance information analytics and ML model, the PCF may provide the determined QoS policy based on the assistance information as the Analytics Feedback Information, which can assist the NWDAF in deciding whether to retrain the ML model and/or re-generate the analytics for QoS policy assistance information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add descriptions on accuracy monitoring for QoS policy assistance information analytics and ML model. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to monitor the accuracy of QoS policy assistance information analytics and ML model. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

6.x.5	Procedures for QoS Policy Assistance Information Analytics / ML Model Accuracy Monitoring

For QoS policy assistance information analytics and ML Model accuracy monitoring, the procedures described in clause 6.2D and clause 6.2E apply, where the NWDAF service consumer (i.e. PCF) may provide the QoS policy for QoS flow, which is sent to the SMF for enforcement and can be the same or modified QoS policy as provided in the QoS policy assistance information by the NWDAF, in the Analytics Feedback Information. Based on the received QoS policy in the Analytics Feedback Information, the NWDAF containing MTLF may optimize the ML model by performing ML model retraining with newly collected data and re-provision the ML model for QoS policy assistance information analytics, and the NWDAF containing AnLF may re-generate the QoS policy assistance information analytics to provide to the PCF.

* * * End of Changes * * * *