SA WG2 Meeting #164 	S2-2407544

# 19 - 23 August 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1107 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of VFL related-features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-8-1 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR propose to add the agreed VFL related feature descriptions in TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add high-level description of VFL features. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No VFL features in TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

4	Reference Architecture for Data Analytics

4.1	General

The NWDAF (Network Data Analytics Function) is part of the architecture specified in TS 23.501 [2] and uses the mechanisms and interfaces specified for 5GC in TS 23.501 [2] and OAM services (see clause 6.2.3.1).

The NWDAF may support the following functionalities:

-	Data collection based on subscription to events provided by AMF, SMF, UPF, PCF, UDM, NSACF, AF (directly or via NEF) and OAM;

-	Analytics and Data collection using the DCCF (Data Collection Coordination Function);

-	Retrieval of information from data repositories (e.g. from UDR via UDM for subscriber-related information or optinally via NEF(PFDF) for PFD information);

-	Data collection of location information using LCS (finer granularity location information determined by LMF);

-	Storage and retrieval of information from ADRF (Analytics Data Repository Function);

-	Analytics and Data collection from MFAF (Messaging Framework Adaptor Function);

-	Retrieval of information about NFs (e.g. from NRF for NF-related information);

-	On demand provision of analytics to consumers, as specified in clause 6.

-	Provision of bulked data related to Analytics ID(s).

-	Provision of Accuracy information about Analytics ID(s).

-	Provision or retrieval of ML Model Accuracy Information or ML Model accuracy degradation about a ML Model.

-	Perform Vertical Federated Learning as VFL Server and/or VFL Client

A single instance or multiple instances of NWDAF may be deployed in a PLMN. If multiple NWDAF instances are deployed, the architecture supports deploying the NWDAF as a central NF, as a collection of distributed NFs, or as a combination of both. If multiple NWDAF instances are deployed, an NWDAF can act as an aggregate point (i.e. Aggregator NWDAF) and collect analytics information from other NWDAFs, which may have different Serving Areas, to produce the aggregated analytics (per Analytics ID), possibly with Analytics generated by itself.

NOTE 1:	When multiple NWDAFs exist, not all of them need to be able to provide the same type of analytics results, i.e. some of them can be specialized in providing certain types of analytics. An Analytics ID information element is used to identify the type of supported analytics that NWDAF can generate.

NOTE 2:	NWDAF instance(s) can be collocated with a 5GS NF.

* * * Next Change * * *

- x	Vertical Federated Learning(VFL) among NWDAFs and NWDAF/AF

Vertical Federated Learning is one of the machine learning techniques that trains an ML Model across multiple decentralized entities(e.g. NWDAF and AF) holding local data set, without exchanging/sharing local data set.

There are two scenarios for VFL in 5GC:

-	VFL among NWDAFs in a single PLMN(NWDAF initiated VFL training or VFL inference).

-	VFL between AF and NWDAF(s) in a single PLMN(AF or NWDAF initiated VFL training or VFL inference).

When an AF initiates the VFL, then only one AF is involved. When the NWDAF initiates the VFL, then multiple AFs and NWDAFs can be involved.

For VFL training or VFL inference supported by multiple NWDAFs containing MTLF, there is one NWDAF containing MTLF acting as VFL server and multiple NWDAFs containing MTLF acting as VFL client.

For VFL training or VFL inference supported between NWDAF and AF, there is one VFL server(i.e. a NWDAF containing MTLF or an AF) and multiple VFL Client(i.e. NWDAF containing MTLF or AF).

The NWDAFs act as VFL Server determines based on internal logic and the operator's policy whether or not to use VFL to provide a particular analytics ID.

The main functionalities of VFL server and VFL Client includes:

VFL server(NWDAF or AF):

-	triggers the VFL training process and VFL inference process

-	discovers and selects VFL client NWDAFs and/or AF to participant in an VFL training process

-    triggers sample alignment process, generates the interclause of samples to be used in the VFL training process.

-	requests VFL clients to do local model training and to report intermediate training result.

-	trains a ML Model and computes intermediate result(e.g. loss information, gradient information) for local ML model update at VFL server and VFL clients.

The loss information may contain loss function, type of loss function and value of loss. The definition of loss information is out of 3GPP scope.

-	determines to terminate the VFL training process(e.g. when the loss information is converged, the ML Model metric(i.e. ML Model accuracy) satisfy consumer requirement or the pre-set number of iteration of VFL training is achieved) and sends the VFL training end notifications to VFL clients.

VFL client(NWDAF or AF):

-	locally trains ML Model as tasked by the VFL server with the available local data set, which includes the data that may not be allowed to be shared with other VFL clients due to e.g. data privacy, data security, data access rights.

-	Perform VFL inference guided by VFL server.

-	reports the intermediate training result to VFL server.

-	Perform local ML Model update base on the loss information or gradient information received from VFL server.

VFL server NWDAF/AF or FL client NWDAF/AF register to NRF with their VFL capability information as described in clause X.

When starting an VFL training procedure, the VFL server is to provide an initial model to each VFL client NWDAF, and then each VFL client NWDAF is to perform local model training using its local data set. The detailed procedure for VFL training is described in clauseY.

Before VFL procedure is initiated by VFL server, appropriate VFL client  should be discovered by VFL server as described in clause Z.

Editor's note:	The clause reference will be updated in future meetings.

* * * End of changes * * *