SA WG2 Meeting #164	S2-2407547

# Maastricht, Netherlands, 19 – 23 August, 2024

Source: 	SK Telecom, Verizon

Title: 	Editorial correction in conclusion of KI#4

Document for: 	Approval

Agenda Item: 	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This contribution corrects the general expression regarding the cause of the signalling storm conclusion related to KI#4.

1	Discussion

Clarifies the ambiguous parts in the expression "massive signalling of UEs and/or NF" as the cause of the signalling storm.

2	Proposal

It is proposed to update TR 23.700-84 as follows.

# * * * * First change * * * *

8.4	Conclusions for KI#4: NWDAF-assisted Network Abnormal Behaviour Mitigation and Prevention

It is agreed to use following principles as baseline for normative work.

-	By means of statistics and predictions, NWDAF supports assistance to signalling storm mitigation and prevention by statistically learning the expected normal level of signalling and identifying a significant deviation indicating a signalling storm caused by massive signalling of UEs and/or NFs.

-	A new Analytics ID to support Signalling storm Mitigation and Prevention caused by NFs signalling will be defined.

-	It will be determined in the normative phase whether to define a new Analytics ID or enhance an existing Analytics ID to support Signalling storm Mitigation and Prevention caused by massive signalling of UEs.

-	Analytics Filter includes information to assist the NWDAF for the signalling storm as defined above.

NOTE 1:	The consumers of the analytics ID and the parameters of the analytics request/subscribe (determined by the consumer) are to be specified in normative work.

-	The NWDAF identifies target NFs to obtain data based on the Analytics Filter.

-	The following Input Data are agreed to be collected from NF, MDAF, AF and/or via NRF to support generation of the analytics by NWDAF:

-	Information related to control plane signalling exchange, including:

-	Request Type of control messages (and related posterior messages), message counts, success and failures, failure cause, fault information, response times for N1 and/or N2 interfaces and 5GC SBIs, taking information as described in clause 6.35.2.2 and table 6.39.1-1.

-	Information on NF contexts, including:

-	UE related number, state transition and timer information of a group of UEs as described in clause 6.35.2.2 and table 6.39.1-1;

-	NF load, status & resource usage as described in table 6.39.1-1.

-	UE related data per group, including:

-	input data of existing UE related analytics.

-	MDAF data, including:

-	5GC Control Plane congestion status/level and/or aggregated data.

-	AF data, including:

-	user active/inactive information as described in table 6.36.1-2.

NOTE 2:	The details of input data (including source and detailed data description) are to be specified in normative work. Input data already available in earlier releases are re-used.

NOTE 3:	The NWDAF collects UE related data via NFs (e.g. AMF, etc.) as specified in TS 23.288 [5].

-	In order to reduce signalling overhead, offline bulk data report (clause 6.2.6.1 in TS 23.288 [5]) for UE related data collection may be used.

-	The following Output Data from NWDAF are agreed to be included in Analytics output:

-	Statistics Analytics, including:

-	Target NF ID, cause of the signalling storm (i.e. signalling storm caused by UEs and signalling storm caused by NF), NF lists or a group of UEs, statistics corresponding to the input data.

-	Prediction Analytics, including:

-	Target NF ID, cause of the signalling storm, NF lists or a group of UEs, prediction corresponding to the input data, confidence level of the prediction.

NOTE 4:	Whether more IEs are needed will be discussed and specified in normative work.

-	Examples of mitigation or prevention for NFs will be discussed and provided in the normative work, considering a similar format as in table 6.7.5.3-3 in TS 23.288 [5] as a basis, the final mitigation or prevention operations are based on operator's policy/configuration and NF implementation. The mitigation and prevention described in Solutions #35 and #39 will be consider for used as a basis for such examples:

-	UE-related configurations (back-off timers, thresholds, slice or priority based overload control, etc.);

-	NF-related configuration ((re)discovery/selection, prioritizations, threshold, etc.).

NOTE 5:	Examples of prevention/mitigation actions based on NWDAF analytics on signalling storm prediction and/or detection may be further refined during the course of the normative work.

NOTE 6:	Whether SCP entity itself can be a source of input data will be discussed and specified in normative work.

# * * * * End of Changes * * * *