3GPP TSG-WG SA2 Meeting #164	S2-2407556

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0536 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMF event exposure to support data collection for AI/ML based positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following conclusions have been reached for KI#1 "Enhancements to LCS to support Direct AI/ML based Positioning” in clause 8.1 of TR 23.700-84: Principle #1: The LMF is enhanced to perform location calculation based on ML model. The interaction between LMF and MTLF is only for model provisioning and possibly, based on coordination with RAN WGs, for data collection for training. Principle #3: MTLF is enhanced to perform model training for AI/ML based Positioning. - Principle #3.2: NWDAF containing MTLF trains ML model for AI/ML based Positioning based on request from the LMF or internal trigger. - Principle #3.3: MTLF collects training data (e.g. data as defined by RAN for AI/ML based Positioning and historical data stored in ADRF) from data sources. Accordingly, the LMF needs to provide the service for the NWDAF containing MTLF to collects data from the LMF for ML model training for AI/ML based positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define LMF service that is used by the NWDAF containing MTLF to collect data from the LMF to train ML model(s) for AI/ML based positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not supported that the NWDAF containing MTLF collects data from the LMF for ML model training for AI/ML based positioning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1, 8.3.x (new), 8.3.x.1 (new), 8.3.x.2 (new), 8.3.x.3 (new), 8.3.x.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

8.3.1	General

The following table shows the LMF Services and LMF Service Operations.

Table 8.3.1-1: List of LMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nlmf_Location | DetermineLocation | Request/Response | AMF |
|  | EventNotify | Notify | GMLC |
|  | CancelLocation | Request/Response | AMF |
|  | LocationContextTransfer | Request/Response | LMF |
|  | MeasurementData | Request/Response | LMF |
|  | UPConfig | Request/Response | AMF |
|  | UPSubcribe | Subscribe/Notify | AMF |
|  | UPNotify |  | AMF |
| Nlmf_Broadcast | CipheringKeyData | Notify | AMF |
| Nlmf_EventExposure | Subscribe | Subscribe/Notify | NWDAF |
|  | UnSubscribe | Subscribe/Notify | NWDAF |
|  | Notify | Subscribe/Notify | NWDAF |

* * * Next Change * * * *

8.3.x	Nlmf_EventExposure service

8.3.x.1	General

Service description: This service enables an NF to subscribe and get notified about events.

The following service operations are defined for the Nlmf_EventExposure service:

-	Nlmf_EventExposure_Subscribe.

-	Nlmf_EventExposure_Unsubscribe.

-	Nlmf_EventExposure_Notify.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	UE positioning information, as defined in clause 6.x.2 of TS 23.288 [37].

Event Filters are used to specify the conditions to match for notifying the event (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides some examples on how the conditions to match for event reporting can be specified for various Event IDs for LMF exposure.

Table 8.3.x.1-1: Example of Event Filters for LMF exposure events

| Event ID for LMF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| UE positioning information | <Parameter Type = TAI, Value = TAI1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |

8.3.x.2	Nlmf_EventExposure_Subscribe service operation

Service operation name: Nlmf_EventExposure_Subscribe.

Description: The consumer NF uses this service operation to subscribe to or modify event reporting for one UE, a group of UE(s) or any UE.

Input, Required: Target of Event Reporting: UE ID(s) (SUPI(s), or Internal Group Identifier, or indication that any UE is targeted), Event ID(s), Notification Target Address (+ Notification Correlation ID), Event Reporting Information.

Input, Optional: NF ID, Event Filter (s) associated with each Event ID, Event Filter (s), Subscription Correlation ID (in the case of modification of the existing subscription), Expiry time.

Output, Required: Operation execution result indication. When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the local policy).

Output, Optional: First corresponding event report is included, if available.

8.3.x.3	Nlmf_EventExposure_UnSubscribe service operation

Service operation name: Nlmf_EventExposure_UnSubscribe.

Description: The NF consumer uses this service operation to unsubscribe for a specific event.

Input, Required: Subscription Correlation ID.

Input, Optional: None.

Output, Required: Operation execution result indication.

Output, Optional: None.

8.3.x.4	Nlmf_EventExposure_Notify service operation

Service operation name: Nlmf_EventExposure_Notify.

Service operation description: The PCF provides the subscribed event information to the NF Consumer which has subscribed to that event.

Input, Required: Notification Correlation Information, Event ID, corresponding UE ID(s), time stamp.

Input, Optional: Event specific parameter list,

Output, Required: None.

Output, Optional: None.

* * * End of Changes * * * *