3GPP TSG-WG SA2 Meeting #164	S2-2407559

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1111 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF service enhancements for AI/ML based positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following conclusions have been reached for KI#1 "Enhancements to LCS to support Direct AI/ML based Positioning” in clause 8.1 of TR 23.700-84: Principle #1: The LMF is enhanced to perform location calculation based on ML model. The interaction between LMF and MTLF is only for model provisioning and possibly, based on coordination with RAN WGs, for data collection for training. Principle #3: MTLF is enhanced to perform model training for AI/ML based Positioning. - Principle #3.2: NWDAF containing MTLF trains ML model for AI/ML based Positioning based on request from the LMF or internal trigger. - Principle #3.3: MTLF collects training data (e.g. data as defined by RAN for AI/ML based Positioning and historical data stored in ADRF) from data sources. Based on the above conclusions, enhancements on NWDAF service operations for provisioning ML model for AI/ML based positioning are proposed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add enhancements on NWDAF service operations for provisioning ML model for AI/ML based positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Model provisioning by the NWDAF containing MTLF for AI/ML based positioning is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2, 7.5.4, 7.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

7.5.2	Nnwdaf_MLModelProvision_Subscribe service operation

Service operation name: Nnwdaf_MLModelProvision_Subscribe

Description: Subscribes to NWDAF ML Model provision with specific parameters.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2 or Indication of ML model for UE positioning, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional:

-	Subscription Correlation ID (in the case of modification of the ML Model subscription).

-	For each Analytics ID or Indication of ML model for UE positioning, the following parameters may be provided: ML Model Filter Information to indicate the conditions for which ML Model for the analytics is requested and Target of ML Model Reporting as defined in clause 6.2A.2 and clause 6.x.1, Requested representative ratio, ML Model Target Period, Use case context, Inference Input Data information, indication of support for multiple ML Models, multiple ML Models Filter Information to indicate the conditions for which multiple ML Models are requested, ML Model Interoperability Information.

-	NF consumer information, ML Model Reporting Information, Time when model is needed, ML Model Monitoring Information (including e.g. desired ML Model metric, ML Model monitoring reporting mode, ML Model Accuracy Threshold, DataSetTag and ADRF ID, ML Model identifier), Expiry time.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: None.

* * * Next Change * * * *

7.5.4	Nnwdaf_MLModelProvision_Notify service operation

Service operation name: Nnwdaf_MLModelProvision_Notify

Description: NWDAF notifies the ML Model information to the consumer instance which has subscribed to the specific NWDAF service.

Inputs, Required: Notification Correlation Information, Set of:

-	the tuple (Analytics ID or Indication of ML model for UE positioning, one or more tuples of unique ML Model identifier and the information as defined in clause 6.2A.2).

Inputs, Optional: ML Model Accuracy Information.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

* * * Next Change * * * *

7.6.2	Nnwdaf_MLModelInfo_Request service operation

Service operation name: Nnwdaf_MLModelInfo_Request

Description: The consumer requests NWDAF ML Model Information.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2 or Indication of ML model for UE positioning.

Inputs, Optional:

-	For each Analytics ID or Indication of ML model for UE positioning, the following parameters may be provided: ML Model Filter Information to indicate the conditions for which ML Model for the analytics is requested and Target of ML Model Reporting as defined in clause 6.2A.2 and clause 6.x.1, Requested representative ratio, ML Model Target Period, Use case context, Inference Input Data Information, indication of support for multiple ML Models, multiple ML Models Filter Information to indicate the conditions for which multiple ML Models are requested, ML Model Interoperability Information.

-	NF consumer information.

-	ML Model Accuracy Monitoring Information.

Outputs, Required: Set of:

-	the tuple (Analytics ID or Indication of ML model for UE positioning, one or more tuples of unique ML Model identifier and the information as defined in clause 6.2A.2.

Outputs, Optional: ML Model Accuracy Information.

* * * End of Changes * * * *