3GPP TSG-SA2 Meeting #164	S2-2407570

# 19 – 23 August 2024, Maastricht, Netherlands							(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1117 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI2: Vertical Federated Learning Procedure where FL Server is AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-07-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following CR adds description for Vertical Federated Learning procedure when the FL Server is an AF taking into account the agreed Principles P#2.1 through P#2.5 in clause 8.2 of 3GPP TR 23.700-84 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding descriptions for vertical federated learning procedure where the FL Server is an AF in a new sub-clause under clause 6.2C.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details on vertical federated learning procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.2.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

##### .6.2C.2.2.z	Support for Model Training using Vertical Federated Learning where FL Server is AF

Figure 6.2C.2.2.z-1: General procedure for Federated Learning among Multiple NWDAF where FL Server is AF

0.	FL Server AF is triggered to train an ML model using Vertical Federated Learning. The trigger is out of scope of 3GPP.

1.	FL Server AF selects NWDAF(s) containing MTLF (FL Client NWDAF(s)) as described in clause 6.2C.2.1).

Editor's Note: How the NEF is involved in the FL Client selection is FFS

NOTE X: The procedure for FL Server AF selecting FL Client AFs is out of scope of 3GPP.

2.	FL Server AF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF(s). The request includes the VFL process ID and may include the maximum response time, the FL Clients has to report the interim VFL parameters (e.g. Loss) to the FL Server AF before the maximum response time elapses. If the request is via the NEF the FL Server AF sends an Nnef_MLModelTraining_Subscribe or Nnef_MLModelTraining Info_Request to the NEF that the NEF forwards to the selected FL Clients.

NOTE Z: The procedure for FL Server AF interacting with other selected FL Client AFs is out of scope of 3GPP.

3.	[Optional] Each FL Client NWDAF determines the local model to train based on the Analytic ID and VFL process ID and collects its local data by using the current mechanism in clause 6.2 if the Client NWDAF has not local data available already. The FL Server may also collect local data to train its own model if local data are available.

NOTE Υ:	The procedure for data collection by the FL Client AF is out of scope of 3GPP

4.	During Vertical Federated Learning training procedure, each FL Client NWDAF further trains its own ML model based on its local data and reports interim VFL parameters (e.g. Loss) to the FL Server AF in Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response.

-	If the FL Client NWDAF is not able to complete the training of the interim local ML Model within the maximum response time provided by the FL Server AF, the FL Client NWDAF shall send the Delay Event Notification that include the delay event indication, an optional cause code (e.g. local ML Model training failure, more time necessary for local ML Model training) and the expected time to complete the training if available to the FL Server AF before the maximum response time elapses.

5.	The FL Server AF trains its own ML model based on interim VFL parameters provided by FL clients and local data information.

6.	The FL Server AF updates or terminates the current FL training process based on the status of the VFL process.

7.	If the FL procedure continues, the FL Server AF sends Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to FL Client NWDAF(s) for next round of Federated Training. The request includes VFL parameters that will be used by each FL Client to train its own ML model.

8.	Each FL Client NWDAF updates its own ML Model based on VFL parameters provided by the FL Server AF at step 7 and local data information..

NOTE G:	The steps 3-8 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

************************************ END OF CHANGES ****************************************