3GPP TSG-SA2 Meeting #164	S2-2407573

# 19 – 23 August 2024, Maastricht, Netherlands							(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1118 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI3: NWDAF-assisted policy control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-07-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following CR adds description for the following conclusion of KI3. The PCF may send a analytics request or subscription to NWDAF for QoS and policy assistance information analytics. The PCF may include an analytics ID, Filter Information, one or multiple sets of QoS parameters, the candidate value list(s) for each QoS parameter, requesting expected service experience (e.g. QoE), etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed the NWDAF to derive new analytics (QoS and Policy Assistance Analytics) in order to satisfy the conclusions of Key Issue 3. The consumer (i.e. PCF) request such analytics by included a candidate list of QoS parameters and an expected QoE target. The analytics provided by the NWDAF provides the QoS parameter from the list of candidate QoS parameter requested by a consumer (i.e. PCF) that meets (or has the least variance) the expected QoE target. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions of KI3 not addressed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

## 6.x	Procedure for QoS and Policy Assistance Analytics

##### 6.x.1	General

The NWDAF provides QoS and Policy Assistance Analytics to a PCF.

The QoS and Policy Assistance Analytics provides the QoS parameter from a list of candidate QoS parameters provided by the PCF, that is closer to the expected Quality of Experience metric.

The QoS parameter is provided by the PCF includes:

-	A QoS profile applied for a service as described in clause 5.7.1.2 of 3GPP TS 23.501 [x]

-	A time window associated to a QoS profile

NOTE X:	Time window is optional and is included by the PCF when analytics are requested for the PCF to determine optimal background transfer policy.

The Quality of Experience metric is provided by the PCF includes an indication that:

-	QoE is measured against a target Observed Service Experience for an application service, network slice for a UE or group of UEs

-	QoE is measured against a target DN performance

-	QoE is measured against a target a time period where a QoS profile applied is sustainable for a UE or group of UEs

NOTE Y:	The PCF can include a combination of QoE metrics

The PCF requesting QoS and Policy Assistance Analytics shall indicate in the request or subscription:

-	Analytics ID = "QoS and Policy Assistance Analytics";

Target of Analytics Reporting as defined in clause 6.1.3;

-	Analytics Filter Information

-	optional S-NSSAI;

-	optional Application identifier;

-	A list of QoS parameters;

-	The Quality of Experience metric or a combination of metrics to apply;

-	A target UE or Group of UEs;

-	optional area of interest;

-	optionally, maximum number of objects and maximum number of SUPIs;

-	optionally, preferred level of accuracy of the analytics;

-	optionally, preferred level of accuracy per analytics subset;

-	Analytics target period that indicates the time window for which the statistics or predictions are requested;

-	in a subscription, the Notification Correlation Id and the Notification Target Address;

##### 6.x.2	Input Data

For the purpose of QoS and Policy Assistance Analytics, the NWDAF may collect the information as listed in Table 6.x.2-1.

Table 6.x.2-1: Data collected by NWDAF for QoS and Policy Assistance Analytics

| Information | Source | Description |
| --- | --- | --- |
| Observed Service Experience related network data analytics | NWDAF NOTE X | Receive analytics of observerd service experience for a service (identified by Application ID or IP filter information) when a specific QoS parameter is applied by the PCF |
| DN Performance Analytics | NWDAF NOTE X | Receive analytics for DN performance for a service (identified by Application ID or IP filter information) when a specific QoS parameter is applied by the PCF |
| QoS sustainability analytics | NWDAF NOTE X | Receive analytics for QoS sustainability for a service (identified by Application ID or IP filter information) when a specific QoS parameters is applied by the PCF |
| NOTE x: If the NWDAF supports derivation of analytics listed in Table 6.x.2-1 then the operation is internal and out of scope of 3GPP. |  |  |

The NWDAF obtains QoS flow level network data related to the QoS profile assigned for a particular service (identified by Application ID or IP filter information) as per Table 6.4.2-2

### 6.x.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	QoS and Policy Assistance statistics information is defined in Table 6.x.3-1.

-	QoS and Policy Assistance predictions information is defined in Table 6.x.3-2.

Table 6.x.3-1: QoS and Policy Assistance statistics

| Information | Description |
| --- | --- |
| Quality of Experience (0..max) | List of Quality of Experience information for a service. |
| > QoE type | The QoE metric used according to the Analytics Filter information |
| > QoE metric | The variance of the QoE metric against the expected QoE target |
| > QoS Profile | The QoS Profile applied for a service |
| > Time Window | A time window where the QoS Profile is applied |
| > UE ID | Identifies the UE or Group of UEs |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > IP filter information | Identification of the service |
| > Spatial validity (NOTE X) | Area where the QoE analytics applies. |
| > Validity period | Validity period for the QoE analytics as defined in clause 6.1.3. |
| NOTE X: The Spatial validity is present in the output parameters if the consumer provided the Area of Interest as defined in Table 6.4.1-1. |  |

Table 6.x.3-2: QoS and Policy Assistance predictions

| Information | Description |
| --- | --- |
| Quality of Experience (0..max) | List of Quality of Experience information for a service. |
| > QoE type | The QoE metric used according to the Analytics Filter information |
| > QoE metric | The expected variance of the QoE metric against the expected QoE target |
| > QoS Profile | The QoS Profile applied for a service |
| > Time Window | An expected time window where the QoS Profile is applied |
| > UE ID | Identifies the UE or Group of UEs |
| > S-NSSAI | Identifies the Network Slice used to access the Application. |
| > Application ID | Identification of the Application. |
| > IP filter information | Identification of the service |
| > Spatial validity (NOTE X) | Area where the QoE analytics applies. |
| > Validity period | Validity period for the QoE analytics as defined in clause 6.1.3. |
| > Confidence | Confidence of this prediction. |
| NOTE X: The Spatial validity is present in the output parameters if the consumer provided the Area of Interest as defined in Table 6.4.1-1. |  |

### 6.x.4	Procedures to request QoS and Policy Assistance analytics

Figure 6.x.4-1: Procedure for NWDAF providing QoS and Policy Assistance analytics

This procedure allows the consumer to request Analytics ID "QoS and Policy Assistance" for a particular service identified by Application ID or IP filter information. The consumer includes as Analytics Filter information a candidate list of QoS Parameters and an expected Quality of Experience target.

1.	Consumer NF sends an Analytics request/subscribe (Analytics ID = QoS and Policy Assistance, Target of Analytics Reporting = UE ID or UE group ID, Analytics Filter Information as specified in clause 6.x.1. to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

2.	NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

3.	NWDAF sends Analytics request/subscribe based on the target QoE metric included in the Analytics Request in step 1 as described in Table 6.x.2.1.

NOTE Y:	Step 3 is ommited if the NWDAF supports derivation of analytics internally.

4. The NWDAF derives the Quality of Experience for each candidate QoS parameter.

5.	The NWDAF provides the analytics  to the consumer NF by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify. The NWDAF may provide one list of Quality of Experience including the QoS parameter that has the less variance from the expected Quality of Experience.

************************************ END OF CHANGES ****************************************