3GPP TSG-SA2 Meeting #164	S2-2407575

# 19 – 23 August 2024, Maastricht, Netherlands							(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1120 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI4: Abnormal NF Signalling Storm Detection Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-07-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR adds description to support analytics for signaliing storm detection due to abnormal NF behaviour according the agreed principles in clause 8.4 of 3GPP TR 23.700-84 |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed the NWDAF to derive new analytics (signalling storm detection) in order to satisfy the conclusions of Key Issue 4. The consumer of such analytics includes as analytic filters the following: - Target NF (which may be impacted due to signalling storm) - (Optionally) Source NF (the NF originating the signaling) - (Optionally) A scenario identifier: An identifier indicating the use case for abnormal signalling storm (e.g. SM signalling storm detection). Input data for determining signalling storm are based on Solutions 35 and 39 of TR 23.700-84 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions of KI4 not addressed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

## 6.x	Procedure for Abnormal NF Signalling Storm Detection Analytics

##### 6.x.1	General

The NWDAF provides Abnormal NF Signalling Storm Detection Analytics to a consumer.

The Abnormal NF Signalling Storm Detection Analytics identifies Network Function that contributed to increased control plane signalling (above a certain threshold) within the 3GPP core network.

A consumer requesting Abnormal NF Signalling Storm Detection Analytics shall indicate in the request or subscription:

-	Analytics ID = " Abnormal NF Signalling Storm Detection Analytics ";

Target of Analytics Reporting as defined in clause 6.1.3;

-	Analytics Filter Information

-	optional S-NSSAI;

-	Target NF ID(s) identifying the NF(s) that may be impacted due to signalling storm

-	(Optionally) Source NF ID(s) identifying the NF(s) originating massive signalling storm

-	(Optionally) A scenario identifier: An identifier indicating the use case for abnormal signalling storm (e.g. SM signalling storm detection).

-	optionally, preferred level of accuracy of the analytics;

-	optionally, preferred level of accuracy per analytics subset;

-	Analytics target period that indicates the time window for which the statistics or predictions are requested;

-	in a subscription, the Notification Correlation Id and the Notification Target Address;

##### 6.x.2	Input Data

For the purpose of Abnormal NF Signalling Storm Detection the NWDAF may collect the information as listed in Table 6.x.2-1.

Table 6.x.2-1: Data collected by NWDAF for Abnormal NF Signalling Storm Detection

| Information | Source | Description |
| --- | --- | --- |
| Signalling exchange information | Any NF, SCP, OAM NOTE X, NOTE Y | Includes number of received signalling information, transmitted signalling information, the type of signalling, indication whether response to received signalling was succesful or not. |
| Analytics for fault management predictions/statistics. | MDAF/MDAS NOTE Z | Receive analytics for fault management predictions/statistics for the NF(s) identified in the Analytics Filter |
| NOTE X: The NWDAF requests input data from NF(s) identified in the Analytics Filter Information and/or based on the scenario identifier if included. NOTE Y. The NWDAF requests Performance Measurements (PM) as per TS 28.552 and Key Performance Indicators (KPIs) as per TS 28.554 from NF(s) identified in the Analytics Filter information and/or based on the scenario identifier if included NOTE Z.: The NWDAF collects input data from MDAS according to clause 6.2.14 |  |  |

### 6.x.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Abnormal NF Signalling Storm Detection statistics information is defined in Table 6.x.3-1.

-	Abnormal NF Signalling Storm Detection predictions information is defined in Table 6.x.3-2.

Table 6.x.3-1: Abnormal NF Signalling Storm Detection statistics

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm. |
| > Target NF ID | The target of signalling storm detected by NWDAF. |
| > Abnormality ID | The potential cause of NF Abnormality (e.g. massive UE/frequent access, NF abnormal signalling, etc.). |
| > Source NF list | NF(s) which cause the signalling storm. |

Table 6.x.3-2: Abnormal NF Signalling Storm Detection predictions

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm. |
| > Target NF ID | The target of signalling storm predicted by NWDAF. |
| > Abnormality ID | The potential cause of NF Abnormality (e.g. NF abnormal signalling, etc.). |
| > Source NF list | Potential NF(s) which cause the signalling storm. |
| > Confidence | Confidence of this prediction. |

### 6.x.4	Procedures to request Abnormal NF Signalling Storm Detection analytics

Figure 6.x.4-1: Procedure for NWDAF providing Abnormal NF Signalling Storm Detection analytics

1.	Consumer NF sends an Analytics request/subscribe (Analytics ID = Abnormal NF Signalling Storm Detection, Analytics Filter Information as specified in clause 6.x.1) to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

2.	NWDAF obtains performance data from OAM and Fault Predictions Analytics for the identified NF(s) based on the Analytics Filter Information and/or Scenario Identifier.

3.	NWDAF subscribes the network data from 5GC NF(s) in the Table 6.x.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

4. The NWDAF derives analytics by determining expected signalling load based on data collected from OAM and monitors the signalling load of NF(s) included in Analytics Filter information. The NWDAF may use Fault Prediction analytics to determine if signalling load considerably increases and an NF outage occurs.

5.	The NWDAF provides the analytics  to the consumer NF by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify..

************************************ END OF CHANGES ****************************************