3GPP TSG-WG SA2 Meeting #164	S2-2407581

Maastricht, Netherlands, 19 - 23 August, 2024

Source:	Lenovo

Title:	KI#2 Conclusions

Document for:	Approval

Agenda Item:	19.3.1

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: Provides evaluation of Solutions for KI#2 and the associated conclusion

# 1. Introduction

Proposes a way forward for conclusions to Key Issue 2.

# 2. Text Proposal

It is proposed to introduce the following changes vs. TR 23.700-70.

# * * * * First change. All new text * * * *

## 8.2	Conclusions for Key Issue #2

Proxying-UDP-in-HTTP solutions (solutions 26 and 24) shall be adopted for the normative work as follows.

1.	An AF when requesting an AF session towards the 3GPP network (NEF) includes additional information indicating to the 3GPP network that the UPF can identify XRM metadata (that may include PDU set information but also data defined in colclusions to KI 4 and KI5) by inspecting the headers of an encapsulation protocol. The AF also includes the address of the XRM application server where the UPF will need to establish a connection using an encapsulation protocol indicated in the AF request . The encapsulation protocol may be Connect-UDP as per IETF RFC 9298 [38] or QUIC-Aware proxying using HTTP as per [40].

2.	The PCF in the 3GPP network providies PCC rules where the PCC rules includes the information provided by the AF in step 1.

3.	The SMF constructs N4 rules based on the PCC rules where the N4 rules include uplink and downlink Packet Detection Rules indicating to the UPF that:

a.	for uplink packets sent from a specific UE (or any UE) to a specific application server the UPF would need to establish a connection using an encapsulation protocol and route the packet via the encapsulation protocol to the Application Server address. QUIC packets with a short header can be forwarded as per QUIC-Aware proxying using HTTP [40].

b.	For downlink packets received by the UPF over N6, N4 configuration information indicates the UPF to:

i.	Extract the encapsulated ciphered UDP packet from the received IP packet. QUIC packets sent in forwarded mode can be retrieved by removing the metadata with a packet transform.

ii.	Retrieve XRM metadata (including PDU set identification) from information contained within the encapsulation protocol.

iii.	Routing the decapsulated ciphered UDP packet (without the XRM metadata) over a QoS flow within GTP signalling towards the RAN.

4.	UPF that supports an encapsulation protocol client establishing a protocol session with a server once the UPF detects that an uplink packet is sent towards a specific destination address. To activate the forwarding mode, UPF extracts the cleartext CIDs from the first long header packets seen from the endpoints and sends them in REGISTER_CLIENT_CID and REGISTER_TARGET_CID messages towards the AS proxy, as per QUIC-Aware proxying using HTTP [40].

NOTE X:	If the UPF determines that there is already a tunnel open towards the XR application server (e.g. from another UE XR session), the UPF can forward the UE packet via the existing connection. The decision is based on UPF implementations or operator configuration, although it may affect load balancing implementation at the Application Server.

5.	The Application Server sends downlink packets via the encapsulation protocol and includes additional XRM metadata provided in-band within the encapsulation protocol to assist the UPF to identify XRM metadata.

-	As per Solution 26, if the end-to-end connection between the UE and the Application Server is based on QUIC, to avoid re-encapsulation and re-encryption, all XRM payload packets carried on QUIC packets with short header after registration shall be forwarded using the Forwarded Mode in QUIC-Aware Proxying using HTTP [40]. The XRM metadata is encoded into the XRM packets by use of a packet transform that is applied to proxied QUIC packets. The XRM metadata shall be encrypted and integrity protected, using a shared key that is exchanged during the negotiation of the transform, and an initialization vector that is obtained from the end-to-end encrypted QUIC packet. Further details of the encoding is to be specified in stage 3.

-	Otherwise the XRM payload packets are forwarded using Connect-UDP [38] as per Solution 24. The XRM metadata is encoded into the XRM payload packets in a HTTP Datagram. Further details of the HTTP Datagram encoding is to be specified in stage 3.

6.	The UPF identifies XRM metadata of PDUs of PDU sets based on the N4 rules and the information provided within the encapsulation protocol and routes the packet via a QoS flow including the XRM metadata for each PDU within the GTP-U header.

NOTE Y:	If not released by the Application Server, it is up to UPF implementation when the tunnelled connection is released.

7.	These solutions have no UE impacts, since the UDP connection is established between the UPF and the AS proxy.

HTTP datagram formats do not need IETF standardization. If HTTP header fields are used to indicate support for the specific format, the names of such fields need to be registered with IANA. The specific datagram format will be specified by CT working groups.

New packet transforms do not require IETF standardization, but the name of the packet transform must be registered with IANA. The specific packet transform will be specified by CT working groups.

Media Over QUIC solutions based on [9] shall be adopted for normative work as follows:

1.	AF sends Encrypted Traffic Handling Assistance Information (ETHAI) along with the Protocol Description to the NEF/PCF. The ETHAI includes the MoQ relay support indication along with one or more URI of AS, AS address.

2.	During PDU session Establishment procedure, SMF selects UPF supporting MoQ relay functionality based on UPF capability and other information (e.g. specific (DNN, S-NSSAI)

3.	SMF initiates an N4 Session Establishment/Modification procedure with the UPF, and sends the AS address received from PCF to UPF to trigger the establishment of QUIC connection with AS. UPF triggers the QUIC connection with the AS with using the AS address (e.g. URI).

4.	The UE obtains the MoQ relay address (i.e. UPF address) via application layer. Then the UE initiates the QUIC connection establishment with the UPF. How the application is aware of the UPF address is going to be defined during normative phase.

5.	For the downlink direction, the PSA UPF identifies PDU Set information from MoQ metadata, The UPF identifies the MoQ metadata and convert the MoQ metadata into PDU set information and sends the PDU Set information in the GTP-U header to RAN. The PDU Set Information can be extracted from the metadata as following:

-	The PDU Set Sequence Number could be extracted from Object Sequence.

-	Indication of End PDU of the PDU Set could be identified based on the Payload Length.

-	PDU Set Size in bytes could be identified based on Object Payload Length.

-	PDU Set Importance could be identified based on Object Send Order.

# * * * * End of changes * * * *