SA WG2 Meeting #164	S2-2407595

# Maastricht, Netherlands, 19 – 23 August, 2024                  (revision of S2-240xxxx)

Source: 	CATT

Title: 	Discussion and conclusion proposals for Topology 1 architecture

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper discussed topology 1 architecture options and made some conclusion proposals.

1	Discussion

Several solutions are captured in TR 23.700-13 to address to the Topology 1 architecture and different architecture options are proposed. This paper discussed the architecture options and tried to make some way-forward proposals.

1) AIoT Functionality

In order to support AIoT Services, new AIoT Functionality needs to be supported by the architecture. Basically there are two options: one option is to introduce a new AIoT Function entity (e.g. Sol#8), and the other option is to add AIoT Functionality into the AMF (e.g. Sol#5).

Consider the Topology 1 deployment scenarios and to have a clear architecture, it is proposed to introduce a new AIoT Function entity to support AIoT Services.

Proposal 1: It is proposed to introduce a new AIoT Function (AIoTF) entity to support AIoT Services.

2) Interface between AIoTF and A-RAN

Assume a new AIoT Function (AIoTF) is introduced as proposed in Proposal 1, the interface between AIoTF and A-RAN needs to be decided. In general, there are two options: one is AIoTF interacts with A-RAN directly (Option A), and the other is AIoTF interacts with A-RAN via AMF (Option B).

Option A                                               Option B

For Option A, AIoTF interacts with A-RAN directly via a new defined Nxx interface.

For Option B, AIoTF interacts with A-RAN via AMF (transparent or not).

From SA2 point of view both options are feasible and it is worth to be noted that architecture options are also under discussion in RAN3, and the Application Protocol between A-RAN and AIoTF is to be defined by RAN3. It is proposed to cooperate with RAN3 to make the final decision.

Proposal 2: It is proposed to cooperate with RAN3 to decide the interface between A-RAN and AIoTF.

3) Interface between AIoTF and AIoT Device

As RAN2 has assumed no AS security is support between AIoT Device and A-RAN, it is necessary to support security mechanism at NAS layer to fulfil the SA1 requirements and to make 3GPP defined AIoT solution as competitive to RFID solution. Consider the nature of the AIoT Devices (e.g. ultra-low complexity power, cost and resource-constrained), the AIoT NAS message is a light weight NAS message.

Proposal 3: It is proposed AIoTF interacts with AIoT Device via AIoT NAS message.

2	Proposal

It is proposed to update TR 23.700-13 as follows.

# * * * * First change * * * *

8.1	Conclusions for Key Issue #1: Architecture support of Ambient IoT Devices

For Key Issue #1, the following principles are concluded:

-	A new AIoT Function (AIoTF) entity is introduced to support AIoT Services.

-	AIoTF may interact with A-RAN directly via a new defined Nxx interface or AIoTF may interact with A-RAN via AMF (transparent or not).

NOTE:	Cooperation with RAN3 is required to decide the interface between A-RAN and AIoTF.

-	AIoTF interacts with AIoT Device via AIoT NAS message.

# * * * * End of Changes * * * *