3GPP TSG-SA WG2 Meeting #164	S2-2407600

# Maastricht, Netherlands, 19 – 23 August, 2024						  (revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0446 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for 5G ProSe multi-hop UE-to-Network Relay Discovery with Model B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 23.700-03 clause 8, it is concluded to support multi-hop UE-to-Network Relay in Rel-19, and procedure for 5G ProSe multi-hop UE-to-Network Relay Discovery with Model B needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for 5G ProSe multi-hop UE-to-Network Relay Discovery with Model B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe multi-hop UE-to-Network Relay Discovery with Model B is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.3.3a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

6.3.2.3.3a	Procedure for 5G ProSe multi-hop UE-to-Network Relay Discovery with Model B

Depicted in Figure 6.3.2.3.3a-1 is the procedure for 5G ProSe multi-hop UE-to-Network Relay Discovery with Model B.

Figure 6.3.2.3.3a-1: 5G ProSe multi-hop UE-to-Network Relay Discovery with Model B

1.	The 5G ProSe multi-hop Remote UE sends a 5G ProSe UE-to-Network Relay Discovery Solicitation message. The 5G ProSe UE-to-Network Discovery Solicitation message contains the Type of Discovery Message, User Info ID of 5G ProSe multi-hop Remote UE, RSC, optionally User Info ID of 5G ProSe multi-hop UE-to-Network Relay, multi-hop indication per RSC and hop count per RSC. The hop count value is set to be the (pre)configured maximum number of hops per RSC. The multi-hop indication indicates the forwarding of Discovery message is supported.

The Solicitation message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.3.

2.	If the RSC contained in the received Solicitation message matches any of the (pre)configured RSC(s) of a 5G ProSe Intermediate Relay, the multi-hop indication associated with the RSC is included in the Solicitation message, and the hop count value is larger than one, then the Intermediate Relay decreases the hop count by one and includes the User Info ID of the Intermediate Relay and the updated hop count value in the Solicitation message and forwards the Solicitation message. If the Solicitation message is forwarded by multiple 5G ProSe Intermediate Relays, an ordered list of User Info IDs of the Intermediate Relay is included in the Solicitation message.

The Solicitation message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.3.

3.	If the RSC contained in the Solicitation message matches any of the (pre)configured RSC(s) of a 5G ProSe multi-hop UE-to-Network Relay, and the User Info ID of 5G ProSe multi-hop UE-to-Network Relay contained in the Solicitation message, if any, matches the 5G ProSe multi-hop UE-to-Network Relay, the 5G ProSe multi-hop UE-to-Network Relays respond to the 5G ProSe Intermediate UE with a UE-to-Network Relay Discovery Response message. The 5G ProSe UE-to-Network Relay Discovery Response message contains the Type of Discovery Message, RSC, User Info ID of 5G ProSe multi-hop Remote UE, User Info ID of 5G ProSe multi-hop UE-to-Network Relay and reversed ordered list of User Info IDs of 5G ProSe Intermediate Relay.

The Response message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.3.

4.	The Response message is forwarded back to the 5G ProSe multi-hop Remote UE by the 5G ProSe Intermediate Relays based on the list of User Info IDs of 5G ProSe Intermediate Relay.

The Response message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.3.

* * * * End of Changes * * * *