3GPP TSG-SA WG2 Meeting #164	S2-2407601

# Maastricht, Netherlands, 19 – 23 August, 2024						  (revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0447 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery with PDU type Ethernet and Unstructured |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 23.700-03 clause 8, it is concluded to support Layer-3 multi-hop UE-to-UE Relay in Rel-19, and procedure for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery with PDU type Ethernet and Unstructured needs to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery with PDU type Ethernet and Unstructured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery with PDU type Ethernet and Unstructured is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.2a (new), 6.3.2.4.3a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

6.3.2.4.2a	Procedure for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery with PDU type Ethernet and Unstructured with Model A

Depicted in Figure 6.3.2.4.2a-1 is the procedure for 5G ProSe Layer-3 multi-hop UE-to-UE Discovery with PDU type Ethernet and Unstructured with Model A.

Figure 6.3.2.4.2a-1: 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery with PDU type Ethernet and Unstructured with Model A

1.	The 5G ProSe Layer-3 multi-hop UE-to-UE Relay-1 has discovered other UEs in proximity and obtains the Direct discovery set from other UEs in proximity per RSC.

2.	The 5G ProSe Layer-3 multi-hop UE-to-UE Relay-1 sends a UE-to-UE Relay Discovery Announcement message. The UE-to-UE Relay Discovery Announcement message contains the Type of Discovery Message, User Info ID of the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-1, RSC and Direct discovery set including list of protected user info (i.e. Application Layer ID) received from the 5G ProSe Layer-3 multi-hop End UEs supporting the RSC, multi-hop indication and hop count. The hop count value is set to be the (pre)configured maximum number of hops associated with the RSC. The multi-hop indication indicates the forwarding of Direct discovery set associated with the RSC is supported.

The UE-to-UE Relay Discovery Announcement message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

The 5G ProSe Layer-3 multi-hop UE-to-UE Relay-1 shall only announce user info (i.e. Application Layer ID) of other UEs in proximity that did not include an Announce Prohibited Indication when they were previously discovered.

NOTE:	5G ProSe Layer-3 multi-hop UE-to-UE Relay announces Direct discovery set from other UEs in proximity only if their PC5 signal strength measured by the 5G ProSe Layer-3 multi-hop UE-to-UE Relay is above configured signal strength threshold as specified in TS 38.331 [16].

3.	If the RSC contained in the received Announcement message matches any of the (pre)configured RSC(s) of the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-2, the multi-hop indication associated with the RSC is included in the Announcement message, and the hop count value is larger than one, then the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-2 decreases the hop count value by one and forwards Announcement message by including the User Info ID of the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-2 in the Announcement message.

A 5G ProSe Layer-3 multi-hop End UE monitors announcement messages from a 5G ProSe Layer-3 multi-hop UE-to-UE Relay. The 5G ProSe Layer-3 multi-hop End UEs determine the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

* * * * 2nd Change * * * *

6.3.2.4.3a	Procedure for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery with PDU type Ethernet and Unstructured with Model B

Depicted in Figure 6.3.2.4.3a-1 is the procedure for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery with PDU type Ethernet and Unstructured with Model B.

Figure 6.3.2.4.3a-1: 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery with PDU type Ethernet and Unstructured with Model B

1.	The discoverer 5G ProSe Layer-3 multi-hop End UE-1 sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the Type of Discovery Message, RSC, the Direct Discovery set which includes the protected user info (i.e. Application Layer ID) of the discoverer 5G ProSe Layer-3 multi-hop End UE-1 and the discoveree 5G ProSe Layer-3 multi-hop End UE-2, multi-hop indication and hop count. The hop count value is set to be the (pre)configured maximum number of hops associated with the RSC. The multi-hop indication indicates the forwarding of Direct discovery set associated with the RSC is supported.

The 5G ProSe UE-to-UE Relay Discovery Solicitation message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

A 5G ProSe Layer-3 multi-hop UE-to-UE Relay determines the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

The discoverer 5G ProSe Layer-3 multi-hop End UE may include an Announce Prohibited Indication in the UE-to-UE Relay Discovery Solicitation message. If a 5G ProSe Layer-3 multi-hop UE-to-UE Relay receives a Relay Discovery Solicitation message with an Announce Prohibited Indication it does not consider the 5G ProSe Layer-3 multi-hop End UE as discovered during this procedure for inclusion in 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery with Model A, see clause 6.3.2.4.2a, step 1.

2.	If the RSC contained in the received Solicitation message matches any of the (pre)configured RSC(s) of the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-1, the multi-hop indication associated with the RSC is included in the Solicitation message, and the hop count value is larger than one, then the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-1 decreases the hop count value by one and forwards Solicitation message by including the User Info ID of the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-1 in the Solicitation message.

3.	If the RSC contained in the received Solicitation message matches any of the (pre)configured RSC(s) of the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-2, the multi-hop indication associated with the RSC is included in the Solicitation message, and the hop count value is larger than one, then the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-2 decreases the hop count value by one and forwards Solicitation message by including the User Info ID of the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-2 in the Solicitation message.

The 5G ProSe UE-to-UE Relay Discovery Solicitation message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

4.	If the RSC contained in the received Solicitation message matches any of the (pre)configured RSC(s) of the discoveree 5G ProSe Layer-3 multi-hop End UE-2, and the discoveree 5G ProSe Layer-3 multi-hop End UE-2 matches the user info (i.e. Application Layer ID) of the discoveree 5G ProSe Layer-3 multi-hop End UE-2 contained in the Solicitation message, then the discoveree 5G ProSe Layer-3 multi-hop End UE-2 responds to the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-2 with a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, RSC, the Direct Discovery set which includes the protected user info (i.e. Application Layer ID) of the discoverer 5G ProSe Layer-3 multi-hop End UE-1 and the discoveree 5G ProSe Layer-3 multi-hop End UE-2, the User Info ID of the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-2 and the User Info ID of the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-1.

The 5G ProSe UE-to-UE Relay Discovery Response message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

If the discoveree 5G ProSe Layer-3 multi-hop End UE-2 receives multiple UE-to-UE Relay Discovery Solicitation messages from different 5G ProSe Layer-3 multi-hop UE-to-UE Relays with the same RSC and the user info (i.e. Application Layer ID) of the discoveree 5G ProSe Layer-3 multi-hop End UE-2, it may choose to respond or not to a 5G ProSe Layer-3 multi-hop UE-to-UE Relay (e.g. based on the PC5 signal strength of each message received).

The discoveree 5G ProSe Layer-3 multi-hop End UE-2 may include an Announce Prohibited Indication in the UE-to-UE Relay Discovery Response message. If a 5G ProSe Layer-3 multi-hop UE-to-UE Relay receives a Relay Discovery Response message with an Announce Prohibited Indication it does not consider the 5G ProSe Layer-3 multi-hop End UE as discovered during this procedure for inclusion in 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery with Model A, see clause 6.3.2.4.2a, step 1.

5.	The 5G ProSe Layer-3 multi-hop UE-to-UE Relay-2 sends a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, User Info ID of the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-2 and the User Info ID of the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-1, RSC, the Direct Discovery set which includes the list of protected user info (i.e. Application Layer ID) of the discoverer 5G ProSe Layer-3 multi-hop End UE-1 and the discoveree 5G ProSe Layer-3 multi-hop End UE-2.

The 5G ProSe UE-to-UE Relay Discovery Response message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

6.	The 5G ProSe Layer-3 multi-hop UE-to-UE Relay-1 sends a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, User Info ID of the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-2 and the User Info ID of the 5G ProSe Layer-3 multi-hop UE-to-UE Relay-1, RSC, the Direct Discovery set which includes the list of protected user info (i.e. Application Layer ID) of the discoverer 5G ProSe Layer-3 multi-hop End UE-1 and the discoveree 5G ProSe Layer-3 multi-hop End UE-2.

The 5G ProSe UE-to-UE Relay Discovery Response message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

* * * * End of Changes * * * *