3GPP TSG-SA WG2 Meeting #164	S2-2407602

# Maastricht, Netherlands, 19 – 23 August, 2024						  (revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0448 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Functional Entities enhancements for 5G ProSe multi-hop Relays |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 23.700-03 clause 8, it is concluded to support both multi-hop UE-to-Network Relay and Layer-3 multi-hop UE-to-UE Relay in Rel-19, and enhancements for Functional Entities for multi-hop UE-to-Network Relay and Layer-3 multi-hop UE-to-UE Relay need to be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Functional Entities enhancements for multi-hop UE-to-Network Relay and Layer-3 multi-hop UE-to-UE Relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The multi-hop UE-to-Network Relay and Layer-3 multi-hop UE-to-UE Relay are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1, 4.3.3, 4.3.4, 4.3.5, 4.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

4.3.1	UE

Any 5G ProSe-enabled UE may support the following functions:

-	Exchange of information for 5G ProSe Direct Discovery between 5G ProSe-enabled UE and the 5G DDNMF over PC3a reference point.

-	Procedures for 5G ProSe Direct Discovery of other 5G ProSe-enabled UEs over PC5 reference point.

-	Procedures for 5G ProSe Direct Communication over PC5 reference point, including Broadcast, Groupcast and Unicast mode 5G ProSe Direct Communication.

-	Procedures to act as a 5G ProSe Layer-2 UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-3 UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-2 Remote UE.

-	Procedures to act as a 5G ProSe Layer-3 Remote UE.

-	Procedures to act as a 5G ProSe Layer-2 UE-to-UE Relay.

-	Procedures to act as a 5G ProSe Layer-3 UE-to-UE Relay.

-	Procedures to act as a 5G ProSe Layer-2 End UE.

-	Procedures to act as a 5G ProSe Layer-3 End UE.

-	Procedures to act as a 5G ProSe Layer-2 multi-hop UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-2 multi-hop Remote UE.

-	Procedures to act as a 5G ProSe Layer-3 multi-hop Remote UE.

-	Procedures to act as a 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

-	Procedures to act as a 5G ProSe Layer-3 multi-hop End UE.

-	Procedures for communication path switching between PC5 and Uu reference points.

-	Procedures for Multi-path communication via Uu and via 5G ProSe UE-to-Network Relay.

-	Indicating UE Policy Provisioning Request in UE Policy Container for UE triggered 5G ProSe Policy provisioning, which requests one or multiple types of policies/parameters as listed below:

-	Policy/parameters for 5G ProSe Direct Discovery;

-	Policy/parameters for 5G ProSe Direct Communication;

-	Policy/parameters for 5G ProSe Layer-2 Remote UE;

-	Policy/parameters for 5G ProSe Layer-3 Remote UE;

-	Policy/parameters for 5G ProSe Layer-2 UE-to-Network Relay;

-	Policy/parameters for 5G ProSe Layer-3 UE-to-Network Relay;

-	Policy/parameters for 5G ProSe Layer-2 End UE;

-	Policy/parameters for 5G ProSe Layer-3 End UE;

-	Policy/parameters for 5G ProSe Layer-2 UE-to-UE Relay;

-	Policy/parameters for 5G ProSe Layer-3 UE-to-UE Relay;

-	Policy/parameters for 5G ProSe Layer-2 multi-hop Remote UE;

-	Policy/parameters for 5G ProSe Layer-3 multi-hop Remote UE;

-	Policy/parameters for 5G ProSe Layer-2 multi-hop UE-to-Network Relay;

-	Policy/parameters for 5G ProSe Layer-3 multi-hop UE-to-Network Relay;

-	Policy/parameters for 5G ProSe Layer-3 multi-hop End UE;

-	Policy/parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay.

-	Receiving the 5G ProSe Policy from 5GC over N1 reference point.

-	Configuration of parameters for 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay (e.g. including IP addresses, ProSe Layer-2 Group IDs, see clause 5.1), 5G ProSe UE-to-UE Relay, 5G ProSe multi-hop UE-to-Network Relay and 5G ProSe Layer-3 multi-hop UE-to-UE Relay. These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over PC1 reference point from the ProSe Application Server.

-	Reporting the following capabilities to 5GC over the N1 reference point:

-	5G ProSe Capability.

* * * * 2nd Change * * * *

4.3.3	PCF

In addition to the functions defined in TS 23.501 [4] and TS 23.503 [9], the PCF includes functions to provision the UE with necessary policies and parameters to use 5G ProSe services, as part of the UE ProSe Policy information as defined in TS 23.503 [9] clause 4.2.2:

-	PC5 usage reporting configuration.

-	Authorization policy and parameters for 5G ProSe Direct Discovery and Communication.

-	Authorization policy and parameters for 5G ProSe UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 Remote UE, as 5G ProSe Layer-3 Remote UE, as 5G ProSe Layer-2 UE-to-Network Relay, as 5G ProSe Layer-3 UE-to-Network Relay).

-	PDUID allocation with its validity timer.

-	Authorization policy and parameters for 5G ProSe UE-to-UE Relay Discovery and Communication.

-	Authorization policy and parameters for 5G ProSe multi-hop UE-to-Network Relay Discovery and Communication.

-	Authorization policy and parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery and Communication.

The PCF may update the 5G ProSe policy and parameters to the UE under certain conditions.

When receiving the 5G ProSe Capability in Npcf_UEPolicyControl_Create Request from the AMF or when receiving the updated subscription data from UDR, the PCF generates the PC5 QoS parameters used by NG-RAN corresponding to a UE as defined in clause 5.4.2 of TS 23.287 [2].

* * * * 3rd Change * * * *

4.3.4	AMF

In addition to the functions defined in TS 23.501 [4], the AMF performs the following functions:

-	Select a PCF supporting 5G ProSe Policy/Parameter provisioning based on indication of 5G ProSe Capability as part of the "5GMM capability" in the Registration Request.

-	Store the 5G ProSe Capability.

-	Forward the 5G ProSe Capability to PCF in Npcf_UEPolicyControl_Create Request.

-	Obtain from UDM the subscription information related to 5G ProSe and store them as part of the UE context data.

-	Obtain PC5 QoS parameters from the PCF and store them as part of the UE context data.

-	Provision the NG-RAN with indication about the UE authorization status about the following:

-	5G ProSe Direct Discovery and 5G ProSe Direct Communication (i.e. as 5G ProSe-enabled UE for ProSe Direct Discovery, as 5G ProSe-enabled UE for ProSe Direct Communication);

-	5G ProSe UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 Remote UE, as 5G ProSe Layer-2 UE-to-Network Relay, as 5G ProSe Layer-3 UE-to-Network Relay);

-	Multi-path communication via direct Uu path and via 5G ProSe Layer 2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE;

-	5G ProSe UE-to-UE Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 End UE, as 5G ProSe Layer-2 UE-to-UE Relay);

-	5G ProSe multi-hop UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 multi-hop Remote UE, as 5G ProSe Layer-2 multi-hop UE-to-Network Relay, as 5G ProSe Layer-3 multi-hop UE-to-Network Relay).

-	Provision the NG-RAN with PC5 QoS parameters related to 5G ProSe Direct Communication.

-	Optionally support security procedures over Control Plane for 5G ProSe UE-to-Network relaying as defined in TS 33.503 [29].

* * * * 4th Change * * * *

4.3.5	UDM

In addition to the functions defined in TS 23.501 [4], the UDM performs the following functions:

-	Subscription management for 5G ProSe Direct Discovery and Communication.

-	Subscription management for 5G ProSe UE-to-Network Relay Discovery and Communication.

-	Subscription management for multi-path communication via direct Uu path and via 5G ProSe Layer 2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE.

-	Subscription management for 5G ProSe UE-to-UE Relay Discovery and Communication.

-	Subscription management for 5G ProSe multi-hop UE-to-Network Relay Discovery and Communication.

-	Subscription management for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery and Communication.

* * * * 5th Change * * * *

4.3.8	ProSe Application Server

The ProSe Application Server supports the following functionalities.

For 5G ProSe Direct Discovery:

-	Maintains permission information for the restricted 5G ProSe Direct Discovery using RPAUIDs;

-	Storage of ProSe Discovery UE IDs and metadata;

-	Mapping of RPAUID and PDUID for restricted 5G ProSe Direct Discovery;

-	Provisioning parameters for Group Member Discovery to UE.

-	Interaction with 5G DDNMF for 5G ProSe Direct Discovery, including:

-	Allocation of the ProSe Restricted Code Suffix pool, if restricted Direct Discovery with application-controlled extension is used;

-	Allocation of the mask(s) for ProSe Restricted Code Suffix, if restricted Direct Discovery with application-controlled extension is used;

-	Allocation of the ProSe Application Code Suffix pool, if open Direct Discovery with application-controlled extension is used;

-	Allocation of the mask(s) for ProSe Application Code Suffix, if open Direct Discovery with application-controlled extension is used.

For 5G ProSe Direct Communication:

-	Provisioning a path preference for 5G ProSe Services to UDR;

-	Provisioning parameters for 5G ProSe Direct Communication to UE.

For 5G ProSe UE-to-Network Relay service:

-	Provisioning parameters for 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-Network Relay Communication to UDR.

For 5G ProSe UE-to-UE Relay service:

-	Provisioning parameters for 5G ProSe UE-to-UE Relay Discovery and 5G ProSe UE-to-UE Relay Communication to UDR.

For 5G ProSe multi-hop UE-to-Network Relay service:

-	Provisioning parameters for 5G ProSe multi-hop UE-to-Network Relay Discovery and 5G ProSe multi-hop UE-to-Network Relay Communication to UDR.

For 5G ProSe Layer-3 multi-hop UE-to-UE Relay service:

-	Provisioning parameters for 5G ProSe Layer-3 multi-hop UE-to-UE Relay Discovery and 5G ProSe Layer-3 multi-hop UE-to-UE Relay Communication to UDR.

* * * * End of changes * * * *