3GPP TSG-WG SA2 Meeting #164	S2-2407623

# Maastricht, Netherlands, 19 August – 23 August, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1315 | rev | ---- | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Awareness of URSP Rule Enforcement – 23.503 mirror CR of R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle, Verizon UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | URSP rule enforcement report may be included in the PDU SESSION ESTABLISHMENT REQUEST sent by the UE (Use case: based on URSP rule matching, UE creates a new PDU session to carry the traffic of a new app, and based on the URSP rule indication, includes a URSP Rule Enforcement report in the PDU session establishment request). In that case the SMF should unconditionally provide it to the SM-PCF as part of the SM Policy Association Establishment request. Below are three references from CT1 and CT3 that capture this functionality. - 24.501 6.4.1.2 “If the UE supports reporting of URSP rule enforcement and is indicated to send URSP rule enforcement report to network based on the matching URSP rule which contains the URSP rule enforcement report indication set to "URSP rule enforcement report is required", the UE shall include the URSP rule enforcement reports IE in the PDU SESSION ESTABLISHMENT REQUEST message.” - 23.502 5.2.5.4.2 Npcf_SMPolicyControl_Create service operation Inputs, Optional: … URSP rule enforcement that including Connection Capability,… - 29.512 4.2.2.2 “The NF service consumer shall include (if available) in the "SmPolicyContextData" data structure: …………………… when the "URSPEnforcement" feature is supported, the URSP rule enforcement information provided by the UE within the "urspEnfInfo" attribute.” - 29.512 5.6.2.3 SmPolicyContextData includes urspEnfInfo attribute (Data Type = UrspEnforcementInfo); i.e. URSP rule enforcement report is provided to the PCF over N7 in the creation of the SM policy association, if available at the SMF. - 29.514 4.2.6.14 Subscription to notifications about URSP rule enforcement information “…If URSP rule enforcement information corresponding to the subscription is available, the PCF shall include the received URSP rule enforcement information within the "urspEnfRep" attribute” //i.e the URSP rule enforcement report can be provided to the PCF of the UE (by the PCF for the PDU Session) in the response to the subscription request. URSP rule enforcement report may alternatively sent by the UE as part of PDU SESSION MODIFICATION REQUEST(Use case: based on URSP rule matching, UE was able to use an existing PDU session to carry the traffic of a new app), and should be forwarded by SMF to SM-PCF in an update request, conditioned by seeing an early corresponding PCRT. Currently, the text in 23.503 clause 6.6.2.4 only covers the 2nd case (URSP rule enforcement report included in PDU SESSION MODIFICATION REQUEST). This has to be fixed along the lines of TS 29.512 that has it correctly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to align the text in 23.503 6.6.2.4 with the text in 29.512 4.2.2.2 and the text in 29.514 4.2.6.14. I.e., the case, when the PCF receives the URSP rule enforcement report for a given UE as part of the SM policy association establishment, is be added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. Misalignment between 23.503 and 29.512/29.514. 2. Text will be misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 6.6.2.4	Support of URSP rule enforcement reporting

The UE may report URSP rule enforcement to PCF, so that PCF will be made aware about the URSP rule enforcement when a given UE enforces specific URSP rule(s) and the PCF may trigger an action upon the reception of such reporting.

In order to activate the URSP rule enforcement reporting for a URSP rule (containing Connection Capabilities in the Traffic descriptor, see clause 6.6.2.1), for a UE indicating the capability of reporting URSP rule enforcement to network (see clause 4.2.2.2.2 of TS 23.502 [3]), the PCF sets the Indication for reporting URSP rule enforcement in a URSP rule sent to the UE (see clause 6.6.2.1).

NOTE 1:	The format and values of the Traffic descriptor component type identifier are defined in clause 5.2 of TS 24.526 [19].

A UE supporting URSP rule enforcement reporting shall send a URSP rule enforcement report to the SMF for a URSP rule which includes an Indication for reporting URSP rule enforcement as well as Connection Capabilities in the Traffic descriptor (see clause 6.6.2.1). The UE shall send a URSP rule enforcement report, i.e. all Connection Capabilities contained in the Traffic descriptor of the associated URSP rule, to the SMF when:

-	the UE associates a newly detected application to a new PDU Session (based on URSP evaluation result (see clause 6.6.2.3) for such a URSP rule), by including the URSP rule enforcement report in the PDU Session Establishment Request (see clause 4.3.2.2.1 of TS 23.502 [3]); or

-	the UE associates a newly detected application to an existing PDU Session (based on URSP evaluation result (see clause 6.6.2.3) for such a URSP rule), by sending the PDU Session Modification Request (see clause 4.3.3.2 of TS 23.502 [3]) including the URSP rule enforcement report; or

-	the UE changes the association of an application to a PDU Session (based on the URSP re-evaluation result (see clause 6.6.2.3) for such a URSP rule), by including the URSP rule enforcement report in a PDU Session Establishment (see clause 4.3.2.2.1 of TS 23.502 [3]) or by sending a PDU Session Modification Request (see clause 4.3.3.2 of TS 23.502 [3]) including the URSP rule enforcement report; or

-	the UE has the association of an application to a PDN Connection/PDU Session in EPC (based on the URSP evaluation result (see clause 6.6.2.3) for such a URSP rule) and the UE moves from EPS to 5GS, by including the URSP rule enforcement report in a PDU Session Establishment Request (for the case without N26, see the step 9 of Figure 4.11.2.3-1 of TS 23.502 [3]) or by sending a PDU Session Modification Request (for the case with N26, see clause 4.3.3.2 of TS 23.502 [3]) including the URSP rule enforcement report.

NOTE 2:	UE reporting of URSP rule enforcement can increase the amount of signalling in the network. Use of this feature is recommended to be restricted to URSP rules for specific application traffic on specific UEs based on the deployment choices of the operator.

If the UE enforces several URSP rules for multiple applications, and these multiple applications are all associated to the same established PDU Session, in order to reduce the number of uplink NAS messages, the UE may include more than one URSP rule enforcement report in the PDU Session Modification Request (see clause 4.3.3.2 of TS 23.502 [3]) and each URSP rule enforcement report includes all Connection Capabilities contained in the Traffic descriptor of the associated URSP rule.

The PCF receives the URSP rule enforcement report for a given UE either via:

-	Policy Control Request Trigger "UE reporting Connection Capabilities from associated URSP rule" (see clause 6.1.3.5), i.e. as part of SM Policy Association  Modification or

-	via SM Policy Association Establishment.

NOTE 3:	It is assumed that if SMF receives from the UE (via the AMF) URSP rule enforcement report as part of the PDU Session establishment request, it provides it to the PCF as part of SM policy association establishment request.

When the PCF serving a given UE is configured to use UE reporting of URSP rule enforcement for this UE and the configuration does not guarantee that the PCF serving the PDU Session is the same as the (H-)PCF serving the UE, then the (H-)PCF serving the UE subscribes to the PCF serving the PDU Session to receive the URSP rule enforcement report for the UE via PCF event reporting (see clause 6.1.3.18 and the related procedure in clause 4.16.16.2 of TS 23.502 [3]).

For LBO roaming session case, the H-PCF for the UE requests to forward UE reporting Connection Capabilities from an associated URSP rule to the V-PCF for the UE to receive the URSP rule enforcement report via PCF event reporting (see clause 6.1.3.18) during the UE Policy Association Establishment or Modification (see clause 4.16.16.3 of TS 23.502 [3]).

The PCF for the UE may check whether the URSP rule enforcement report and the relate PDU Session parameters (e.g. DNN/S-NSSAI) are compliant to the corresponding URSP rule of the UE. The PCF may perform the following actions:

-	To identify at least one of the URSP rule sent to the UE, the PCF compares the value of the URSP rule enforcement with the Connection Capabilities in all the URSP Rules, that includes "the Indication for reporting URSP rule enforcement" set and are provisioned to the UE.

-	To identify the RSD in the URSP rule used for the establishment/modification of the PDU Session, the PCF compares the PDU Session parameters with the Route Selection Components of each RSD in the identified URSP rule as follows: the Requested DNN and the S-NSSAI of the HPLMN in the PDU Session parameters is compared with the DNN included in the DNN selection and the list of S-NSSAI(s) in the Network Slice Selection, if any. The SSC mode and the PDU Session type in the PDU Session parameters with the SSC Mode and PDU Session type selection in the Route Selection Component.

If the PCF for the UE found an inconsistency in the PDU Session parameters, e.g., the Requested S-NSSAI are not matching the any Route Selection Component in an RSD of the identified URSP Rule, the PCF for the UE may perform additional check (e.g. check the Selected S-NSSAI) and may perform appropriate actions (e.g. initiating slice replacement procedure).

NOTE 4:	The identification of the URSP rule sent to the UE using the URSP rule enforcement information by the PCF can fail if the same list of Connection Capabilities is included in more that one URSP Rule with the Indication for reporting URSP rule enforcement. The possible actions at the PCF are implementation specific.

NOTE 5:	The PCF cannot check the Route Selection Validation Criteria, given that the UE can delay the reporting of the URSP Rule to reduce the amount of generated signalling.

Policy control decisions based on awareness of URSP rule enforcement are described in clause 6.1.1.5.

The USRP rule enforcement reporting is not supported by 5G-RG and FN-RG.

# *** End of Change ***