3GPP TSG-SA2 Meeting #164	S2-2407637

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1124 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sample and Feature alignment procedure for Vertical Federated Learning with AF as the VFL server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Futurewei, LG Electronics, Tencent, Sony, ICS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-07-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the conclusion in the TR 23.700-84, for AF acting as VFL Server and NWDAFs acting as VFL clients, AF performs sample alignment, selects samples to be used in the training process and generates the intersection of samples.The Feature description information may be registered to the NRF or locally configured in the NWDAF or the AF, negotiated between VFL server and VFL client when performing feature alignment. Feature alignment is optional in a VFL process. However, the feature is tightly related to the sample and it is a very dynamic information, it is not suitable to be registered to the NRF. The feature negotiation is needed. Therefore, this CR specify the procedures for sample and/or feature alignment with the AF as the VFL server based on the above conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 6.2X is added. The sub-clause 6.2X.1 and 6.2X.2 are added to specify the procedure for sample and feature alignment with AF as the VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can not perform VFL if do not specify the sample and feature ailgnemnt procedure to make sure the same sample and different feature used in the VFL server and client. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X (new), 6.2X.1 (new). 6.2X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new)* * * *

6.2X	Veretical Federated Learning between AF and NWDAFs

6.2X.1	General

This clause specifies how NWDAF containing MTLF and AF can leverage Vertical Federated Learning technique to perform VFL preparation, model training and inference.

6.2X.2	Procedures

6.2X.2.X	Sample and Feature alignment procedure for Vertical Federated Learning with AF as the VFL server

Figure 6.2X.2.X-1: Sample and Feature alignment procedure for Vertical Federated Learning with AF as the VFL server

0. VFL Server AF selects NWDAF(s) containing MTLF (VFL Client NWDAF(s)) as described in clause X.

1.	VFL server AF sends the VFL sample and feature alignmemnt request to the NEF using the Nnef_VFLSampleFeatureAlignment_request including the selected VFL client NWDAF ID(s), Analytics ID, the target samples (i.e. external UE IDs), and optionally, the feature profile (including features per each sample) indicates what features the AF can provide (i.e., a list of VFL server AF supported features) and what features are needed for performing the VFL model training (i.e., a list of required features for VFL tarining).

NOTE 1:	NEF is needed only when the AF is untrusted AF.

Editor's note:	Whether the feature profile is a part of the ML Model Interoperability Information is FFS.

2.	NEF maps the external UE IDs to the internal UE IDs (e.g. GPSI to SUPI).

3.	NEF sends the VFL sample and feature alignment request to the VFL client NWDAF(s) using Nnwdaf_MLModelProvision_Subscribe including the analytics ID, the internal UE IDs (e.g. SUPIs), and feature profile. If multiple NWDAFs involved, the NEF sends the VFL sample and feature alignment request to each VFL client NWDAF, respectively.

Editor's note:	Whether the existing or new service operation will be used to support the sample and feature alignment between NEF and NWDAF is FFS.

4.	[Optional] Based on the feature profile and requested analytics ID, each VFL client NWDAF may collect input data by using the current mechanism in clause 6.2 if local data is not currently available at the VFL Client NWDAF.

5.	Based on the available local data, each VFL Client NWDAF may determine whether it can support target samples and required features provided by the VFL server AF. Each VFL client NWDAF may down-select UEs based on its served UEs to ensure the samples are same among VFL client NWDAF(s) and VFL server AF as requested by the VFL server AF. The supported features of each VFL client NWDAF may also a subset of required features but different with supported features of the VFL server AF as requested by the VFL server AF. Each VFL Client NWDAF sends the sample and feature alignment response using Nnwdaf_MLModelProvision_Notify to the NEF including the samples and optionaly the features that each VFL Client NWDAF can support.

6.	If multiple VFL Client NWDAFs involved, the NEF collects the sample and feature alignment response from each VFL Client NWDAFs. NEF may generate the intersection of samples and maps the intersection of samples related UE IDs to the external UE IDs.

7.	NEF sends the VFL sample and feature alignment response to the VFL server AF using the Nnef_VFLSampleFeatureAlignment_response including the intersection of samples, the feature supported by each VFL client NWDAF and the analytics ID.

8.	The VFL server AF determine the final VFL client NWDAFs with the selected samples and features. If the VFL server AF determines that there is an insufficient quantity of samples or features, it can stop the sample and feature alignment procedure. Subsequently, it has the capability to initiate a new sample and feature alignment procedure by adjusting the sample and feature requirements.

9.	[Optional] When the selected VFL client NWDAF need update, the VFL server AF evaluates whether sample and feature realignment is needed for the ongoing VFL procedure. If necessary, the VFL server AF initiates the new sample and feature alignment procedure.

# * * * * End of changes * * * *