3GPP TSG-SA2 Meeting #164	S2-2407638

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1125 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General training procedure for vertical federated learning between AF and NWDAFs with AF as the VFL server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, ZTE, ETRI, China Telecom, CMCC, Tencent, Samsung, ICS,vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-07-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the conclusion captured in the TR 23.700-84 clause 8.2 P#2.4, this CR aims for specify the general training procedure for vertical federated learning between AF and NWDAFs with AF as the VFL server. The NEF is involved when the AF is an untrusted AF. An identifier is allocated by a VFL server, which is used to correlate the participants during the VFL training and subsequent VFL inference processes and it is associated with the distributed ML Models in the VFL joint model training process. |  |  |  |  |  |  |  |  |
| Is |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 6.2X is added. The sub-clause 6.2X.1 and 6.2X.2 are added to specify the general training procedure for vertical federated learning between AF and NWDAFs with AF as the VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can not perform VFL training if do not specify the VFL training procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X (new), 6.2X.1 (new). 6.2X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new)* * * *

6.2X	Veretical Federated Learning between AF and NWDAFs

6.2X.1	General

This clause specifies how NWDAF(s) containing MTLF and AF can leverage Vertical Federated Learning technique to perform VFL preparation, model training and inference.

6.2X.2	Procedures

6.2X.2.X	General training procedure for vertical federated learning between AF and NWDAFs with AF as the VFL server

Figure 6.2X.2.X-1: General training procedure for vertical federated learning between AF and NWDAFs with AF as the VFL server

0.	[Optional]For the VFL model training between AF and NWDAFs with AF as the VFL server, the consumer maybe the VFL server AF or the NWDAF containing AnLF. If the consumer is the NWDAF containing AnLF, the consumer sends a subscription request to VFL server AF via NEF to request the VFL server AF to initiate the VFL model training using Nnef_Event Exposure_Subscribe and Naf_EventExposure_Subscribe including Analytics ID and optionally, the time when the VFL training need to becompleted and the requested performance metric (e.g. required accuracy of trained ML model).

NOTE 1:	NEF is needed only when the AF is untrusted AF.

If the consumer provides the time when the VFL training needs to be completed, the VFL Server AF can take this information into account to decide the maximum response time for its VFL Client NWDAF(s).

1.	VFL server AF sends a Nnef_MLmodelTrainingAssistance_Subscribe to the NEF including the external UE IDs as the target samples and the feature profile to indicate whichs feature are used for the VFL training, the initial model parameters in order to configure the VFL local model for the VFL client NWDAF(s), the VFL model correlation ID in order to correlate the training processes at VFL client NWDAF(s) during the VFL training and subsequent VFL inference processes. It is associated with the distributed ML Models in the VFL training process. The VFL server AF may specify a maximum response time for each iteration (i.e. the maximum allowed time between receiving local intermediate training results from each VFL client NWDAF and sending back the intermediate training information).

2.	NEF maps the external UE IDs to the internal UE IDs (e.g. GPSI to SUPI) and sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF(s) containing MTLF (i.e. VFL Client NWDAF(s)). The request includes the parameters received from the step1. The VFL Client NWDAF has to report the local intermediate training results to the VFL Server AF before the maximum response time elapses.

3.	[Optional] Each VFL Client NWDAF collects its local data by using the current mechanism in clause 6.2 if the VFL Client NWDAF has not local data available already.

4.	During VFL training procedure, each VFL Client NWDAF trains the ML Model based on its local data to report the local intermediate training results to the VFL Server AF.

5.	Each VFL Client NWDAF sends the Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response to the NEF including the local intermediate training results and the VFL model correlation ID. Optionally, one VFL client NWDAF may send the local intermediate training results to the other VFL client NWDAF and let other VFL client NWDAF send the aggregates local intermediate  training results to the VFL server AF.

Editor's note:	Whether VFL client NWDAFs may also provide local intermediate training results to other VFL client NWDAFs is FFS.

The local intermediate training results, which were sent from the VFL Client NWDAF(s) to the VFL Server AF during the VFL training process, are the information for the VFL Server AF to aggregates local intermediate training results and generate the intermediate training information (e.g. gradient information, loss information) for updating its own local model and the local models at each VFL client NWDAF.

If the VFL Client NWDAF is not able to complete the training of the local VFL Model within the maximum response time provided by the VFL Server AF, the VFL Client NWDAF shall send the Delay Event Notification that includes the delay event indication, an optional cause code (e.g. local ML Model training failure, more time necessary for local ML Model training) and the expected time to complete the training if available to the VFL Server AF before the maximum response time elapses.

6.	NEF sends the Nnef_MLmodelTrainingAssistance_Notify to the VFL server AF including the parameters received in the step 5. The NEF may wait to collect local intermediate training result from each VFL client NWDAF and send all collected local intermediate training results to VFL server AF in one notification.

7.	The VFL Server AF aggregates all the local intermediate training results to generate the intermediate training information (e.g. gradient information, loss information) based on the VFL model correlation ID and the label stored in the VFL server AF for updating its own local model and the local models at VFL client NWDAF.

8-9. [Optional] Based on the consumer request in step 0, the VFL Server AF sends a Notify message to update the VFL training status to the consumer periodically or dynamically when some pre-determined status is achieved (e.g. the VFL Model Accuracy threshold is achieved or training time expires) using the Nnef_Event Exposure_Notify and Naf_EventExposure_Notify.

10-11. [Optional] The consumer decides whether the current model performance can fulfil the requirement, e.g. ML Model metric value is satisfactory for the consumer and determines to stop or continue the training process. The consumer re-invokes Nnef_Event Exposure_Subscribe and Naf_EventExposure_Subscribe as used in step 0 to continue the training process or invokes Nnef_Event Exposure_Unsubscribe and Naf_EventExposure_Unsubscribe service operation to stop the training process.

12.	[Optional] Based on the subscription request sent from the consumer, the VFL Server AF updates or terminates the current VFL training process.

If the VFL Server AF received a request in steps 10-11 to stop the VFL Training process, steps 13-15 are skipped.

13.	If the VFL procedure continues, the VFL Server AF may send Nnef_MLmodelTrainingAssistance_Subscribe that includes the intermediate traning information and VFL model correlation ID to selected VFL Client NWDAF(s) for next round of VFL.

14. NEF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF(s) containing MTLF (VFL Client NWDAF(s)). The request includes the parameters received from the step13.

15.	Each VFL Client NWDAF updates its own ML Model based on the intermediate traning information distributed by the VFL Server AF in steps 13-14.

NOTE 2:	The steps 3-15 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

When the 	VFL Training procedure is complete, the VFL Server AF requests the VFL client NWDAF(s) to terminate the VFL procedure by invoking Nnef_MLModelTrainingAssistance_Unsubscribe and Nnwdaf_MLModelTraining_Unsubscribe service with a cause code that the VFL process has finished. Then the VFL client NWDAF(s) terminate the local model training.

16-17.	After the training process is complete, the VFL Server AF may send Nnef_Event Exposure_Notify and Naf_EventExposure_Notify that includes the model traning complete indication to the consumer.

# * * * * End of changes * * * *