3GPP TSG-SA2 Meeting #164	S2-2407639

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1126 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General inference procedure for vertical federated learning between AF and NWDAFs with AF as the VFL server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, ETRI, China Telecom, Tencent, ICS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-07-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the conclusion captured in the TR 23.700-84 clause 8.2 P#2.4, this CR aims for specify the general inference procedure for vertical federated learning between AF and NWDAFs with the AF as the VFL server. The NWDAF contain AnLF is the consumer to request the VFL inference result to the VFL server AF. And the NWDAF contain AnLF based on the VFL inference result generate the analytics output and send the analytics output to the analytics consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 6.2X is added. The sub-clause 6.2X.1 and 6.2X.2 are added to specify the general inference procedure for vertical federated learning between AF and NWDAFs with AF as the VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can not perform VFL inference if do not specify the VFL inference procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X (new), 6.2X.1 (new). 6.2X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new)* * * *

6.2X	Veretical Federated Learning between AF and NWDAFs

6.2X.1	General

This clause specifies how NWDAF containing MTLF and AF can leverage Vertical Federated Learning technique to perform VFL preparation, model training and inference.

6.2X.2	Procedures

6.2X.2.X	General inference procedure for vertical federated learning between AF and NWDAFs with AF as the VFL server

Figure 6.2X.2.X-1: General inference procedure for vertical federated learning between AF and NWDAFs with AF as the VFL server

0.	Analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= UE IDs, Analytics Filter Information (e.g. Application ID, S-NSSAI, DNN, Application Server Address(es), Area of Interest, Analytics Reporting Information=Analytics target period)) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1-2.	The consumer (i.e NWDAF containing AnLF) sends a subscription request to VFL server AF via NEF using Nnef_AnalyticsExposure_Subscribe and Naf_EventExposure_Subscribe including Analytics ID, Target of Analytics Reporting = UE IDs, Analytics Filter Information (e.g. Application ID, S-NSSAI, DNN, Application Server Address(es), Area of Interest, Analytics Reporting Information=Analytics target period).

NOTE 1:	NEF is needed only when the AF is untrusted AF.

3.	Based on the information received in the step 2, VFL server AF decides to iniate the VFL inference procedure with the VFL client NWDAF(s) to generate more accurate analytics result. VFL server AF sends a Nnef_AnalyticsExposure_Subscribe to the NEF including the information received in the step 2 and the VFL model correlation ID to indicate the VFL client NWDAF which previously well-trained VFL local model associated with this ID will be used.

4.	NEF sends a Nnwdaf_MLModelInference_Subscribe or Nnwdaf_MLModelInference_Request to the NWDAF(s) containing MTLF (i.e. VFL Client NWDAF(s)) which were selected during the VFL training procedure. The request includes the parameters received from the step3.

5.	Each VFL Client NWDAF collects its local data by using the current mechanism in clause 6.2 if the VFL Client NWDAF has not local data available already.

6.	Based on the VFL coorelation ID, Each VFL Client NWDAF selects the VFL local model to generate the intermediate local inference results.

7.	Each VFL Client NWDAF sends the Nnwdaf_MLModelInference_Notify or Nnwdaf_MLModelInference_response to the NEF including the intermediate local inference results and the VFL model correlation ID. Optionally, one VFL client NWDAF may send the local intermediate inference results to the other VFL client NWDAF and let other VFL client NWDAF send the aggregates local intermediate inference results to the VFL server AF.

Editor's note:	Whether VFL client NWDAFs may also provide local intermediate inference results to other VFL client NWDAFs is FFS.

The intermediate local inference results, which are sent from the VFL Client NWDAF(s) to the VFL Server AF during the VFL inference process, are the information for the VFL Server NWDAF to aggregates and generates the VFL inference results.

8.	NEF sends the Nnef_AnalyticsExposure _Notify to the VFL server AF including the parameters received in the step 7. The NEF may wait to collect local intermediate training result from each VFL client and send all collected local intermediate training results to AF in one notification.

9.	The VFL Server AF aggregates all the intermediate local inference results to generate the VFL inference results based on the VFL model correlation ID.

10. The VFL server NWDAF sends Naf_EventExposure_Notify to the NEF including the VFL inference results and the analytics ID.

11.	NEF sends the Nnef_AnalyticsExposure_Notify to the consumer including the VFL inference results and the analytics ID. The consumer (i.e NWDAF containing AnLF) generates the analytics output based on the VFL inference results.

12. The NWDAF provides the analytics output to the analytics consumer NF by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

# * * * * End of changes * * * *