3GPP TSG-WG SA2 Meeting #164	S2-2407643

Maastricht, Netherlands, 19 - 23 August, 2024	(revision of S2-240xxx) (revision of S2-2406045)

Source:	ETRI

Title:	Conclusion for KI #1.3

Document for:	Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract: This paper proposes a conclusion for KI #1.3.

1. Introduction

This contribution proposes interim conclusion for KI#1.3 based on NWM discussion.

2. Proposal

It is proposed to include the following changes in TR 23.700-54 (V1.0.0).

# * * * * Start of change (ALL NEW TEXT) * * * *

8	Conclusions

8.1	Conclusions for DualSteer

8.1.X	Interim Conclusion for KI#1.3: Session management aspects for DualSteer

The following aspects are concluded as principles for normative work:

-	DualSteer Device performs separate PDU Session establishment procedures for the two SUPIs.

-	DualSteer Device does not include the Linked SUPI information in PDU Session establishment request.

-	DualSteer Device may include an indication whether the PDU session is of potential DualSteer switching in PDU Session Establishment Procedure.

-	The two PDU sessions established for DualSteer share a common UPF and a common SMF.

-	The two PDU sessions established for DualSteer are allocated the same IP address.

# * * * * End of change * * * *